3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2207864

Electronic Meeting, May 09 - May 20, 2022

Title:	Reply LS on modulation order for DMRS bundling

Response to:	R1-2202927 “LS on updated Rel-17 RAN1 UE features list for NR”

Release:	Rel-17

Work Item:	NR_cov_enh

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact person:	Shan YANG

yangshan@chinatelecom.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

In the RAN1 LS on updated Rel-17 RAN1 UE features list for NR in [2], the following issue was raised:

For NR_cov_enh, based on RAN4 feedback, FG 30-4 is defined without distinction of the modulation order. RAN1 has no intention to revert any existing RAN4 agreements on maximum duration. It is up to RAN4 to decide whether FG 30-4 is applicable only to QPSK and lower modulation order.

In RAN4, only modulation orders up to QPSK have been simulated when deriving the phase continuity requirements within certain maximum duration, and RAN4 has not discussed the required phase continuity and power consistency tolerances for other higher modulation orders.  Therefore, to make the specification clearer, from RAN4 perspective, it is recommended to add the restriction on the modulation order in the column of “note” for FG 30-4 in the UE feature list, i.e.,

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | FFS | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A | This is applicable for QPSK or lower modulation orders. | Optional with capability signalling |

2	Actions

To RAN1 and RAN2

ACTION: RAN4 respectfully asks RAN1 and RAN2 to take the above information in to consideration in their work.

3	Dates of next TSG RAN WG 4 meetings

RAN4 #104		2022-08-22 to 2022-08-26 		Toulouse, FR

RAN4 #104e-bis		2022-10-10 to 2022-10-19 		Electronic Meeting