3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207924

Electronic Meeting, 9 – 20 May 2022

Agenda Item:	9.15.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS receiver requirements for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The work item for extending current NR operation to 71 GHz was extended at TSG RAN#95-e [1]. Two of the incomplete tasks listed in the approved exception sheet are requirements in 38.104 clause 10.3 (FRC parameters for FR2 OTA reference sensitivity level) and clause 10.8 (modulated interfering signal type for receiver intermodulation requirement).

This contribution provides further proposals on these requirements based on the agreed WF in RAN4#102-e [2].

# 2.	Discussion

2.1	Reference sensitivity and FRC

The following point were included in [2]:

- The scaling factor of 9 dB of EISREFSENS relevant for FR2-2 is confirmed according to Table 2.1-1.

Table 2.1-1: FR2 OTA reference sensitivity requirement

| Frequency Range | BS channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
|  | 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
|  | 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| FR2-2 | 100,400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
|  | 400, 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 9 + ΔFR2_REFSENS |
|  | 400, 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 9 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |  |

- In general companies are ok with option1 but want to also confirm SU. The FRC parameters described in Table 2.2-1 will be used as baseline. Further alignment with respect to SU agreement is required.

Table 2.2-1: FRC parameters for FR2 OTA reference sensitivity level, OTA ACS, OTA in-band blocking, OTA out-of-band blocking, OTA receiver intermodulation and OTA in-channel selectivity

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 | G-FR2-A1-6 | G-FR2-A1-7 | G-FR2-A1-8 | G-FR2-A1-9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 | 480 | 960 | 480 | 960 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 | 66 | 33 | 33 | 17 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 | 5632 | 2856 | 2856 | 1608 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 | 24 | 16 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) (Note 3) | 5656 | 2808 | 5656 | 2872 | 1432 | 5656 | 2872 | 2872 | 1624 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 | 19008 | 9504 | 9504 | 4896 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 | 9504 | 4752 | 4752 | 2448 |

- FFS on defining a new dedicated FRC for 960 kHz SCS and 800 MHz carrier bandwidth for OTA reference sensitivity.

We have performed simulation for the new FRCs G-FR2-A1-6 to G-FR2-A1-9 together with the simulation assumptions recorded in TR 38.817-02 Annex B [3], and the simulation results are provided in Figure 1 below, where results for the existing FRCs G-FR2-A1-3 to G-FR2-A1-5 are also provided as references.

Figure 1: BLER Vs SNR for FRCs in FR2-2

It can be seen from Figure 1 that the required SNR to achieve 95% of maximum throughput (i.e., 5% BLER) are similar (within 1 dB) among the existing and new FRCs. Therefore, the scaling factor of 9 dB of EISREFSENS relevant for FR2-2 is justifiable.

The working assumptions for the transmission bandwidth configuration for FR2-2 in the agreed WF [4] are shown in Table 1 below.

Table 1: Transmission bandwidth configuration NRB for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 66 | 264 | N/A | N/A | N/A |
| 480 | N/A | 66 | 132 | 264 | N/A |
| 960 | N/A | 33 | 66 | 132 | [156] |

It can be seen from Table 1 that the new FRC parameters for G-FR2-A1-6 to G-FR2-A1-9 are aligned with the corresponding transmission bandwidth configurations for 100 MHz and 400 MHz channel bandwidths in the agreed WF. Therefore, we currently see no need to update the new FRC parameters, unless there are major updates on the transmission bandwidth configurations for 100 MHz and 400 MHz channel bandwidths comparing to the working assumptions in the agreed WF.

Proposal 1: No need to update the new FRC parameters, unless there are major updates on the transmission bandwidth configurations for 100 MHz and 400 MHz channel bandwidths comparing to the working assumptions in the agreed WF.

Moreover, we currently see no need to define a new dedicated FRC for 960 kHz SCS and 800 MHz carrier bandwidth, as multiple instances of G-FR2-A1-7 can be used to cover the full BS channel bandwidth as currently done for reference sensitivity in FR1 and FR2-1.

Proposal 2: No need to define a new dedicated FRC for 960 kHz SCS and 800 MHz carrier bandwidth.

2.2	Receiver intermodulation

The following points were included in [2]:

- Define the modulated interfering signal type for receiver intermodulation requirement as 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs.

- FFS on defining the modulated interfering signal type for receiver intermodulation requirement as 400 MHz DFT-s-OFDM NR signal.

Table 2.6-1: Interfering signals for intermodulation requirement

| Frequency Range | BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- |
| FR2-1 | 50 | ±7.5 | CW |
|  |  | ±40 | 50MHz DFT-s-OFDM NR signal (Note 1) |
|  | 100 | ±6.88 | CW |
|  |  | ±40 | 50MHz DFT-s-OFDM NR signal (Note 1) |
|  | 200 | ±5.64 | CW |
|  |  | ±40 | 50MHz DFT-s-OFDM NR signal (Note 1) |
|  | 400 | ±6.02 | CW |
|  |  | ±45 | 50MHz DFT-s-OFDM NR signal (Note 1) |
| FR2-2 | 100 | [±7.5] | CW |
|  |  | [±65] | 100MHz DFT-s-OFDM NR signal (Note 2) |
|  | 400 | [±6.28] | CW |
|  |  | [±70] | 100MHz DFT-s-OFDM NR signal (Note 2) |
|  | 800 | [±6.32] | CW |
|  |  | [±80] | [100MHz DFT-s-OFDM NR signal (Note 2)] |
|  | 1600 | [±6.4] | CW |
|  |  | [±100] | [100MHz DFT-s-OFDM NR signal (Note 2)] |
|  | 2000 | [±5.52] | CW |
|  |  | [±160] | [100MHz DFT-s-OFDM NR signal (Note 2)] |
| NOTE 1: Number of RBs is 64 for the 60 kHz subcarrier spacing, 32 for the 120 kHz subcarrier spacing. NOTE 2: Number of RBs is 64 with 120 kHz subcarrier spacing. |  |  |  |

Considering the test coverage enhancement for 800 MHz, 1600 MHz and 2000 MHz BS channel bandwidths using 400 MHz DFT-s-OFDM NR signal instead of 100 MHz DFT-s-OFDM NR signal as the modulated interfering signal, as well as the test equipment complexity for generating the 400 MHz DFT-s-OFDM NR signal, we currently see no need to define the modulated interfering signal type for receiver intermodulation requirement as 400 MHz DFT-s-OFDM NR signal.

Proposal 3: No need to define the modulated interfering signal type for receiver intermodulation requirement as 400 MHz DFT-s-OFDM NR signal.

# 3.	Conclusion

This contribution has provided further proposals on BS receiver requirements for extending current NR operation to 71 GHz based on the agreed WF. They are summarized as follows.

Proposal 1: No need to update the new FRC parameters, unless there are major updates on the transmission bandwidth configurations for 100 MHz and 400 MHz channel bandwidths comparing to the working assumptions in the agreed WF.

Proposal 2: No need to define a new dedicated FRC for 960 kHz SCS and 800 MHz carrier bandwidth.

Proposal 3: No need to define the modulated interfering signal type for receiver intermodulation requirement as 400 MHz DFT-s-OFDM NR signal.

# References

[1]	RP-220991, “Exception sheet on Extending current NR operation to 71GHz”, Qualcomm CDMA

Technologies, Intel.

[2]	R4-2207217, “WF on >52.6 GHz BS RF Rx Requirements”, Ericsson.

[3]	3GPP TR 38.817-02 v15.9.0: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”.

[4]	R4-2206536, “WF on 60 GHz UE RF”, Qualcomm Incorporated.