3GPP TSG-RAN WG4 Meeting # 103-e	R4-2207926

Electronic Meeting, 9 – 20 May 2022

Agenda Item:	9.15.5.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS receiver conformance testing for extending current NR operation to 71 GHz

Document for:	Approval

# 1.	Introduction

The work item for extending current NR operation to 71 GHz was approved at TSG RAN#91-e [1]. One of the objectives of this work item is to define BS conformance testing requirements for the band(s) in the frequency range between 52.6GHz and 71GHz. This topic was discussed at TSG RAN4#102-e and a WF was agreed [2].

This contribution provides proposals on BS receiver conformance testing requirements for extending current NR operation to 71 GHz based on the agreed WFs in the previous meetings.

# 2.	Discussion

2.1	Out-of-band blocking measurement step size

It was agreed in [3]:

- For out-of-band blocking consider:

  - Test system feasibility needs to be considered together with setting upper frequency limit for blocker.

  - Current measurement step size can be the starting point, but test time needs to be considering taking into account the extended upper frequency limit.

And it was agreed in [4]:

- Frequency definitions

  - For out-of-band blocking RF core requirement define the interferer signal upper frequency limit to 2nd harmonic similar as for FR2.

  - Postpone measurement step size discussion to performance part.

Current measurement step size for BS type 2-O is defined in TS 38.141-2 [5] depending on minimum supported channel bandwidth, with the maximum step size of 60MHz defined for minimum supported BS channel bandwidth 200MHz and 400MHz. Larger step size 120MHz or 240MHz should be defined for minimum supported BS channel bandwidth larger than 400MHz for NR operation in 52.6 – 71 GHz range to reduce the time required for this time-consuming testing. We propose to update Table 7.6.4.2.3-1 in TS 38.141-2 as follows for NR operation in 52.6 – 71 GHz range.

Table 7.6.4.2.3-1: Interferer signal step size

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000[142000] | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |
|  | 800 | 120 |
|  | 1600 | 240 |
|  | 2000 | 240 |

Proposal 1: Update Table 7.6.4.2.3-1 in TS 38.141-2 to include larger step size 120MHz or 240MHz for minimum supported BS channel bandwidth larger than 400MHz for NR operation in 52.6 – 71 GHz range.

# 3.	Conclusion

This contribution has provided proposals on BS receiver conformance testing requirements for extending current NR operation to 71 GHz based on the agreed WFs in the previous meetings. They are summarized as follows.

Proposal 1: Update Table 7.6.4.2.3-1 in TS 38.141-2 to include larger step size 120MHz or 240MHz for minimum supported BS channel bandwidth larger than 400MHz for NR operation in 52.6 – 71 GHz range.

Table 7.6.4.2.3-1: Interferer signal step size

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000[142000] | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |
|  | 800 | 120 |
|  | 1600 | 240 |
|  | 2000 | 240 |

# References

[1]	RP-210862, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC.

[2]	R4-2207218, “WF on >52.6 GHz BS RF conformance aspects”, Keysight Technologies.

[3]	R4-2106114, “WF on BS RF RX requirements for 52.6 – 71 GHz”, Ericsson.

[4]	R4-2108639, “WF on BS RX RF requirements for 52.6 – 71 GHz”, Huawei.

[5]	3GPP TS 38.141-2 v17.5.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.