3GPP TSG-RAN WG4 Meeting # 103-e						R4-2207935

Electronic Meeting, May 09 – 20, 2022

Source:		Verizon, Samsung

Title:			TP for TR 38.841: CA_n13-n77

Agenda item:		8.24.2

Document for:		Approval

# 1. Introduction

This contribution is a text proposal to introduce PC2 CA_n13-n77 in [1].

# 2. Reference

- 3GPP TR 38.841

- 3GPP TR 38.717-02-01

# 3. Text Proposal

<Start of Text Proposal>

### 5.x	CA_n13-n77

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n13A-n77A | CA_n13A-n77A8,9 | n13 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n13A-n77A | CA power class | Carrier n13 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n13A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.x.3	REFSENS requirements

According to the PC3 CA_n13A-n77A study in [2], the 5th harmonic mixing products from band n13 may fall into band n77 UL frequency range. Also, two-tone 5th order IMD products may fall into the own Rx Band of band n13. Thus additional MSD should be considered to mitigate the impact of the interference for PC2 CA_n13A-n77A combination.

#### 5.x.3.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 CA_n13A-n77A.

The additional MSD due to IMD for PC2 CA_n13A-n77A are defined in table 5.x.3.1-1.

Table 5.x.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13A-n77A | n13 | 782 | 5 | 20 | 751 | 20.5 | FDD | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |

#### 5.x.3.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for Case B are defined in table 5.x.3.2-1.

Table 5.5.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for NR CA  in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n131 | 34 | 31 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 Case B CA_n13A-n77A are same as the Case A defined in table 5.x.3.1-1.

#### 5.x.3.3	Single uplink power Class 1.5

The additional MSD due to receiver harmonic mixing for Case B are defined in table 5.x.3.3-1.

Table 5.x.3.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for NR CA  in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n131 | 37 | 34 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n13A-n77A requirements are applied for PC2 CA_n13A-n77A.

<End of Text Proposal>