3GPP TSG-RAN WG4 Meeting # 103-e						R4-2207937

Electronic Meeting, May 09 – 20, 2022

Source:		Verizon, Samsung

Title:			TP for TR 38.841: CA_n2-n77

Agenda item:		8.24.2

Document for:		Approval

# 1. Introduction

This is a text proposal to include MSD due to cross band isolation for the approved PC2 CA_n2-n77 combo in [1].

# 2. Reference

- 3GPP TR 38.841

# 3. Text Proposal

<Start of Text Proposal>

## 5.5		CA_n2-n77

### 5.5.1	Configurations

Table 5.5.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n77A | n778,9 CA_n2A-n77A8 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n77(2A) | n778,9 CA_n2A-n77A8 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n77C | n778,9 CA_n2A-n77A8 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2(2A)-n77A | n778,9 CA_n2A-n77A8 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n77C | n778,9 CA_n2A-n77A8 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | CA_n2(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.5.2	Maximum output power

Table 5.5.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n2A-n77A | CA power class | Carrier n2 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n2A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.5.3	REFSENS requirements

According to the PC3 CA_n2A-n77A study, the 2nd harmonic mixing products from band 2 may fall into band n77 UL frequency range. And, the 2nd, 4th and 5th order IMD products generated by dual uplink from both band 2 and band n77 may fall into band 2 Rx frequency range. Thus additional MSD should be considered to mitigate the impact of the interference for PC2 CA_n2A-n77A combination.

#### 5.5.3.1	Power class 2 Case A

The MSD due to receiver harmonic mixing for PC2 Case A are same as PC3 CA_n2A-n77A.

The additional MSD due to intermodulation for PC2 CA_n2A-n77A are defined in table 5.5.3.1-1.

Table 5.5.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n2A-n77A | n2 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD2 |
|  |  |  |  |  |  | 34.755 |  |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 19.10 | FDD | IMD44 |
|  |  |  |  |  |  | 21.855 |  |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

#### 5.5.3.2	Power class 2 Case B

The additional MSD due to receiver harmonic mixing for Case B are defined in table 5.5.3.2-1.

Table 5.5.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for NR CA  in NR FR1

| NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n2 | 9.1 | 8.0 | 7.0 | 6.7 |  |  |  |  |  |  |  |  |

Although no cross-band isolation MSD was identified for PC3 band n77 into n2, additional MSD due to cross band isolation from PC2 band n77 into mid-band shall be considered because of transmitter noise floor rising ~ 3dB. Referring the proposed MSD value for PC2 CA_n25-n77, the MSD requirements are defined in Table 5.5.3.2-2 and Table 5.5.3.2-3 when band n77 transmitting with PC2.

Table 5.5.3.2-2: UE PC2 MSD due to cross band isolation

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n2 | 1.0 | 1.0 | 1.0 | 1.0 |  |  |  |  |  |  |  |  |  |

Table 5.5.3.2-3: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | n2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The additional MSD due to intermodulation for PC2 Case B CA_n2A-n77A are same as the Case A defined in table 5.5.3.1-1.

#### 5.5.3.3	Single uplink power Class 1.5

The additional MSD due to receiver harmonic mixing for Case B are defined in table 5.5.3.3-1.

Table 5.5.3.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for NR CA  in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n2 | 11.8 | 10.6 | 9.5 | 9.2 |  |  |  |  |  |  |  |  |  |

Additional MSD due to cross band isolation from PC1.5 band n77 into mid-band shall be considered because of transmitter noise floor rising ~6dB. Referring the proposed MSD value for CA_n25-n77, the MSD requirements are defined in Table 5.5.3.3-2 and Table 5.5.3.3-3 when band n77 transmitting with PC1.5.

Table 5.5.3.3-2: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 n77 single band uplink into n2

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n2 | 1.8 | 1.8 | 1.8 | 1.8 |  |  |  |  |  |  |  |  |  |

Table 5.5.3.3-3: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | n2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.5.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n2A-n77A requirements are applied for PC2 CA_n2A-n77A.

<End of Text Proposal>