3GPP TSG-RAN4 Meeting #103-e	R4-2207968

E-meeting, 9 – 20 May, 2022

Source: 	Ligado Networks

Title: 	Updates to TS 38.101-5 related to n255 A-MPR clause

Agenda item:	9.12.5.1

Document for: 	Approval

# Introduction

This contribution reflects changes to TS 38.101-5 [1] based on proposal in concurrently submitted R4-2207967[2].

# Proposed Change

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR satellite band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in TS 38.101-1 [5] clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20 | Table 5.3.2-1 in TS 38.101-1 [5] | N/A |
| NS_24 | 6.5.3.3.13 in TS 38.101-1 [5] | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in TS 38.101-1 [5] | Clause 6.2.3.7 in TS 38.101-1 [5] |
| NS_57 | 6.5.3.3.1 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_100 | 6.5.2.4.2 in TS 38.101-1 [5] | n2561 |  |  | Table 6.2.3.1-2 in TS 38.101-1 [5] |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [8] absent is default for all NR bands.]

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_57 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [8]. |  |  |  |  |  |  |  |  |

# References

[1] TS 38.101-5, v0.1.0 (2022-03), UE radio transmission and equipment; Part 5: Satellite access RF and performance requirements (Release 17)

[2] R4-2207967, Measurements for n255 A-MPR evaluation, Ligado Networks, RAN4#103e