3GPP TSG-RAN WG4#103-e	R4-2207984

Electronic meeting, 9th – 20th May 2022

Source: 	Ericsson

Title:  	FR2 HST PUSCH demodulation requirements

Agenda Item:	8.9.5.3.1

Document for:	Discussion

# Introduction

During RAN4#102-e, a WF on BS demodulation requirements was agreed in [1]. This contribution provides simulation results and proposal for PUSCH demodulation requirements.

# Discussion

The following parameters have been used as the baseline configuration for PUSCH demodulation simulations based on the simulation assumptions in WF documents [1].

Table 1: Simulation assumption for PUSCH demodulation requirements.

| Channel Model |  | Bi-directional scenario B The details can be referred as R4-2115725 |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | [Pos0], [Pos1] and [Pos2] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | 50MHz and 200MHz |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| MCS |  | 19 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

The simulation results are presented in the tables below.

Table 2: Simulation ideal results for PUSCH demodulation.

| CBW | SNR @ 70% of Maximum TP |  |  |
| --- | --- | --- | --- |
|  | 1DMRS | 2DMRS | 3DMRS |
| 50MHz | 10.3 dB | 9.5 dB | 9.4 dB |
| 200MHz | 9.8 dB | 9.4 dB | 9.4 dB |

Table 3: Simulation results with impairments for PUSCH demodulation.

| CBW | SNR @ 70% of Maximum TP |  |  |
| --- | --- | --- | --- |
|  | 1DMRS | 2DMRS | 3DMRS |
| 50MHz | 12.5 dB | 11.5 dB | 11.4 dB |
| 200MHz | 12.0 dB | 11.4 dB | 11.4 dB |

# References

Nokia, Nokia Shanghai Bell, “WF on BS demodulation requirement for FR2 HST”, R4-2207235, RAN4#102-e, March 2022