3GPP TSG-RAN WG4 Meeting #103-e                                                    													R4-2207987

Online, 9th – 20th May, 2022

Source:	Vodafone

Title:	TP for TR 37.717-11-21: DC_1-20_n7-n78

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-11-21 to include DC_1-20_n7-n78.

The proposed TIB,c and RIB values were derived from the constituent fallback modes.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

## 7.x	DC_1-20_n7-n78

### 7.x.1	Configurations for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_20A_n7A DC_20A_n78A |

### 7.x.2	Channel bandwidths per operating band for DC

### Table 7.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_20A_n7A DC_20A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.x.3	Coexistence Studies

Co-existence studies of DC_1A-20A_n7A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.x.4	∆TIB and ∆RIB values

Table 7.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n7-n78 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |

Table 7.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n7-n78 | n78 | 0.5 |

### 7.x.5	Reference sensitivity exceptions

All MSD requirements for DC_1-20_n7-n78 are covered by constituent fallback modes. No additional MSD requirement is needed.

<End of Text Proposal>