3GPP TSG-RAN WG4 Meeting#103-e                	R4-2208009

Electronic Meeting, 9 May – 20 May 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PUSCH demodulation performance of Rel-17 NR coverage enhancements: simulation results

Agenda item: 	9.17.2.1

Document for: 	Discussion

Introduction

In the RAN4#102-e meeting, most of the parameters have been discussed. Agreements and the remaining steps during the last meeting regarding NR Coverage Enhancements BS Demodulation work are captured on the WFs  [1] and [2].

In this contribution, we provide the results of our simulation campaign taking into account the recommendation in the WFs [1] and [2].

PUSCH enhancements

TBoMS

Test parameters

We provide in this section the simulation results of TBoMS using the parameters below:

Table 1: TBoMS simulation parameters.

| Parameters | TBoMS |
| --- | --- |
| Metric | SNR at 70% throuhput |
| MCS | MCS2 and MCS4 |
| Resource allocation | 5 PRB |
| Number of slots | 2 available slots 4 available slots |
| TDD pattern / FR | FR1: 30 kHz TDD Option 1: 7D1S2U, S=6D:4G:4U |
| DMRS | DMRS 1+0 and 1+1 |
| Inter slot frequency hopping | Disabled |
| Channel condition | TDLB100-400 Low for FR1 |
| Antenna configuration | 2 Rx 4Rx 8Rx |
| RV sequence | [0 2 3 1] in case no repetition is used |
| PUSCH mapping type | PUSCH mapping type B |
| HARQ | 4 |
| Repetition | No repetition |
| transform precoding | CP-OFDM |

Simulation results: SNR at 70% throughput

The Table 2 and Table 3 give the SNR at 70% throughput for two TBoMS configurations: TBoMS over 2 slots and TBoMS over 4 slots respectively.

Table 2: TBoMS over 2 slots (SNR at 70% throughput).

| TBoMS over 2 slots |  |  |  |  |
| --- | --- | --- | --- | --- |
| SNR(dB) | MCS2 DMRS1+0 | MCS2 DMRS1+1 | MCS4 DMRS1+0 | MCS4 DMRS1+1 |
| 2RX | 0,94 | 0,39 | 2,71 | 1 ,84 |
| 4RX | -2,52 | -3,10 | -0,99 | -1,65 |
| 8RX | -5,72 | -6,32 | -4,20 | -4,81 |

Table 3: TBoMS over 4 slots (SNR at 70 % throughput).

| TBoMS over 4 slots |  |  |  |  |
| --- | --- | --- | --- | --- |
| SNR(dB) | MCS2 DMRS1+0 | MCS2 DMRS1+1 | MCS4 DMRS1+0 | MCS4 DMRS1+1 |
| 2RX | 0,69 | 0,29 | 2,43 | 1,67 |
| 4RX | -2,80 | -3,31 | -1,21 | -1,80 |
| 8RX | -5,93 | -6,43 | -4,41 | -5,20 |

PUSCH JCE

Test parameters

We provide in this section the simulation results of JCE using the parameters below:

Table 4: JCE simulation parameters.

| Parameters | JCE |
| --- | --- |
| Metric | SNR at 70% throughput |
| MCS | MCS4 |
| Resource allocation | full slot allocation in time domain full applicable test bandwidth |
| Number of slots | cTDW= 32 slots aTDW=2 |
| TDD pattern / FR | TDD: 7D1S2U, S=6D:4G:4U FR1 SCS 30 kHz |
| DMRS | DMRS 1+0 and 1+1 |
| Inter slot frequency hopping | Disabled |
| Channel condition | TDLA30-10 for FR1 |
| Antenna configuration | 2 Rx 4Rx 8Rx |
| RV sequence | Use [0 3 0 3] for TDD |
| PUSCH mapping type | PUSCH mapping type A and type B |
| HARQ | 4 |
| Repetition | repetition type A only // 8 repetitions and 2 repetitions |
| transform precoding | CP-OFDM |
| Phase offset model | with ideal phase offset |

Simulation results: SNR at 70% throughput and 2% BLER

The Table 5 and Table 6 give the SNR gain for JCE with 2 and 8 repetitions respectively using both 70% throughput  and 2% BLER values.

Table 5: SNR gain at 70% throughput using 2 repetitions

| Repetition | Rx number | DMRS | SNR gain (dB) at : 70 % throughput | SNR gain at : 2% BLER |  |
| --- | --- | --- | --- | --- | --- |
| 2 | 2 | Type A 1+1 [4,12] | 0,55 | 0,64 |  |
|  |  | Type A 1+0 [4] | 0,27 | 0,28 |  |
|  |  | Type B 1+1 [1,11] | 0,45 | 0,55 |  |
|  |  | Type B 1+0 [1] | 0,25 | 0,3 |  |
|  | 4 | Type A 1+1 [4,12] | 1,05 | 1,04 |  |
|  |  | Type A 1+0 [4] | 0,34 | 0,39 |  |
|  |  | Type B 1+1 [1,11] | 0,76 | 0,82 |  |
|  |  | Type B 1+0 [1] | 0,36 | 0,34 |  |
|  | 8 | Type A 1+1 [4,12] | 1,31 | 1,26 |  |
|  |  | Type A 1+0 [4] | 0,43 | 0,4 |  |
|  |  | Type B 1+1 [1,11] | 1,09 | 1,08 |  |
|  |  | Type B 1+0 [1] | 0,44 | 0,41 |  |

Table 6: SNR gain at 70% throughput using 8 repetitions.

| Repetition | Rx number | DMRS | SNR gain (dB) at : 70 % throughput | SNR gain at : 2% BLER |
| --- | --- | --- | --- | --- |
| 8 | 2 | Type A 1+1 [4,12] | 1,59 | 1.44 |
|  |  | Type A 1+0 [4] | 0,5 | 0,75 |
|  |  | Type B 1+1 [1,11] | 1,22 | 1,21 |
|  |  | Type B 1+0 [1] | 0,41 | 0,46 |
|  | 4 | Type A 1+1 [4,12] | 1,72 | 1,7 |
|  |  | Type A 1+0 [4] | 0,53 | 0,64 |
|  |  | Type B 1+1 [1,11] | 1,3 | 1,34 |
|  |  | Type B 1+0 [1] | 0,54 | 0,4 |
|  | 8 | Type A 1+1 [4,12] | 1,97 | 2,19 |
|  |  | Type A 1+0 [4] | 0,74 | 0,68 |
|  |  | Type B 1+1 [1,11] | 1,34 | 1,65 |
|  |  | Type B 1+0 [1] | 0,77 | 0,73 |

- We don’t see the same gain at 2% BLER as some other companies show in last meetings.

Conclusion

In this contribution we provided the results of our simulation campaign on both PUSCH TBoMS and PUSCH JCE for NR_Cov_enh Rel-17 BS demodulation performance requirements.

References

- R4-2203030 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement

- R4-2207210 WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement