3GPP TSG-RAN WG4 Meeting #103-e	R4-2208029

Online Meeting, May 9 – 20, 2022

Agenda item:	9.20.2.6

Source: 	Qualcomm Incorporated

Title: 	On performance requirements for PRS-RSRPP

Document for:	Discussion

# Introduction

RAN4 officially completed the core requirements for Rel-17 NR positioning enhancements in RAN4#102-e. Agreements and remaining issues were documented in a WF [1].

In this paper we will discuss the following topics:

- Propagation channel model for PRS-RSRPP accuracy requirements

- PRS-RSRPP relative accuracy requirements

# Discussion

There were two agreements related to PRS-RSRPP performance in RAN4#102-e [1]. A typo was corrected in the second agreement (PRS-RSRPP instead of PRS-RSRP).

Issue 3-1-1: PRS-RSRPP accuracy

Agreements:

- RAN4 to discuss what propagation model to use for defining PRS-RSRPP accuracy requirements during the performance part.

Issue 3-2-1: Differential PRS-RSRPP measurement requirements

Agreements:

- Discuss relative PRS-RSRPP accuracy requirements after RAN1/RAN2 conclusion on relative PRS-RSRPP measurement

Regarding the choice of propagation channel model used to define PRS-RSRPP measurement accuracy requirements, our view is that a fixed two-path channel would be a good choice. There is already a two-path channel defined in TS 38.101-4 that may be leveraged for this purpose.

## B.2.4	Two-tap propagation conditions for CQI tests

For Channel Quality Indication (CQI) tests, the following additional multi-path profile is used:

in continuous time representation, with  the delay, a constant value of a andthe Doppler frequency. The same h(t,τ) is used to describe the fading channel between every pair of Tx and Rx.

Proposal 1: Leverage the two-tap propagation channel model defined in 38.101-4, Annex B.2.4, for the purpose of defining PRS-RSRPP measurement accuracy requirements.

- FFS: The values for channel parameters a,  and .

Regarding the second agreement above, our understanding is that the definition of PRS-RSRPP has been finalized by RAN1 [2]. It is an absolute measurement, as defined below.

Agreement

The following definition for DL PRS RSRPP in 38.215 is endorsed:

- DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

Observation 1: According to RAN1 definition, PRS-RSRPP is an absolute power measurement, not a relative measurement.

# Conclusions

Proposal 1: Leverage the two-tap propagation channel model defined in 38.101-4, Annex B.2.4, for the purpose of defining PRS-RSRPP measurement accuracy requirements.

- FFS: The values for channel parameters a,  and .

Observation 1: According to RAN1 definition, PRS-RSRPP is an absolute power measurement, not a relative measurement.

# References

[1] R4-2206979, WF on NR Positioning Enhancements (Part 1), RAN4#102-e

[2] RP-220803, Status report for NR Positioning Enhancements, RAN#95-e