3GPP TSG RAN4 Meeting #103-e	R4-2208051

Electronic Meeting, 9th – 20th May, 2022

Title: 	Discussion on Rel-17 RAN4 UE feature list

Source: 	Intel Corporation

Agenda item:	7

Document for:	Discussion

# Introduction

In the previous RAN4 WG meeting, RAN4 UE feature list for Rel-17 was updated in R4-2206571 [1] and an LS was sent to RAN2 about the approved feature list in [2]. In this paper we continue to propose for some of the items the needed RAN4 features list and updates.

# Discussion

## NR positioning enhancements [NR_pos_enh]

The following UE features update (removal of brackets) proposed for the Rel-17 NR positioning enhancements WI are listed in table 1.

Table 1. NR_pos_enh feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14. NR_pos_enh | 14-1 | per-FR MG for PRS measurement | Capability of supporting per-FR MG for PRS measurement | Rel-15 per-FR gap (independentGapConfig) | yes | no |  | Per UE | No | No | N/A |  | Optional with capability signalling |
| 14 NR_pos_enh | 14-2 | PRS measurement for reduced sample in RRC_inactive state | Capability of supporting reduced number of samples (M=1, 2) for PRS measurement in RRC_inactive state | 27-17 | no |  | The reduced number of samples (M=1,2) for PRS measurement in RRC_inactive state cannot be supported. The UE is assumed to support M=4 only. | Per UE | No | No | N/A |  | Optional with capability signaling |
| 14. NR_pos_enh | 14-3 | PRS measurement without MG | Capability for the threshold used to be compared against with the Rx timing difference to determine whether the PRS from the non-serving cell satisfy the condition of PRS measurement outside MG. | 27-3-2 | yes |  |  | Per UE | No | No | N/A | The candidate threshold values: (CP length, half of slot) | Optional with capability signaling |
| 14. NR_pos_enh | 14-4 | Parallel PRS measurements in RRC_INACTIVE state | Capability for the support of performing RRM measurement and PRS measurement in parallel |  | yes | no | RRM measurement and PRS measurement cannot be performed in parallel | Per UE | No | Yes | N/A | Measurement period for UE suporting this capability scales with Kcarrier_PRS=1 | Optional with capability signalling |

## Extending current NR operation to 71GHz [NR_ext_to_71GHz]

The following UE features are proposed for the Rel-17 Extending current NR operation to 71GHz WI. We have a dedicated discussion paper for the below table in [3].

Table 2. NR_ext_to_71GHz feature list

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-2 | FR2-2 channel bandwidths for each SCS in each band for DL and UL for a single CC | Support of FR2-2 channel bandwidths 1) 120 kHz SCS: {100, 400} MHz CBW 2) 480 kHz SCS: {400, 800, 1600} MHz CBW 3) 960 kHz SCS: {400, 800, 1600, 2000} MHz CBW | FFS | Yes | No | UE cannot support some UE channel bandwidths | Per band | N/A | Applicable to FR2-2 only | N/A | UE indicating the support of specific SCS per band (RAN1 features X-Y) is required to support all CBWs corresponding to this SCS No additional capability signalling is needed. | NA |
| 15-3 | [Improved ON/ON transient period] | 1) Support of improved ON/ON transient period of X < 5us (X is FFS) | FFS | Yes | No | UE does not support improved ON/ON transient period and support 5us transient period | Per UE | N/A | Applicable to FR2-2 only | N/A | Further RAN4 discussion is required on whether to support improved ON/ON transient period and X value | Optional with capability signalling |

## NR NTN solutions [NR_NTN_solutions]

In the previous RAN4 meeting, features for NTN solutions were agreed. In order for RAN2 to implement the features correctly, it is of significance that we remove all the square brackets in the list. Summary of our proposal (removal of brackets) is presented in Table 4.

Table 4. NR_NTN_solutions feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_NTN_solutions | 25-1 | Parallel measurements on multiple SMTC-s for a single frequency carrier | Support of measurements on target cells belonging to maximum of 2 or 4 different NGSO satellites |  | Yes | N/A | UE does not support NTN RRM measurements with multiple SMTC configurations | Per UE | FDD only | FR1 only | NA | RAN2 recommendation is that UE is mandatory to support 2 and optional to support 4. | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-2 | Parallel measurements on cells belonging to different NGSO satellites as the serving cell without scheduling restrictions | Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions |  | Yes | N/A | UE does not support measurements on cells belonging to different [NGSO satellites] as the serving cell in parallel with normal operation | Per UE | FDD only | FR1 only | NA | For UEs not able to perform measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-3 | Parallel measurements with multiple measurement gaps | Support of maximum number of supported measurement gaps |  | Yes | N/A | UE does not support more than one measurement gap for NTN RRM measurements | Per UE | FDD only | FR1 only | NA | Candidate values are 1 or 2. The decision can be revisited in case it is identified that the agreement contradicts to RAN2 design. | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-4 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Support of enhanced RRM requirements for measurements in IDLE and INACTIVE modes as specified in TS 38.133 |  | No | N/A | UE does not support enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Per UE | FDD only | FR1 only | NA |  | Optional without capability signaling |

# Summary

In the last meeting we sent RAN4 latest version of feature list [1] in an LS in [2]. It is important to provide our feature list to other WG-s after this meeting so in this paper we continue to share some of our views on specific topics.

# References

- R4-2206571, Rel-17 RAN4 UE feature list for NR, CMCC

- R4-2206572, LS on Rel-17 RAN4 UE features list for NR, CMCC

- R4-2208045, UE features for NR ext. to 71GHz WI, Intel Corporation