3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2208061

Electronic Meeting, 9th-20th May, 2022

Source: 	Intel

Title: 	Discussions on UE power saving for RLM and BM

Agenda Item:	9.13.1

Document for:	Discussion

# 1 Introduction

In RAN4 #103-e meeting, the relaxation of RLM/BFD was discussed and a WF [1] was approved. In this contribution, we share our view on the following topics for relaxation of RLM/BFD measurements.

- explicit signalling clarification - Relaxation criteria for multiple RLM-RS/BFD-RS

# 2 Discussion

Issue 2-1-1: The cases that UE is allowed to apply the relaxed RLM/BFD requirement UE is allowed to apply the relaxed RLM/BFD requirement, - provided UE is configured the explicit signalling to enable RLM/BFD relaxation and UE has fulfilled good serving cell criterion, if low mobility criteria is NOT configured, or - provided UE is configured the explicit signalling to enable RLM/BFD relaxation and UE has fulfilled both good serving cell criterion and low mobility criterion if low mobility criteria is configured - The following note shall be further clarified in the maintenance phase - Note: if the explicit signalling is configured to enable the RLM/BFD relaxation and fulfilment of the low mobility criteria, or if it means low mobility state of the UE may be implicitly inferred.

In last meeting, the explicit signalling needs more clarification. From our point of view, explicit signalling only enable and disable RLM/BFD relaxation, it’s not related to low mobility criteria or assumption. There is a LS[2] from RAN2,

Besides, RAN2 also discussed the mechanism for RLM/BFD relaxation, and achieved the following working agreements: - UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. - The feature is configured by RRC dedicated signalling, this is the only enable disable function that is supported.

From the agreement of RAN2, explicit signalling is the only enable/disable function which is not related to low mobility criteria. Therefore, we suggest not to add such note.

Proposal 1: Don’t need to add the note for explicit signalling which involves low mobility assumption.

Issue 2-6-2: Clarification on multiple RLM-RS/BFD-RS - The following can be clarified in the maintenance phase - For entering condition, - Option 1: the radio link quality of at least one RLM-RS is better than the entering threshold. (Huawei, Qualcomm, Xiaomi, Apple, vivo. OPPO) - Option 2: The UE is allowed to operate RLM/BFD in relaxed mode for a certain cell (SpCell or SCell) when the radio link quality is better than the threshold (e.g. Qout + X1) for all RLM-RS resource. (CMCC, Ericsson, CATT, Nokia, ZTE) - Option 3: pending by other issues (Intel) - For exiting condition, - Option 1: the radio link quality for all the RLM-RS resources are worse than the exiting threshold. (Huawei, Qualcomm, Apple, OPPO) - Option 2: The UE shall exit the relaxed mode when the radio link quality is worse than the threshold (e.g. Qout + X2) for any the RLM-RS resources. (CMCC, Ericsson, CATT, Nokia, ZTE) - Option 3: The UE behaviour on checking the exiting condition of cell quality criterion regarding multiple RLM-RSs/BFD-RSs is not specified. (vivo, Xiaomi) - Option 4: pending by other issues (Intel)

Since the same criteria is defined for both RLM and BFD. Similar with legacy requirement, if at least one RLM-RS is better than the entering threshold, UE is allowed to enter relaxation mode. If the requirement is based on all the RL-RS will be higher than entering threshold, the condition may be too strict and UE may not enter relaxation mode.

If all the RLM-RS resources are worse than Qout, UE will exit relaxation mode. Since if some of the RLM-RS can still provide good channel quality, UE can still be in relaxation mode.

Proposal 2: For the case of multiple RLM-RS/BFD-RS , entering condition is that the radio link quality of at least one RLM-RS is better than the entering threshold.

Proposal 3: For the case of multiple RLM-RS/BFD-RS , exiting condition is that all the RLM-RS resources are worse than Qout.

# 3 Conclusion

In this contribution, we provide our views regarding to UE power saving for RLM/BFD:

Proposal 1: Don’t need to add the note for explicit signalling which involves low mobility assumption.

Proposal 2: For the case of multiple RLM-RS/BFD-RS , entering condition is that the radio link quality of at least one RLM-RS is better than the entering threshold.

Proposal 3: For the case of multiple RLM-RS/BFD-RS , exiting condition is that all the RLM-RS resources are worse than Qout.

# 4 Reference

[1] R4-2206909, WF on RLM/BM relaxation, MTK

[2] R2-2204238, Reply LS to RAN4 on RLM/BFD relaxation for ePowSav, vivo