3GPP TSG-RAN WG4 Meeting # 103-e 	                                            R4-2208070

Electronic Meeting, May 09 – May 20

Agenda Item:	9.20.2.3.

Source:	Intel Corporation

Title:	Discussion on performance requirements for PRS Measurement in RRC_INACTIVE

Document for:		Discussion

# Introduction

In the last RAN4 meeting, the core part of NR positioning enhancement requirements was completed[1]. In this contribution we will provide some considerations on PRS performance requirement in RRC_INACTIVE below.

- SINR side condition

- Measurement accuracy requirements

- Test cases list

# Side Condition

In current TS38.133, the SSB SINR side condition in case of RRC_INACTIVE (RRC_IDLE) is -4dB.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1[2]

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Observation 1: For PRS measurement in RRC_INACTIVE, the same assumption on SINR side condition as these of SSB measurement need to be investigated.

Therefore, it is safe to propose that:

Proposal 1: SINR side conditions for PRS measurements in RRC_INACTIVE can be

- PRS Es/Iot = -4 dB for reference cell and

- PRS Es/Iot = [TBD] dB for neighbor cells

# Reduced number of samples in RRC_INACTIVE state

In the last meeting, RAN4 also agreed that the reduced number of samples in RRC_INACTIVE state can be supported by UE.

Agreements: - Support of the reduced number of samples in RRC_INACTIVE state is UE capability - The UE capability is under discussion in RAN1, and wait for RAN1 agreement. - PRS measurement requirements with reduced number of samples in RRC_INACTIVE state are applicable only for UE which supports PRS measurements with reduced number of samples. - The requirements with reduced sample number are applicable when UE is requested by LMF to perform measurement with reduced sample number. - FFS: PRS measurement requirements with reduced number of samples in RRC_INACTIVE are defined under the same side conditions as agreed for RRC CONNECTED state.

However, whether the same accuracy requirements RRC_INACTIVE are defined under the same side conditions as agreed for RRC CONNECTED state can depend on the conclusion on the needed SINR side conditions.

Proposal 2: The necessary accuracy requirements for PRS measurements in RRC_INACITVE shall be defined if the SINR side conditions in case of RRC_INACTIVE is different with these in RRC_CONNECT.

# Test cases list

Similarly, the following test cases for NR positioning in RRC_INACTIVE state are necessary as in RRC_CONNECT state:

- Reporting delay test cases for NR standalone cells

  - RSTD

  - UE RX-TX time difference

  - PRS RSRP

- Measurement accuracy tests for NR standalone cells

  - RSTD

  - UE RX-TX time difference

  - PRS RSRP

Proposal 3: The following test cases for core requirement (e.g. reporting delay tests) can be defined for PRS measurements in RRC_INACTIVE.

| No | Type of Test | Description | Test purpose | Notes |
| --- | --- | --- | --- | --- |
| 1-1 | RSTD measurement reporting with reduced number of samples | FDD/TDD, FR1/FR2 PRS configuration in Rel16 DRX cycle 3 cells in total Total measured positioning frequency layer is 2 AWGN | Core requirements in section 5.6.2. which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay with reduced number of samples |  |
| 2-1 | PRS RSRP measurement reporting with reduced number of samples | FDD/TDD, FR1/FR2 PRS configuration in Rel16 DRX cycle 2 cells in total Total measured positioning frequency layer is 2 AWGN | Core requirements in section 5.6.3. which is also rely on UE’s processing capability to be verified. UE reports PRS RSRP within required delay with reduced number of samples |  |
| 3-1 | UE Rx-Tx time difference measurement reporting with reduced number of samples | FDD/TDD, FR1/FR2 PRS configuration in Rel16 DRX cycle 2 cells in total Total measured positioning frequency layer is 2 AWGN | Core requirements in section 5.6.4. which is also rely on UE’s processing capability to be verified. UE reports PRS RSRP within required delay with reduced number of samples |  |

# Conclusion

In this contribution, some considerations on the performance requirements of PRS measurements in RRC_INACTIVE are provided and the following observations and proposals can be drawn:

Observation 1: For PRS measurement in RRC_INACTIVE, the same assumption on SINR side condition as these of SSB measurement need to be investigated.

Proposal 1: SINR side conditions for PRS measurements in RRC_INACTIVE can be

- PRS Es/Iot = -4 dB for reference cell and

- PRS Es/Iot = [TBD] dB for neighbor cells

Proposal 2: The necessary accuracy requirements for PRS measurements in RRC_INACITVE shall be defined if the SINR side conditions in case of RRC_INACTIVE is different with these in RRC_CONNECT.

Proposal 3: The following test cases for core requirement (e.g. reporting delay tests) can be defined for PRS measurements in RRC_INACTIVE.

| No | Type of Test | Description | Test purpose | Notes |
| --- | --- | --- | --- | --- |
| 1-1 | RSTD measurement reporting with reduced number of samples | FDD/TDD, FR1/FR2 PRS configuration in Rel16 DRX cycle 3 cells in total Total measured positioning frequency layer is 2 AWGN | Core requirements in section 5.6.2. which is also rely on UE’s processing capability to be verified. UE reports RSTD within required delay with reduced number of samples |  |
| 2-1 | PRS RSRP measurement reporting with reduced number of samples | FDD/TDD, FR1/FR2 PRS configuration in Rel16 DRX cycle 2 cells in total Total measured positioning frequency layer is 2 AWGN | Core requirements in section 5.6.3. which is also rely on UE’s processing capability to be verified. UE reports PRS RSRP within required delay with reduced number of samples |  |
| 3-1 | UE Rx-Tx time difference measurement reporting with reduced number of samples | FDD/TDD, FR1/FR2 PRS configuration in Rel16 DRX cycle 2 cells in total Total measured positioning frequency layer is 2 AWGN | Core requirements in section 5.6.4. which is also rely on UE’s processing capability to be verified. UE reports PRS RSRP within required delay with reduced number of samples |  |

# References

- R4-2206779

- 3GPP TS 38.133 v17.5.0: "NR: Requirements for support of radio resource management "