3GPP TSG-RAN WG4 Meeting #103-e              	R4-2208077

Electrical Meeting, 9th May- 20th May, 2022

Agenda item:	9.8.4.2.1

Source:	Samsung

Title:	Discussion and simulation results of PUSCH requirement for Rel-17 FR2 HST

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, all the remaining issues on the test parameters on PUSCH requirement for Rel-17 FR2 HST were discussed.

The related agreement was captured in the WF [1] as

- Test applicability - FR2 HST PUSCH requirement test shall apply only for the additional DM-RS position declared to be supported. If more than one DMRS configuration is declared to be supported, the test shall be done for the minimum number of DMRS supported. - Manufacturer declaration on HST FR2 DM-RS support - PUSCH - Adopt the following manufacturer declaration for different additional DM-RS position support for FR2 HST. - Additional DM-RS position for FR2 high speed train: - Declaration of supported additional DM-RS position(s) for FR2 high speed train scenario for PUSCH and UL timing adjustment, i.e., pos0, pos1, pos2, or any combination. - Receiver baseline assumption for simulation - Assume a receiver with post FFT FOC - MCS - MCS 19 - Requirement selection - Apply standard requirement selection to (post-FFT) results with outlier selection, as in Rel-15 [R4-1904713] [R4-19004714]. Choose ideal result alignment threshold as [2.5dB], and impairment threshold as [4dB]. - It’s encouraged that companies can further update their results in future meetings to specify final performance requirements.

In this contribution, the view on the requirement derivation is provided. Meanwhile, the ideal and impairment results are provided for requirement derivation.

# 2	Discussion

Requirement selection

In the previous meeting, interested companies provided the simulation results for PUSCH with different RS combination and MCS. Due to different implementation, a very large span was still observed in PUSCH simulation results; especially at higher MCS.

As agreed, MCS 19 was selected for requirement. Based on the simulation summary [2], we still observe that a large gap among companies, up to [3.5] dB. Regarding requirement derivation, it was agreed to apply standard requirement selection to (Post-FFT) results with outlier selection, as in Rel-15. Choose ideal result alignment threshold as [2.5dB], and impairment threshold as [4dB]. And It’s encouraged that companies can further update their results in future meetings to specify final performance requirements.

As agreed in the last meeting, the baseline receiver assumption for simulation is post-FFT FOC. And it is up to BS implementation. With high doppler and high MCS, the ICI impact will be more seriously with post-FFT FOC. With ICI cancellation or not, the performance will be different, and also pending on BS implementation, it may result in the performance gap among companies.

Since RAN4 is to define the minimum requirement and to verify the basic receiver, in case the large gap is stilled existed, we would like to suggest to choose the ideal results alignment threshold as [3.0] dB for requirement derivation

Proposal 1:  The ideal result alignment threshold as [3.0] dB for requirement derivation can be considered, in case the results has still large gap existed.

# 3	Simulation Results

In this subsection, the ideal and impairment results are provided for requirement derivation.

Table 3-1 Simulation Assumption for PUSCH requirement

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna layout |  | 1T2R, ULA Low Correlation |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Option1: 1+0 Option2: 1+1 Option3: 1+1+1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
|  | Bandwidth | 50MHz and 200MHz |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | KPT-RS=2 |
|  | Time density (LPT-RS) | LPT-RS = 1 |
| MCS |  | MCS19 |
| Propagation channel |  | Bi-directional with scenario B |

Table 3-2:  Ideal and impairment results for MCS level for scenario B with Bi-directional deployment scenario (Ds=700m, Dmin=150m), 120KHz, 200MHz

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP (ideal) | SNR@70 of TP (Impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1T2R | 120KHz, 200MHz | B | 10 | 19 | 1+0 | 11.1 | 13.6 |
| Case 2 | 1T2R | 120KHz, 200MHz | B | 10 | 19 | 1+1 (0,8) | 10.7 | 13.2 |
| Case 3 | 1T2R | 120KHz 200MHz | B | 10 | 19 | 1+1+1(0.4.8) | 11.0 | 13.5 |

Table 3-3:  Ideal and impairment results for MCS level for scenario B with Bi-directional deployment scenario (Ds=700m, Dmin=150m), 120KHz, 50MHz

| Test Case | Tx/Rx | SCS&BW | Mapping type | Symbol length | MCS (64QAM) | DMRS configuration | SNR@70 of TP (ideal) | SNR@70 of TP (Impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1T2R | 120KHz, 50MHz | B | 10 | 19 | 1+0 | 11.1 | 13.6 |
| Case 2 | 1T2R | 120KHz,50MHz | B | 10 | 19 | 1+1 (0,8) | 10.8 | 13.3 |
| Case 3 | 1T2R | 120KHz, 50MHz | B | 10 | 19 | 1+1+1(0.4.8) | 10.6 | 13.1 |

# 4	Conclusion

In this contribution, the view on the PUSCH requirement derived for Rel-17 FR2 HST is provided. Meanwhile, the ideal and impairment results are provided for requirement derived

Proposal 1:  The ideal result alignment threshold as [3.0] dB for requirement derivation can be considered, in case the results has still large gap existed.

# References

R4-2207235, WF on BS demodulation requirement for FR2 HST, Nokia, Samsung

R4-2203542, Simulation results summary for Rel-17 FR2 HST BS demod, Samsung