3GPP TSG-RAN WG4 Meeting #103-e             	R4-2208081

Electrical Meeting, 9th May- 20th May, 2022

Agenda item:	10.8.6.2.2

Source:	Samsung

Title:	Discussion and simulation results for Rel-17 NB-IoT

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, it was agreed to introduce UL 16QAM requirement for NPUSCH in Rel-17 NB-IoT. The detail simulation assumption was discussed to facilitate the requirement definition. The related agreement was captured into the WF [1] as

- Number of allocated subcarriers - Option 1: 3, 6, 12 tones - Option 2: 12 tones - Number of scheduled RUs and TBS - Option 1: (IRU, ITBS)=(0,15), TBS=280 bits - Option 2: (IRU, ITBS)=(0,14), TBS=256 bits - RV - [0,2,0,2] - Maximum number of HARQ transmission: 4 - Simulation assumption for NPUSCH format 1 (just for information) Parameter; Value. Number of antennas; 1T2R. Propagation condition; ETU 1Hz Low. SCS; 15KHz. Frequency Resource; Option 1: 3,6,12 tones Option 2: 12 tones. Modulation order; 16QAM. Number of Repetition; 1. TB Scheduling; single-TB scheduling. Noise Estimation; Practical. Channel Estimation; Practical. Frequency offset; 0. Time offset; 0. (IRU, ITBS); [(0,14) or (0,15)]. RV; {0,2,0,2}. Max number of HARQ transmission; 4.

In this contribution, the view on the remaining issue for NB-IoT with 16QAM are provided, and initial results are provided to analysis the impact of  IRU, ITBS on 16QAM

# 2	Discussion

Frequency resource

Based on RAN1 design, 16QAM can be applied for the basic Rel-13 NPUSCH format1 with 3, 6 and 12 tones transmission. RAN4 has already defined the related requirement for format1 with QPSK.

In general, the purpose of 16QAM is to increase the peak date rate while retaining differentiation of LTE-MTC and NB-IoT. It is more benefit to schedule with 16QAM for small number of tones, considering the number of available RE is small for 3 or 6 tones.

Since the test purpose is to verify the 16QAM functionality, it may not need to specify the requirements with all the tones. With considering the test complexity, we prefer to specify the requirement with one tone configuration. Either 3 or 12 can be selected.

Proposal 1: Specify the UL 16 QAM requirement with one tone configuration, either 3 or 12 can be selected

Simulation assumption.

Regarding the number of RU configured and Index of TBS, from RAN1 aspect, 16QAM is applied for I_TBS from 14 to 21, and applied for I_RU from 0 to 6, the following TBS table can be supported in RAN1.

Table 1: TBS table for 16QAM

| I_TBS | I_RU |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |

In general, with large number of TBS, the coding rate is large, less coding gain is achieved, and with large number of RU, the related TBS is large and transmission period is long, which will increase the test and simulation time.

In Rel-13, the NPUSCH format1 requirement with both 3.75 KHz and 15KHz SCS were specified, the related FRC was introduced as

Table 2: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A16-1 | A16-2 | A16-3 | A16-4 | A16-5 |
| --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 | 15 | 15 |
| Number of allocated subcarriers | 1 | 1 | 3 | 6 | 12 |
| Diversity | No | No | No | No | No |
| Modulation | BPSK | BPSK | QPSK | QPSK | QPSK |
| ITBS / IRU | 0 / 1 | 0 / 1 | 3 / 0 | 7 / 0 | 9 / 0 |
| Payload size (bits) | 32 | 32 | 40 | 104 | 136 |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.22 | 0.44 | 0.56 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 288 | 288 | 288 |
| Total symbols per resource unit | 96 | 96 | 144 | 144 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 4 | 4 | 4 | 2 (when repetition = 2) 4 (when repetition > 2) |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |

Observation 1: small number of RU is considered for NPUSCH format1 requirements.

# 2	Initial simulation results

In this subsection, the performance with different RU and TBS, for different tones are provided.

Figure 1: 10% BLER for all TBS entries intended to be used for 16QAM in UL for 3 subcarriers allocation

Figure 2: 10% BLER for all TBS entries intended to be used for 16QAM in UL for 6 subcarriers allocation

Figure 3: 10% BLER for all TBS entries intended to be used for 16QAM in UL for 12 subcarriers allocation

Observation 2: Minor performance difference with different number of RU configured for 3, 6 and 12 subcarrier allocations.

Observation 3: Similar performance can be achieved for 3, 6 and 12 tones for given the number of RU.

Observation 4: About 0.5dB performance gap compared with I_TBS as 14 and 15 for N_RU=0.

In LTE Rel-13, the target SNR with 70%TP without repetition is about -3dB for 3 subcarrier allocations, and about -0.6dB for 12 subcarriers allocation.  To compare with LTE Rel-13 with QPSK, we prefer to choose the small number of I_TBS and I_RU for requirement with 16QAM.

Proposal 2: RAN4 applies (IRU, ITBS) = (0, 14) for NPUSCH format1 requirement with 16QAM

In summary, the following simulation assumption for 16QAM requirement can be considered

Proposal 3: RAN4 applies the following simulation assumption for NPUSCH format1 requirement with 16QAM

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T2R |
| SCS | 15KHz |
| Frequency Resource | Either 3 or 12 |
| Number of Repetition | 1 |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| I_RU | 0 |
| I_TBS | 14 |

# 3	Conclusion

In this contribution, the view on the test setup for NPUSCH format 1 with 16QAM was provided.

Proposal 1: Specify the UL 16 QAM requirement with one tone configuration, either 3 or 12 can be selected

Simulation assumption.

Observation 1: Small number of RU is considered for NPUSCH format1 requirements.

Observation 2: Minor performance difference with different number of RU configured for 3, 6 and 12 subcarrier allocations.

Observation 3: Similar performance can be achieved for 3, 6 and 12 tones for given the number of RU.

Observation 4: About 0.5dB performance gap compared with I_TBS as 14 and 15 for N_RU=0

Proposal 2: RAN4 applies (IRU, ITBS) = (0, 14) for NPUSCH format1 requirement with 16QAM

Proposal 3: RAN4 applies the following simulation assumption for NPUSCH format1 requirement with 16QAM

| Parameter | Value/Description |
| --- | --- |
| Number of antennas | 1T2R |
| SCS | 15KHz |
| Frequency Resource | Either 3 or 12 tone |
| Number of Repetition | 1 |
| Noise Estimation | Practical |
| Channel Estimation | Practical |
| Frequency offset | 0 |
| Time offset | 0 |
| I_RU | 0 |
| I_TBS | 14 |

# References

R4-2207200, WF on Rel-17 NB-IoT and eMTC performance requirement, Huawei