3GPP TSG-RAN WG4 Meeting #103-e	R4-2208096

Online, May 9-20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38133 | CR | draftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on introduction of relaxed RLM/BFD measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. According to RAN conclusion, the relaxation criteria start to be evaluated only after receiving the criteria configuration. However, RAN4 defines the relaxation conditions assuming good serving cell quality is always evaluated. This is misaligned with RAN conclusion. 2. The UE is not allowed to relax the measurements if the T310 timer is running. The condition “the UE has trigered T310 timer” is not accurate. 3. According to WID, the RLM/BFD relaxation is allowed for UE with short DRX periodicity/cycle. This needs to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the relaxation conditions to align with RAN conclusion. - Change the condition where relaxation is not allowed. - Add that UE is not allowed to relax RLM/BFD measurements when no DRX is used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability of RLM/BFD relaxation is misaligned with RAN conclusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1, 8.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1 >>

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state  and configured with lowMobilityEvaluationConnected and/or goodServingCellEvaluationRLM :

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	the UE has fulfilled low mobility criterion defined in TS 38.331 section 5.7.13.1 if lowMobilityEvaluationConnected is configured and goodServingCellEvaluationRLM is not configured, or

-	the UE has fulfilled good serving cell quality criterion defined in TS 38.331 section 5.7.13.2 if goodServingCellEvaluationRLM is configured and lowMobilityEvaluationConnected is not configured, or

-	the UE has fulfilled both low mobility criterion as defined in TS 38.331 section 5.7.13.1 and good serving cell quality criterion defined in TS 38.331 section 5.7.13.2, if both lowMobilityEvaluationConnected and goodServingCellEvaluationRLM are configured,

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM and CSI-RS based BFD on SCell, and

-	the UE has fulfilled low mobility criterion defined in TS 38.331 section 5.7.13.1 for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if lowMobilityEvaluationConnected is configured and goodServingCellEvaluationBFD is not configured, or

-	the UE has fulfilled good serving cell quality criterion defined in TS 38.331 section 5.7.13.2for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if goodServingCellEvaluationBFD is configured and lowMobilityEvaluationConnected is not configured, or

-	the UE has fulfilled both low mobility criterion as defined in TS 38.331 section 5.7.13.1 and good serving cell quality criterion defined in TS 38.331 section 5.7.13.2 for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell, if both lowMobilityEvaluationConnected and goodServingCellEvaluationBFD are configured.

The UE is allowed to apply the relaxed requirements defined in clause 8.1.2.4 for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3.4 for CSI-RS based radio link monitoring; Otherwise, UE shall apply the requirements defined in clause 8.1.2.2 for SSB based radio link monitoring and the requirements defined in clause 8.1.3.2 for CSI-RS based radio link monitoring.

The UE is not allowed to relax RLM measurements and apply the relaxed radio link monitoring requirements provided that at least one of the following conditions is met:

-	The UE sends out-of sync indications to the higher layers,

-	The timer T310 is running

-	No DRX is used

## << End of change 1 >>

## << Start of change 2 >>

#### 8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

The relaxed beam failure detection procedures specified in this clause apply provided for the UE supporting [connected mode power saving] and configured with lowMobilityEvaluationConnected and/or goodServingCellEvaluationBFD:

-	when the UE is not performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	the UE has fulfilled low mobility criterion defined in TS 38.331 section 5.7.13.1 if lowMobilityEvaluationConnected is configured and goodServingCellEvaluationBFD is not configured, or

-	the UE has fulfilled good serving cell quality criterion defined in TS 38.331 section 5.7.13.2 if goodServingCellEvaluationBFD is configured and lowMobilityEvaluationConnected is not configured, or

-	the UE has fulfilled both low mobility criterion as defined in TS 38.331 section 5.7.13.1 and good serving cell quality criterion defined in TS 38.331 section 5.7.13.2, if both lowMobilityEvaluationConnected and goodServingCellEvaluationBFD are configured.

-	when the UE is performing intra-band carrier aggregation configured with CSI-RS based RLM on SpCell and CSI-RS based BFD on SCell provided that:

-	the UE has fulfilled low mobility criterion defined in TS 38.331 section 5.7.13.1 for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if lowMobilityEvaluationConnected is configured and goodServingCellEvaluationBFD is not configured, or

-	the UE has fulfilled good serving cell quality criterion defined in TS 38.331 section 5.7.13.2 for CSI-RS based RLM on SpCell and for CSI-RS based BFD on serving cell if goodServingCellEvaluationBFD is configured and lowMobilityEvaluationConnected is not configured, or

-	the UE has fulfilled both low mobility criterion as defined in TS 38.331 section 5.7.13.1 and good serving cell quality criterion defined in TS 38.331 section 5.7.13.2 for CSI-RS based RLM on SpCell and for CSI-RS based BFD on a serving cell, if both lowMobilityEvaluationConnected and goodServingCellEvaluationBFD are configured.

The scenario and RS resource configurations in the set  defined in section 8.5.1 apply for this section.

The UE is allowed to apply the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection; otherwise, UE shall apply the requirements defined in clause 8.5.5 for SSB based beam failure detection and the requirements defined in clause 8.5.3 for CSI-RS based beam failure detection.

The UE is not allowed to apply the relaxed link recovery procedures provided that at least one of the following conditions is met:

-		The UE sends a beam failure instance indication to the higher layers,

-

The timer T310 is running

-	No DRX is used

## << End of change 2>>