3GPP TSG-RAN WG4 Meeting # 103-e		R4-2208151

# Electronic Meeting, May 09 – May 20, 2022

Agenda item:	9.7.2

Source: 	CATT

Title: 	Discussion on RRM test cases for FR1 HST

Document for:	Discussion

Introduction

In RAN4#102-e meeting, the core requirements of Rel-17 NR FR1 HST are completed. Although there is one open issue related to L1-SINR, the framework of core requirements is finalized. This contribution provides discussion on the RRM test case list.

Discussion

According to the work plan for performance part, the test cases should be designed and specified in performance part.

The enhanced RRM requirements for NR FR1 HST are summarized as below:

- Idle mode for inter-frequency measurement

- Connected mode: intrafrequency cell identification on activated SCell without MG

- Connected mode: intrafrequency cell identification on activated SCell with MG

- Connected mode: interfrequency measurement without MG

- Connected mode: interfrequency measurement with MG

- L1-RSRP measurement

In general, RAN4 should design test cases to verify new defined requirements.

Proposal 1: the test case list is proposed as following table:

| Core requirements | Test cases |
| --- | --- |
| Idle mode for inter-frequency measurement | Add one test case to test new requirements for cell reselection delay for newly/already detectable cell. DRX = 0.32s |
| Connected mode: intrarequency cell identification on activated SCell without MG | Add one test case for event triggered reporting in activated SCell in DRX mode |
| Connected mode: intrarequency cell identification on deactivated SCell without MG | Add one test case for event triggered reporting in activated SCell in DRX mode |
| Connected mode: intrafrequency cell identification on activated SCell with MG | Add one test case for event triggered reporting in activated SCell in DRX mode |
| Connected mode: interfrequency measurement without MG | Add test case for event triggered reporting |
| Connected mode: interfrequency measurement with MG | Add test case for event triggered reporting |
| L1-RSRP measurement | Add test case to verify L1-RSRP measurement |
| SCell activated/deactivated delay | The same as legacy. No need to add new test case |

Conclusion

In this contribution, we provide our consideration of test case list for FR1 HST. It is proposed to define the test cases according to the following list:

| Core requirements | Test cases |
| --- | --- |
| Idle mode for inter-frequency measurement | Add one test case to test new requirements for cell reselection delay for newly/already detectable cell. DRX = 0.32s |
| Connected mode: intrarequency cell identification on activated SCell without MG | Add one test case for event triggered reporting in activated SCell in DRX mode |
| Connected mode: intrarequency cell identification on deactivated SCell without MG | Add one test case for event triggered reporting in deactivated SCell in DRX mode |
| Connected mode: intrafrequency cell identification on activated SCell with MG | Add one test case for event triggered reporting in activated SCell in DRX mode |
| Connected mode: interfrequency measurement without MG | Add test case for event triggered reporting |
| Connected mode: interfrequency measurement with MG | Add test case for event triggered reporting |
| L1-RSRP measurement | Add test case to verify L1-RSRP measurement |
| SCell activated/deactivated delay | The same as legacy. No need to add new test case |

Reference