3GPP TSG-RAN WG4 Meeting #103-e	R4-2208175

# Electronic Meeting, May 9 – 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test case for interruptions at NR FR1 one SRS symbol with antenna port switching in asynchronous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-4-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case should be introduced for interruptions at NR FR1 for one SRS symbol with antenna port switching in asynchronous EN-DC. Sounding Reference Symbol Configuration with antenna port switching have not been defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test case for interruptions at NR FR1 for one SRS symbol with antenna port switching in asynchronous EN-DC. Define Sounding Reference Symbol Configuration with antenna port switching |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interruption requirements will not be tested on NR FR1 for one SRS symbol with antenna port switching in asynchronous EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections A.4.5.2.x2, A.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================= First Change Request ===========================

A.4.5.2.x2	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in asynchronous EN-DC

      - A.4.5.2.x2.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit periodic NR FR1 one SRS symbol with antenna port switching in asynchronous EN-DC, the UE can perform the SRS transmission with antenna port switching. The test will verify the interruption requirements on E-UTRA PCell and NR SCell in clause 8.2.1.2.18. Supported test configurations are shown in table A.4.5.2.x2.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.4.5.2.x2.1-2 and A.4.5.2.x2.1-3 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is E-UTRAN PCell, Cell2 is an activated NR SCell. The PCell and SCell are in FR1 band belongs to the IE txSwitchImpactToRx indication, or belongs to the IE txSwitchWithAnotherBand indication in UE srs-TxSwitch capability [14]. Cell3 is NR PSCell on FR1 NR aggressor carrier for UE transmitting SRS with antenna port switching.

The test consists of two successive time periods, with duration of T1 and T2, respectively. Throughout the test the UE shall be continuously scheduled on PCell and SCell. During T1 the UE shall be configured periodic one SRS symbol transmission with antenna port switching on PSCell. Immediately at the beginning of T2, the UE starts transmit the SRS signal with antenna port switching.

Table A.4.5.2.x2.1-1: Interruptions at SRS with antenna port switching test configurations in FR1

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, SCell FDD duplex mode, PSCell TDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, SCell TDD duplex mode, PSCell TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, SCell FDD duplex mode, PSCell TDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, SCell TDD duplex mode, PSCell TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.5.2.x2.1-2: General test parameters for E-UTRAN – NR FR1 interruptions at SRS with antenna port switching in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRA RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRA RF channel number 1. |
| Configured SCell |  | Cell2 | Configured activated SCell on NR RF channel number 2. |
| Configured PSCell |  | Cell3 | Configured PSCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3. |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | s | 5 |  |
| T2 | s | Sufficient long | UE shall perform periodic SRS transmission with antenna port switching during T2. T2 will be sufficient long for the test statistical performance for number of SRS transmit. |

Table A.4.5.2.x2.1-3: NR Cell specific test parameters for E-UTRAN – NR FR1 interruptions at SRS with antenna port switching in asynchronous EN-DC

| Parameter |  | Unit | Cell2 | Cell3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1,4 |  | FDD | TDD |
|  | Config 2,3,5,6 |  | TDD | TDD |
| TDD configuration | Config 1,4 |  | Not Applicable | TDDConfig.1.1 |
|  | Config 2,5 |  | TDDConf.1.1 | TDDConfig.1.1 |
|  | Config 3,6 |  | TDDConf.2.1 | TDDConfig.2.1 |
| BWchannel | Config 1,2,4,5 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  | Config 3,6 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| DL Initial BWP configuration | Config 1-6 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1-6 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL Initial BWP configuration | Config 1-6 |  | - | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1-6 |  | - | ULBWP.1.1 |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  | Config 2,5 |  | SR.1.1 TDD | SR.1.1 TDD |
|  | Config 3,6 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  | Config 2,5 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3,6 |  | CR.2.1 TDD | CR.2.1 TDD |
| RMC CORESET Reference Channel | Config 1,4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  | Config 2,5 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3,6 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD | TRS.1.1 TDD |
|  | Config 2,5 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  | Config 3,6 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| SMTC configuration |  |  | SMTC.1 | SMTC.1 |
| SSB configuration | Config 1,2,4,5 |  | SSB.1 FR1 | SSB.1 FR1 |
|  | Config 3,6 |  | SSB.2 FR1 | SSB.2 FR1 |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 kHz | 15 kHz |
|  | Config 3,6 |  | 30 kHz | 30 kHz |
| SRS Configuration | Config 1,2,4,5 | kHz | - | SRS.4 TDD |
|  | Config 3,6 |  | - | SRS.5 TDD |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,4,5 | kHz | 15 kHz | - |
|  | Config 3,6 |  | 30 kHz | - |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  | dBm/15kHz | -104 | -104 |
| Note2 | Config 1,2,4,5 | dBm/SCS | -104 | -104 |
|  | Config 3,6 |  | -101 | -101 |
| SS-RSRP Note3 | Config 1,2,4,5 | dBm/SCS | -87 | -87 |
|  | Config 3,6 |  | -84 | -84 |
|  |  | dB | 17 | 17 |
|  |  | dB | 17 | 17 |
| Io Note3 | Config 1,2,4,5 | dBm/9.36MHz | -58.96 | -58.96 |
|  | Config 3,6 | dBm/38.16MHz | -52.86 | -52.86 |
| Time offset to Cell1 Note 4 |  | s | 10 | -100 |
| Propagation condition |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

      - A.4.5.2.x2.2	Test Requirements

During the time duration T2, the interruption on NR SCell shall not exceed 2 slots for each SRS transmission with antenna port switching.

During the time duration T2, the interruption on E-UTRA PCell shall not exceed 2 slots for each SRS transmission with antenna port switching.

The rate of correct events observed during repeated tests shall be at least 90%.

========================= Second Change Request ===========================

A.3.24	SRS configuration

Table A.3.24-1: Sounding Reference Symbol Configuration for SCS=15kHz

|  | SRS.1 TDD | POS-SRS.1 |  |
| --- | --- | --- | --- |
| Field | Value |  | Comment |
| c-SRS | 12 | Same as NRB,c in the test case |  |
| b-SRS | 0 | n.a. |  |
| b-hop | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | n.a. |  |
| startPosition | 0 | 0 | resourceMapping setting |
| nrofSymbols | 4 | 4 |  |
| repetitionFactor | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n4 |  |
| combOffset-n2 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Periodic |  |
| periodicityAndOffset-p | sl40, 2 | sl160, 20 | SRS transmission periodicity |

Table A.3.24-2: Sounding Reference Symbol Configuration for SCS=30kHz

|  | SRS.2 TDD | POS-SRS.2 |  |
| --- | --- | --- | --- |
| Field | Value |  | Comment |
| c-SRS | 24 | Same as NRB,c in the test case |  |
| b-SRS | 0 | n.a. |  |
| b-hop | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | n.a. |  |
| startPosition | 0 | 0 | resourceMapping setting |
| nrofSymbols | 4 | 4 |  |
| repetitionFactor | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n4 |  |
| combOffset-n2 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Periodic |  |
| periodicityAndOffset-p | sl80, 4 | Sl320, 40 | SRS transmission periodicity |

Table A.3.24-3: Sounding Reference Symbol Configuration for SCS=120kHz

|  | SRS.3 TDD | POS-SRS.3 |  |
| --- | --- | --- | --- |
| Field | Value |  | Comment |
| c-SRS | 17 | Same as NRB,c in the test case |  |
| b-SRS | 0 | n.a. |  |
| b-hop | 0 | n.a. | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching | n.a. |  |
| startPosition | 0 | 0 | resourceMapping setting |
| nrofSymbols | 4 | 4 |  |
| repetitionFactor | n1 | n.a. | without repetition. |
| transmissionComb | n2 | n4 |  |
| combOffset-n2 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Periodic |  |
| periodicityAndOffset-p | sl320, 16 | Sl1280, 160 | SRS transmission periodicity |

Table A.3.24-4: Sounding Reference Symbol Configuration with antenna port switching for SCS=15kHz

| Field | SRS.1 TDD |  | Comment |
| --- | --- | --- | --- |
|  | SRS resource |  |  |
|  | 1 | 2 |  |
| c-SRS | 12 |  |  |
| b-SRS | 0 |  |  |
| b-hop | 0 |  | Frequency hopping is disabled |
| groupOrSequenceHopping | neither |  | No group or sequence hopping |
| freqDomainPosition | 0 |  | Frequency domain position of SRS |
| freqDomainShift | 0 |  |  |
| pathlossReferenceRS ssb-Index | 0 |  | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching |  |  |
| startPosition | 3 | 1 | resourceMapping setting |
| nrofSymbols | 1 | 1 |  |
| repetitionFactor | n1 |  | without repetition. |
| transmissionComb | n2 |  |  |
| combOffset-n2 | 0 |  | transmissionComb setting |
| cyclicShift-n2 | 0 |  |  |
| nrofSRS-Ports | port1 |  | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |  |
| periodicityAndOffset-p | sl40, 2 |  | SRS transmission periodicity |

Table A.3.24-5: Sounding Reference Symbol Configuration with antenna port switching for SCS=30kHz

| Field | SRS.2 TDD |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | SRS resources |  |  |  |
|  | 1 |  | 2 |  |
| c-SRS | 24 |  |  |  |
| b-SRS | 0 |  |  |  |
| b-hop | 0 |  |  | Frequency hopping is disabled |
| groupOrSequenceHopping | neither |  |  | No group or sequence hopping |
| freqDomainPosition | 0 |  |  | Frequency domain position of SRS |
| freqDomainShift | 0 |  |  |  |
| pathlossReferenceRS ssb-Index | 0 |  |  | SSB #0 is used for SRS path loss estimation |
| usage | antennaSwitching |  |  |  |
| startPosition | 3 | 1 |  | resourceMapping setting |
| nrofSymbols | 1 | 1 |  |  |
| repetitionFactor | n1 |  |  | without repetition. |
| transmissionComb | n2 |  |  |  |
| combOffset-n2 | 0 |  |  | transmissionComb setting |
| cyclicShift-n2 | 0 |  |  |  |
| nrofSRS-Ports | port1 |  |  | Number of antenna ports used for SRS transmission |
| resourceType | Periodic |  |  |  |
| periodicityAndOffset-p | sl80, 4 |  |  | SRS transmission periodicity |