3GPP TSG-RAN WG4 Meeting #103-e                                                             R4-2208223

Electronic Meeting, May 09 – May 20, 2022

Source: 	CATT

Title: 	Simulation results for PUSCH demodulation requirements for FR2 HST

Agenda item:	9.8.4.2.1

Document for:	Approval

- Introduction

In RAN4#102-e, a way forward [1] on FR2 HST demodulation was approved

In this contribution, we provide our simulation results for PUSCH demodulation following latest agreements.

- Simulation results

As per the WF [1, 2, 3], the simulation assumptions for PUSCH requirement are shown in Table A-1.

The simulation results for PUSCH requirement for FR2 HST are shown in Table2-1.

Table 2-1, Simulation results for PUSCH requirement for FR2 HST

| PUSCH |  |  |  |  |  |  |  | SNR@70% of maximum throughput（dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Channel | Frequency offset | DMRS mapping type | PUSCH data length | SCS and BW | MCS | RS configuration | Ideal | Impairment |
| Bi-directional scenario B | AWGN | 19444Hz | B | 10 | 120kHz, 50MHz | MCS19 | DMRS1+0 | 10.94 | 13.44 |
|  |  |  |  |  |  | MCS19 | DMRS1+1 | 10.89 | 13.39 |
|  |  |  |  |  |  | MCS19 | DMRS1+1+1 | 10.89 | 13.39 |
|  |  |  |  |  | 120kHz, 200MHz | MCS19 | DMRS1+0 | 10.45 | 12.95 |
|  |  |  |  |  |  | MCS19 | DMRS1+1 | 10.67 | 13.17 |
|  |  |  |  |  |  | MCS19 | DMRS1+1+1 | 10.62 | 13.12 |

- Conclusion

This contribution provides our simulation results for PUSCH demodulation requirements for FR2 HST for alignment.

- References

- R4-2207235, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, RAN4#102-e

- R4-2203006, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, RAN4#101-bis-e

- R4-2120703, WF on BS demodulation requirement for FR2 HST, Nokia, Nokia Shanghai Bell, Samsung, RAN4#101-e

- Annex

Table A-1, Simulation assumption for PUSCH requirement for FR2 HST

| Parameter |  | Value |
| --- | --- | --- |
| Channel Model |  | HST FR2 mode based on Bi-directional scenario B, the details can be referred as R4-2115726 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| MCS |  | 19 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos0, Pos1, Pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | 50MHz and 200MHz |
| resource | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |