3GPP TSG-RAN WG4 Meeting #103-e 	R4-2208256

Electronic Meeting, 9-20 May., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Agenda item:	9.11.2.1.1

Document for:	Information

Introduction

In this contribution simulation results on PDSCH demodulation performance in inter-cell interference using MMSE-IRC receiver are presented. The simulation assumptions are as follows:

| Parameters | Serving cell FDD |  | Serving cell TDD |  | Interferer cell(s) |
| --- | --- | --- | --- | --- | --- |
| Bandwidth/SCS | 10 MHz/ 15 KHz |  | 40 MHz/30 KHz |  | Same as serving cell |
| Cell ID | 0 |  | 0 |  | 1,2 |
| TDD configuration | N/A |  | 7DS2U, S=6D+4G+4U |  | Same as serving cell |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Periodicity 20 slots; Offset 10 for CSI-RS resource 1 and 2; 11 for CSI-RS resource 3 and 4 |  | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Periodicity 40 slots; Offset 20 for CSI-RS resource 1 and 2; 21 for CSI-RS resource 3 and 4 |  | Same as serving cell |
| PDCCH allocation | symbols #0 and #1 of each slot for PDCCH |  |  |  | Randomly modulated QPSK symbols #0 and #1 of each slot for PDCCH |
| PDSCH configuration | Type A, Start symbol 2, Duration 12​, PRB bundling size 2, Full PRB allocation |  |  |  | Same as serving cell |
| DMRS configuration | Type 1, Single symbol front loaded with 1 additional DMRS, number of CDM groups without data 1, |  |  |  | 2 |
| MCS | MCS 13 (16QAM, CR=0.5) |  |  |  | 16 QAM randomly modulated symbols |
| Rank | 1 |  |  |  | random rank with 70% and 30% probability for rank 1 and rank 2 transmissions in the interfering cell(s) |
| PDSCH precoding | Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2. |  |  |  | Same as serving cell |
| Time Offset | 0 us | 0 us |  | TDD: 1.5 us, -0.5 us FDD: 3 us, -1us |  |
| Frequency offset | 0 Hz | 0 Hz |  | 300 Hz, -100 Hz |  |
| Number of HARQ Processes | 4 | 8 |  | N/A |  |
| Maximum HARQ transmission | 4 | 4 |  | N/A |  |
| Antenna configuration | 2x2, 2x4 |  |  | Same as serving cell |  |
| Antenna Correlation | ULA Low |  |  | Same as serving cell |  |
| Propagation condition | Homogeneous: TDLC300-100 Heterogeneous: TDLA30-10 |  |  | Same as serving cell |  |
| INR levels | N/A |  |  | Homogenous: 7.77 dB, 2.29 dB Heterogenous: 7.58 dB |  |
| Metric | SNR to achieve 70% of maximum throughput |  |  |  |  |

Simulation Results

Simulation Results for FDD

Simulation results for FDD network deployment are presented inTable 1 .

Table 1 FDD deployment simulation results

| Channel Model | INR1 | INR2 | Antenna Config | SNR (dB) at 70 % throughput, IRC | SNR(dB) at 70 % throughput, MRC | IRC receiver gain (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| TDLA | 7.58 dB | NA | 2Tx 2Rx | 12.7 | 10.0 | 2.7 |
|  |  |  | 2Tx 4Rx | 9.3 | 7.1 | 2.2 |
| TDLC | 7.77 dB | 2.29 dB | 2Tx 2Rx | 14.2 | 12.3 | 1.9 |
|  |  |  | 2Tx 4Rx | 10.7 | 8.6 | 1.9 |

  - Simulation Results for TDD

Simulation results or TDD network deployment are presented in Table 2.

Table 2 TDD deployment simulation results

| Channel Model | INR1 | INR2 | Antenna Config | SNR(dB) at 70 % throughput, IRC | SNR(dB) at 70 % throughput, MRC | IRC receiver gain (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| TDLA | 7.58 dB | NA | 2Tx 2Rx | 12.9 | 10.2 | 2.7 |
|  |  |  | 2Tx 4Rx | 9.4 | 7.8 | 1.6 |
| TDLC | 7.77 dB | 2.29 dB | 2Tx 2Rx | 14.4 | 12.7 | 1.7 |
|  |  |  | 2Tx 4Rx | 11.1 | 9.0 | 2.1 |

- References

- R4-2207242- WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC