3GPP TSG-RAN WG4 Meeting # 103-e			                                  R4-2208360

Electronic Meeting, May 9 – May 20, 2022

Agenda item:	9.12.6.4

Source: 	OPPO

Title: 	Discussion on timing requirements for NR NTN

Document for:	Discussion

Introduction

This paper will provide our consideration on the timing requirements for NR NTN.

Discussion

2.1 Application time of TA adjustment

- Issue 2-4-3: Application time of TA adjustment - Agreement: - RAN4 shall introduce the TA adjustment delay requirement for NTN in TS38.133 if the application time of TA adjustment upon TAC considering newly introduced K_offset and K_mac parameters is specified in RAN1 specification.

In the RAN4 meeting #101e, it was agreed to introduce the TA adjustment delay for the TA adjustment upon TAC.

In the RAN1 meeting #104-bis and #105e, it was agreed to introduce K_offset to enhance the adjustment of uplink transmission timing as follows.

RAN1#104-bis: Agreement: Confirm the following working assumption: Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

RAN1#105e: Agreement: If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+Kmac, where µ is the SCS configuration for the PUCCH. Note: Here K_mac is assumed to have the unit of the PUCCH slot. This can be revisited after the K_mac signaling design is finalized.

From the agreement, we can see that if it is uplink transmission timing, the adjustment slot is at uplink slot. And K_offset is agreed to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command. We suggest the timing advance adjustment delay should be introduced by the parameter ‘Koffset’ and Koffset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2].

Proposal 1: For Application time of TA adjustment, the following text proposal can be introduced for TS 38.133:

7.3C.2.1 Timing Advance adjustment delay UE shall adjust the timing of its uplink transmission timing at time uplink slot n+ k+1+Koffset for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3], Koffset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2]. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

Conclusion

In this contribution, we provide our considerations on the timing requirements for NR NTN and the following proposals.

Proposal 1: For Application time of TA adjustment, the following text proposal can be introduced for TS 38.133:

7.3C.2.1 Timing Advance adjustment delay UE shall adjust the timing of its uplink transmission timing at time uplink slot n+ k+1+Koffset for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3], Koffset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2]. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

- References

- R4-2120310 , WF on GNSS-related and timing requirements for NR NTN_final2, Xiaomi