3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208361

Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to the timing requirements for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  |  | Date: |  | 2022-04-19 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For NTN scenario, the uplink and downlink timing is not aligned. So it is necessary to introduce the Koffset to enhance the adjustment of uplink transmission timing. And in order to avoid the uplink and downlink timing, the slot or the subframe should be distinguished. But how to enhance and how to distinguished is missed in the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Considering the agreement in the RAN4 and RAN1, the Koffset should be delayed backward on the time slot of the current uplink transmission timing at time uplink slot n+ k+1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The uplink and downlink timing will be misaligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | 38.533 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

### 7.3C.2.1	Timing Advance adjustment delay

UE shall adjust the timing of its uplink transmission timing at time uplink slot n+ k+1+Koffset for a timing advance command received in time slot n, and the value of k is defined in clause 4.2 in TS 38.213 [3], Koffset is a parameter configured by higher layer as specified in [TS 38.213 clause 4.2]. The same requirement applies also when the UE is not able to transmit a configured uplink transmission due to the channel assessment procedure.

# < End of change 1>