3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208376

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on test case for Selection / Reselection of relay UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The delay requirements of reselection / reselection of relay UE were defined in clause 12.10 and the corresponding test cases should also be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test case for delay requirements of reselection / reselection of relay UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The delay requirements of reselection / reselection of relay UE cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.1.X1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### A.9.1.X1	 Selection / Reselection of relay UE

#### A.9.1.X1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to selection / reselection of relay UE defined in clauses 12.10. In the test the UE under test is configured with PCell and is configured with resource pools for NR sidelink discovery message as required for remote UE operation.

This test is applicable to UEs capable of NR sidelink communication and sidelink discovery, and further support the optional feature of sidelink remote UE operation.

The test parameters are given in Table A.9.1.X1.1-1, Table A.9.1.X1.1-2, Table A.9.1.X1.1-3, Table A.9.1.X1.1-4, and Table A.9.1.X1.1-5 below. The test consists of one active serving cell (cell 1), one remote UE and two active Sidelink relay UEs (Sidelink Relay UE 1, Sidelink Relay UE 2). The relay UEs are configured in mode 2 to be transmitting relay discovery messages every discovery period, which is determined by resource reservation period indicated by sl-ResourceReservePeriodList.

The test system shall ensure that the remote UE under test has transmitted SidelinkUEInformationNR message and has been configured with the sidelink discovery resource pool and sidelink communication resource pool for relay operation prior to the start of the test.

The tests consist of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively.

During T1, RSRP of cell 1 is kept higher than threshHighRemote (within sl-remoteUE-Config), and the remote UE is not required to perform relay UE selection.

During T2, RSRP of cell 1 is configured to be lower than threshHighRemote. The UE is expected to start looking for relay UE and request the serving cell for sidelink communication resources for communicating with a candidate sidelink relay UE. The test system shall ensure that the UE under test transmits the SidelinkUEInformationNR message (requesting the sidelink communication resources) and has been configured the resource pool prior to end of T2 duration. During T2, the SD-RSRP of Sidelink Relay UE 1 and Sidelink Relay UE 2 is configured to be lower than the detection threshold and no relay UE will be available for the remote UE under test.

During T3, the SD-RSRP of Sidelink Relay UE 1 is raised above the threshold sl-RSRP-Thresh and the UE is expected to perform relay selection to Sidelink Relay UE 1. The test system can determine that the remote UE has selected a relay by monitoring the configured sidelink communication resource for the Prose direct link establishment request message to the relay UE.

During T4, the UE is expected to complete the sidelink connection establishment with the relay UE. Note that the RSRP of the serving cell (cell 1) and the SD-RSRP of sidelink relay UEs is kept unchanged during T3 and T4. The period T4 ends when Sidelink Relay UE1 sends the sidelink communication response message back to the remote UE.

During T5, SD-RSRP of Sidelink Relay UEs are modified such that the remote UE is expected to reselect to Sidelink Relay UE2.

Table A.9.1.X1.1-1: Test parameters for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| Active cell |  | Cell 1 | Serving NR cell will broadcasting SIB12 |
| Active Relay UEs |  | Relay UE 1, Relay UE 2 | Transmitting relay discovery message |
| CP length of Cell 1 |  | Normal |  |
| T1 | ms | 100 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [1] sec if UE does not transmit SidelinkUEInformationNR during this period. |  |
| T3 | s | 1 |  |
| T4 | s | 2 |  |
| T5 | s | 1 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

Table A.9.1.X1.1-2: Sidelink discovery configuration for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Resource pool configuration for sidelink discovery |  | As specified in Table A.3.21.2-2 Note 2 | IE values unless specified otherwise in this test. |
| sl-TimeResource included in sl-DiscTxPoolSelected and sl-DiscRxPool |  | 0000111100 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel included in sl-DiscTxPoolSelected and sl-DiscRxPool |  | 2 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize included in sl-DiscTxPoolSelected and sl-DiscRxPool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| sl-ResourceReservePeriodList | s | 0.04 | Indicates the resource reservation period for discovery period |
| threshHighRemote within sl-remoteUE-Config |  | -91dBm |  |
| sl-RSRP-Thresh within sl-remoteUE-Config |  | -91dBm |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: This test is according to the principle defined in section A.3.21. |  |  |  |

Table A.9.1.X1.1-3: Sidelink Communication configuration for selection / reselection of relay UE test

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | - | 1 | HD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Sidelink Communication configuration |  | - | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
| sl-TimeResource |  | - | 0000000011 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| Number of Active Sidelink Relay UEs |  | - | 2 | Active Sidelink UE i = 0, 1 |
| Active Sidelink Relay UEs (UE i = 0, 1) | PSCCH Reference Measurement Channel | - | CC.1A HD | As specified in Table A.3.21.3-1 |
|  | PSSCH Reference Measurement Channel | - | CD.1A HD | As specified in Table A.3.21.3-2 |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool | - | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool | - | floor(i)x10 | Indicates the lowest RB index of the subchannel with the lowest index for Sidelink Relay UE i = 0 starts RB=0; Sidelink Relay UE i = 1 starts RB=10. |
|  | PSBCH-RSRP | dBm/30kHz | -95 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.9.1.X1.1-4: Sidelink Relay UE specific test parameters for selection / reselection of relay UE test for NR FR1

| Parameter | Unit | Relay UE 1 |  |  |  |  | Relay UE 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  | 1 (HD carrier in Band n47 or n38) |  |  |  |  |  |  |  |  |  |
| BWchannel Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |  |  |  |  |  |  |  |
| SCS | kHz | 30 |  |  |  |  |  |  |  |  |  |
| Sidelink Discovery resource pool configuration |  | As specified in Table A.9.1.X1.1-2 |  |  |  |  |  |  |  |  |  |
| Transmission frequency |  | Every discovery period (40ms) |  |  |  |  |  |  |  |  |  |
| Resource allocation |  | Non-overlapping PRBs |  |  |  |  |  |  |  |  |  |
| Note1 | dBm/30 kHz | -97 |  |  |  |  |  |  |  |  |  |
|  | dB | -inf | -inf | 10.5 | 10.5 | -1.5 | -inf | -inf | -inf | -inf | 10.5 |
| SD-RSRP / SL-RSRP Note2, Note 3 | dBm/30 kHz | -inf | -inf | -86.5 | -86.5 | -98.5 | -inf | -inf | -inf | -inf | -86.5 |
| Antenna Configuration | - | 1x2 |  |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SD-RSRP / SL-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: PSCCH-DMRS Es/Iot is set the same as PSSCH-DMRS Es/Iot. Note 4: This test is according to the principle defined in section A.3.21.2. |  |  |  |  |  |  |  |  |  |  |  |

Table A.9.1.X1.1-5: Cell specific test parameters for selection / reselection of relay UE test for NR FR1

| Parameter |  | Unit | Cell1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  | 2 |  |  |  |  |
| Serving/Non-serving |  |  | Serving |  |  |  |  |
| Duplex Mode | Config 1 |  | FDD |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| Channel Bandwidth (BWchannel) | Config 1,2 |  | 10: NRB,c = 52 |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| DRX Cycle |  |  | N/A |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC Configuration |  |  | SMTC.2 |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |
| drx-Configuration |  |  | N/A |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  |  | R.11 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  |  | OP.16 FDD |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -97 |  |  |  |  |
| Note2 | Config 1,2, | dBm/SCS | -97 |  |  |  |  |
|  | Config 3 |  | -94 |  |  |  |  |
|  |  | dB | 10.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| RSRP Note3 |  | dBm/15 kHz | -86.5 | -95.5 | -95.5 | -95.5 | -95.5 |
| Propagation condition |  |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.7.5.12.2	Test Requirements

Sidelink relay UE selection delay is defined as the time from the beginning of time period T3 to the moment when the UE selects the Sidelink Relay UE1 and transmits the PC5-SP direct communication setup message using Sidelink Communications.

The test system shall verify that the sidelink relay UE selection delay is less than 680ms.

NOTE:	The sidelink relay UE selection delay can be expressed as (Tevaluate, ProSe_Relay_intra + 40ms).

Sidelink relay UE reselection time is defined as the time from the beginning of time period T5 to the moment when the UE reselects to Sidelink relay UE2 and transmits the direct communication setup message using Sidelink Communications.

The test system shall verify that the sidelink relay UE reselection delay is less than 840ms.

NOTE:	The sidelink relay UE reselection delay can be expressed as (Tmeasure,ProSe_Relay_Intra + Tevaluate, ProSe_Relay_intra + 40ms).

<End of Change 1>