3GPP TSG-RAN WG4 Meeting # 103-e 		R4-2208383

Electronic Meeting, May 09 – May 20, 2022

Title:	[Draft] LS on GP positioning between two UL SRS resource sets

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Hiromasa Umeda

hiromasa.umeda@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 has further discussed the Question 2 captured in R4-2202413 and concludes a following:

It is essential to clarify on which symbol(s) UL SRS antenna switching occurs in a time interval between two UL SRS resource sets when the time interval is larger than minimum GP in order to make maximum use of resources like PUSCH/PUCCH in the time interval due to the following reason.

It is noted that whether or not minimum GP is defined within the time interval is FFS.

It’s RAN4’s understanding that in some cases, e.g., usage of 1T4R is assumed, RF performance including physical antenna and associated RF chain which mapped to an antenna port to transmit the last UL SRS resource in the first UL SRS resource set may be even poorer than the one mapped to an antenna port to transmit the first UL SRS resource of the next UL SRS resource set or vice versa. With respect to the former case, placing a symbol(s) for switching on the first symbol in the time interval is a reasonable selection while with respect the latter case, placing a symbol(s) for switching on the last symbol in the time interval is a reasonable selection. In another case, the UE may prefer to use the best performance antenna port(s) which is/are not among the aforementioned antenna ports if the time interval is long. In this case, outermost symbols in the time interval may be needed for switching to allow the use of the best performance antenna port(s) in the time interval.

It is also note that the above principle also applies to symbols outside two UL SRS resource sets.

# 2	Actions

To RAN1

ACTION: RAN4 respectfully asks RAN1 to take the above information into consideration in their work.

# 3	Dates of next TSG RAN WG 4 meetings

RAN4#104	2022-08-22 2022-08-26 		Toulouse, FR