# 3GPP TSG-RAN WG4 Meeting #103-e                                                              R4-2208418

# Electronic Meeting, May 09 – May 20, 2022

Title:	Draft LS on UE capability and network assistant signalling for CRS interference mitigation in the scenario with overlapping spectrum for LTE and NR with 30kHz SCS

Release:	Rel-17

Work Item:	NR_perf_enh2_Demod

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

Name:	Shiyuan Wang

E-mail Address:	wangshiyuan@chinamobile.com

Attachments:

1. Overall Description:

Within the Release 17 work item on further enhancement on NR demodulation performance (NR_perf_enh2_Demod), RAN4 has discussed UE capability and network assistant signalling for CRS interference mitigation (CRS-IM) in the following two scenarios with overlapping spectrum for LTE and NR as illustrated in TR 38.833:

- Scenario 1(DSS scenario): Serving and neighbouring cells are both operating with dynamic spectrum sharing (DSS) of NR and LTE. The NR UE in the serving cell is configured with lte-CRS-ToMatchAround, and is suffering interference from the LTE CRS of neighbouring cells.

- Scenario 2 (non-DSS scenario): Serving cell is operating in NR, and neighbouring cells are operating in LTE. The NR UE in the serving cell is suffering interference from the LTE CRS of neighbouring cells.

In the past RAN4#102-e meeting, RAN4 has agreed the UE capability and network assistant signalling for CRS interference mitigation (CRS-IM) in the Scenario1 and Scenario 2 with serving cell 15kHz SCS, and the corresponding LS R4-2207238 was sent to RAN2.

In this RAN4#103-e meeting, the UE capability and network assistant signalling for CRS interference mitigation (CRS-IM) in Scenario 2 with serving cell 30kHz has been discussed and agreed to be introduced. Following agreements are reached by RAN4:

- From the RAN4 perspective, the following new NR UE capabilities are agreed to be defined for CRS-IM:

- Capability #4: NR UE capable of performing CRS-IM in scenario 2 with 30kHz SCS without Rel-17 new network assistant signalling on LTE channel bandwidth.

- Capability #5: NR UE capable of performing CRS-IM in scenario 2 with 30kHz SCS with Rel-17 new network assistant signalling on LTE channel bandwidth.

- The granularity of the above capabilities is Per Feature Set per CC.

- The above capabilities are applicable for FR1 only without FDD/TDD difference.

- The above capabilities are optional for UE to report.

- To perform CRS-IM in Scenario 2, RAN4 has agreed that the UE should have the following default network configuration assumptions for each neighbour LTE cell:

- For scenario 2, the CRS port number is 4.

- Network-based CRS interference mitigation (i.e., CRS muting) is not enabled by crs-IntfMitigConfig in TS 36.331 for scenario 2.

- For scenario 2, MBSFN is not configured.

- With the above default network configuration assumptions, RAN4 has agreed that for UE supporting Capability #4, the UE can perform CRS-IM without Rel-17 new RRC network assistant signalling in scenario 2 with 30 kHz SCS when MeasObjectEUTRA IE is configured and the configured measurement gaps overlap with neighbour LTE cell PBCH position.

- Meanwhile, new RRC based network assistant signalling is agreed to be introduced to assist CRS-IM in Rel-17, and the Rel-17 new RRC signalling are optionally to be indicated to UE supporting Capability #4.

- Regarding the content of the Rel-17 new RRC network assistant signalling, for each neighbour LTE cell, RAN4 reach following agreements for Scenario 2 with serving cell 30kHz SCS:

- Downlink Centre frequency

  - For scenario 2, the downlink centre frequency can be optionally signalled to UE.

- Channel bandwidth

  - For scenario 2 with 30kHz SCS, the LTE channel bandwidth can be optionally signalled to UE supporting Capability #4, and should be signalled to UE supporting Capability #5 but not supporting Capability #4.

- CRS port number

  - For scenario 2, the CRS port number can be optionally signalled to the UE, if the above default CRS port number configuration assumption is not valid

- Cell ID

  - For scenario 2, the physical Cell ID can be optionally signalled to UE.

- v-Shift

  - For scenario 2, the v-Shift information can be optionally signalled to UE.

  - If Cell ID information is informed, then v-Shift information shall not be signalled to UE.

- CRS muting

  - For scenario 2, the configuration of CRS muting can be optionally signalled to the UE if the above default CRS muting assumption is not valid.

- MBSFN configuration

  - For scenario 2, the MBSFN subframe configuration can be optionally signalled to the UE if the above default MBSFN configuration assumption is not valid.

- The above information is signalled under each serving cell with flexibility to support per UE level configuration with up to 8 interference cell information.

2. To RAN WG2 group.

ACTION: RAN4 respectfully request RAN2 to take the above information into account and design the corresponding UE capability and network assistance signalling for CRS-IM in scenario with overlapping spectrum for LTE and NR with 30kHz SCS.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN WG4 Meeting #104 	22 Aug - 26 Aug 2022

TSG-RAN WG4 Meeting #104-bis-e 	10 Oct - 19 Oct 2022	                                  Online