3GPP TSG-RAN WG4 Meeting #103-e	R4-2208501

Online, 9th May – 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7153 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HO with PSCell in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2_Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correction on HO with PSCell in 36.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the TPCell_DU in PSCell change in EN-DC to EN-DC. if it is target NR PSCell is in FR2, TPCell_DU = 0 ms. - Editorial correction in PSCell change in EN-DC to EN-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for HO with PSCell is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

## 5.8	EN-DC Handover with PSCell

### 5.8.1	Introduction

When the UE receives a RRC message implying handover with PSCell change, the UE shall be ready to start the transmission of the new uplink PRACH channel on target E-UTRA PCell within DHOwithPSCell_PCell ms from the end of the last TTI containing the RRC command, and UE shall be ready to start the transmission of the new uplink PRACH channel on target NR PSCell within DHOwithPSCell_PSCell ms from the end of the last TTI containing the RRC command.

Where

DHOwithPSCell_PCell equals the maximum RRC procedure delay defined in clause 11.2 in TS 36.331 [2] plus the interruption time stated in clause 5.8.1.1.

DHOwithPSCell_PSCell is the PSCell addition delay stated in clause 5.8.1.2.

#### 5.8.1.1	Handover with PSCell Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH excluding the RRC procedure delay. This requirement applies when UE is not required to perform any synchronisation procedure before transmitting on the new PRACH or on the new PUSCH.

When Handover with PSCell is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing ms

Where:

Tsearch is same as the Tsearch defined in section 5.1.2.1.2.1.

TIU is same as the one defined in section5.1.2.1.2.1.

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 25 ms if source NR PSCell and target NR PSCell are in the same FR.  Tprocessing = 45 ms if source NR PSCell and target NR PSCell are in the different FR.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.1 for intra-frequency handover and Clause 8.1.2.3.1 for inter-frequency handover.

#### 5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements

When Handover with PSCell is commanded, the NR PSCell changing delay shall be less than DHOwithPSCell_PSCell:

DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + TPCell_DU + 2 ms

TRRC_delay is maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 25 ms if source NR PSCell and target NR PSCell are in the same FR.  Tprocessing = 45 ms if source NR PSCell and target NR PSCell are in the different FR.

Tsearch is same as Tsearch in section 7.31.2

T∆ is same as T∆ in section 7.31.2.

TPSCell_ DU is same as TPSCell_ DU in section 7.31.2.

TPCell_DU is the delay uncertainty due to PCell RACH preamble transmission defined in TS 38.213 [39] if target NR PSCell is in FR1, otherwise TPCell_DU = 0 ms.

Trs is same as Trs in section 7.31.2.

In FR1 and FR2, the NR PSCell is known if it has been meeting the following conditions:

During the last 5 seconds before the reception of the HO with PSCell command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	One of the SSBs measured from the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 9.3 of TS 38.133 [50],

-	One of the SSBs measured from NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 9.3 of TS 38.133 [50].

otherwise it is unknown.

# < End of change 1>