3GPP TSG-RAN WG4 Meeting #103-e                 	R4-2208502

Online, 9th May – 20th May, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on test cases for HO with PSCell

Agenda item:	9.9.2.2

Document for:	Discussion

Introduction

RAN4 has completed the core part requirements for HO with PSCell in RAN4#102e and RRM performance part for HO with PSCell is started in this meeting.

In this contribution we discuss and give our view on the test cases for HO with PSCell.

Discussion

The core requirements for HO with PSCell have been almost completed in last meeting and have been specified in [1], and in this meeting, we will start performance requirements for HO with PSCell.

Looking at the core requirements [1] and the WID [2], we would need to consider from below aspects when we start to work to design the test cases:

| Scenarios | Parallel / Sequential processing | PCell |  | PSCell |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Source PCell | Target PCell | Source PSCell | Target PSCell |
| NR-SA to EN-DC | Parallel processing | FR1 Cell | Known E-UTRA Cell unknown E-UTRA Cell |  | Known FR1 Cell unknown FR1 Cell Known FR2 Cell unknown FR2 Cell |
|  | Sequential processing |  |  |  | Unknown FR1 Cell Unknown FR2 Cell |
| EN-DC to EN-DC | Parallel processing | E-UTRA Cell | Known E-UTRA Cell unknown E-UTRA Cell (considering intra-F and inter-F) | FR1 Cell FR2 Cell | Known FR1 Cell Unknown FR2 Cell Known FR2 Cell Unknown FR2 Cell |
| NE-DC to NE-DC (FR1 PCell) | Parallel processing | FR1 Cell | Known FR1 Cell Unknown FR1 Cell (considering intra-F and inter-F) | E-UTRA Cell | Known E-UTRA Cell unknown E-UTRA Cell |
| NR-DC to NR-DC (FR1+FR2 NR-DC) | Parallel processing | FR1 Cell | Known FR1 Cell Unknown FR1 Cell (considering intra-F and inter-F) | FR2 Cell | Known FR2 Cell Unknown FR2 Cell |
|  | Sequential processing |  |  |  | Unknown FR2 Cell |

If we take all these aspects into account and each configuration as a test case, it will be a huge test case list which is up to 64 test cases. Below table lists the test cases numbers for each scenario:

| Scenarios | Test cases number |
| --- | --- |
| NR-SA to EN-DC | 12 |
| EN-DC to EN-DC | 32 |
| NE-DC to NE-DC (FR1 PCell) | 8 |
| NR-DC to NR-DC (FR1+FR2 NR-DC) | 12 |
| Total | 64 |

Test cases are designed to verify the relative core requirements. However, it does not mean we need to test all possible cases. It is not realistic and necessary to test all these test cases. We would need to choose some to design sufficient test cases crossing known/unknown target PCell / PSCell, FR1 / FR2 cells, intra-F / inter-F cells.

To avoid duplicated work and consider the tight schedule that there are only two meetings for performance/test cases part, moderator proposed a list of test cases for triggering the work split in email before meeting as below:

TC No.; TC; PCell Handover; PSCell addition/change; Purpose; Clause; Note. 1; Handover with PSCell from NR SA to EN-DC with sequential processing; NR SA-EUTRA handover with known target PCell Source Pcell in FR1; SMTC of target unknown PSCell is present in RRCConnectionReconfiguration Target PSCell in FR1; To verify delay requirements for handover with PSCell from NR SA to EN-DC with sequential processing; A.6.3.1.x1. 2; Handover with PSCell from NR SA to EN-DC with parallel processing; NR SA-EUTRA handover with known target PCell Source Pcell in FR1; SMTC of target known PSCell is NOT present in RRCConnectionReconfiguration Target PSCell in FR1; To verify delay requirements for handover with PSCell from NR SA to EN-DC with parallel processing; A.6.3.1.x2. 3; Handover with PSCell from NR SA to EN-DC with sequential processing; NR SA-EUTRA handover with known target PCell Source Pcell in FR1; SMTC of target unknown PSCell is present in RRCConnectionReconfiguration Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR SA to EN-DC with sequential processing; A.7.3.1.x1; Option 1: No test case Option 2: Only requirements for PSCell are verified. 4; Handover with PSCell from NR SA to EN-DC with parallel processing; NR SA-EUTRA handover with known target PCell Source Pcell in FR1; SMTC of target known PSCell is NOT present in RRCConnectionReconfiguration Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR SA to EN-DC with parallel processing; A.7.3.1.x2; Option 1: No test case Option 2: Only requirements for PSCell are verified. 5; Handover with PSCell from NE-DC to NE-DC with known target PSCell; Intra-frequency handover from FR1 to FR1 with known target PCell; Target known E-UTRA PSCell; To verify delay requirements for handover with PSCell from NE-DC to NE-DC; A.4A.1.x1. 6; Handover with PSCell from NE-DC to NE-DC with unknown target PSCell; Inter-frequency handover from FR1 to FR1 with known target PCell; Target unknown E-UTRA PSCell; To verify delay requirements for handover with PSCell from NE-DC to NE-DC; A.4A.1.x2. 7; Handover with PSCell from NR-DC to NR-DC with parallel processing; Inter-frequency handover from FR1 to FR1 with unknown target PCell; SMTC of target known PSCell is present in reconfigurationWithSync Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR-DC to NR-DC with parallel processing; A.7.3.1.x3; Option 1: No test case Option 2: Only requirements for PSCell are verified. 8; Handover with PSCell from NR-DC to NR-DC with sequential processing; Inter-frequency handover from FR1 to FR1 with unknown target PCell; SMTC of target unknown PSCell is present in targetcellSMTC-SCG-r16 but not in reconfigurationWithSync Target PSCell in FR2; To verify delay requirements for handover with PSCell from NR-DC to NR-DC with sequential processing; A.7.3.1.x4; Option 1: No test case Option 2: Only requirements for PSCell are verified. 9; Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1; E-UTRA intra-frequency handover; Source PSCell in FR1 Target known PSCell in FR1; To verify delay requirements for handover with PSCell from EN-DC to EN-DC; A.4.3.x1. 10; Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR2; E-UTRA inter-frequency handover; Source PSCell in FR1 Target known PSCell in FR2; To verify delay requirements for handover with PSCell from EN-DC to EN-DC; A.5.3.x1; Option 1: No test case Option 2: Only requirements for PSCell are verified. 11; Are additional test cases needed?.

When going through the proposed test cases by Moderator, some test cases are similar, but some configurations are not considered. To raise the test coverage and avoid repeating similar testing to verify the core requirements, we think that some test cases could be updated and reduce the test case list.

- NR SA to EN-DC test cases: TC1 ~ TC4 are designed for NR SA to EN-DC HO with PSCell. Since TC1&TC2 will verify the delay requirements including PCell handover and PSCell addition, if TC3 and TC4 focus on PSCell addition delay verification, it will be similar as TC7 and TC8 for NR-DC to NR-DC test cases. We can drop TC3 and TC4 to avoid repeating the similar test.

- NE-DC to NE-DC test cases: TC5~TC6 are designed for NE-DC to NE-DC HO with PSCell. Unknown target PCell is no considered. We can update the target PCell to unknown cell in TC6.

- NR-DC to NR-DC test cases: TC7~TC8 are designed for NR-DC to NR-DC HO with PSCell. Since TC1&TC2 will verify the delay requirements including PCell handover and PSCell addition, and current existing NR Cell handover test cases can fulfill PCell handover in this case, we can focus on PSCell change delay verification in TC7 & TC8.

- EN-DC to EN-DC test cases: TC9~TC10 are designed for EN-DC to EN-DC HO with PSCell. Since TC1&TC2 will verify the delay requirements including PCell handover and PSCell addition, and current existing E-UTRA Cell handover test cases can fulfill PCell handover in this case, we can focus on PSCell change delay verification in TC9 & TC10.

- Remove TC3 and TC4 in NR SA to EN-DC since it is similar as TC7 and TC8 in NR-DC to NR-DC

- Update the target PCell to unknown cell in TC6 to raise the test coverage for HO with PSCell.

- Only focus on PSCell addition / change delay verification in TC7, TC8, TC9 and TC10.

In Rel-15 performance maintenance phase, we have discussed FR1/LTE + FR2 testability and agreement captured in WF [3]. Here we have TC3, TC4, TC7, TC8, TC10 which is FR1/LTE + FR2 test cases with OTA testability problem. TC4, TC7 and TC10 could be tested with the configuration in A.3.7, A.3.7A, A.3.7C. However, the target PSCell addition/change delay will depend on the cell detection of target PCell in FR1 in TC3 and TC8, it will be hard to test because of this FR1+FR2 test limitation.

- TC4, TC7 and TC10 could be tested with the LTE/FR1+FR2 test setup defined in A.3.7, A.3.7A and A.3.7C.

- TC3 and TC8 need further discuss since it will be hard to test because of this FR1+FR2 test limitation

Conclusion

In this contribution we have discussed the test cases design HO with PSCell. We have made the following proposals:

- Remove TC3 and TC4 in NR SA to EN-DC since it is similar as TC7 and TC8 in NR-DC to NR-DC

- Update the target PCell to unknown cell in TC6 to raise the test coverage for HO with PSCell.

- Only focus on PSCell addition / change delay verification in TC7, TC8, TC9 and TC10.

- TC4, TC7 and TC10 could be tested with the LTE/FR1+FR2 test setup defined in A.3.7, A.3.7A and A.3.7C.

- TC3 and TC8 need further discuss since it will be hard to test because of this FR1+FR2 test limitation

- References

- TS 38.133 v17.5.0

- RP-202874, Revised	WID of REL-17 NR RRM further enhancement, Apple, RAN #90

- R4-2115240	WF on Rel-15 NR RRM test case related issues, Ericsson, RAN4#100e