3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208505

Electronic Meeting, May 09 – May 20, 2022

Source:	ZTE Corporation

Title:          Discussion on timing issues for simultaneous operation of IAB

Agenda Item:	9.16.2

Document for:	Approval

- Introduction

In this contribution, the analysis on the issue of timing for simultaneous operation between IAB-DU and IAB-MT of the same IAB-Node is provided.

- Discussion

In RAN4 #102-e meeting, TEA requirement was agreed for simultaneous operation were agreed[1] as following:

Issue 1-2-1: Timing error between intra-node MT TX and DU TX for case#6 Candidate options: - Option 1: To specify TAE between IAB-MT and IAB-DU in timing case #6 (Nokia, Samsung, ZTE) - The requirement value is min [3us , 4.69 / (SCS/15 kHz) µs]. - Option 2: No TAE between IAB-MT and IAB-DU (Huawei, Ericsson) Agreement: Introducing this requirement into core specification - Whether IAB need to pass corresponding conformance test cases if introduced pending on IAB declaration - Further discuss the details of conformance test cases if introduced in conformance phase - It’s FFS whether conformance test cases needed or not considering test feasibility - Note: Cell phase sync performance maybe relaxed by TAE. If relaxtion not acceptable, RAN4 can further revisit the agreement.

The TAE between IAB-DU node and IAB-MT introduced for Rel-17 IAB. The value of min [3us , 4.69 / (SCS/15 kHz) µs] needs to be confirmed.

With regard to DL Tx timing between parent node and child IAB node, the IAB node can use OTA timing scheme to ensure DL Tx timing alignment[2].

If an IAB-node is provided an index Tdelta in a Timing Delta MAC CE [11, TS 38.321] from a serving cell, the IAB-node may assume that TTA/2+Ndelta+Tdelta⋅Gstep-NTA,Offset/2⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-MT when TTA/2+Ndelta+Tdelta⋅Gstep-NTA,Offset/2⋅Tc>0, where - TTA is the difference between the IAB-MT reception time and the IAB-MT transmission time for IAB-MT transmission timing mode 'Case6', and is defined in clause 4.3.1 of [4, TS 38.211] for IAB-MT transmission timing mode 'Case1' and parent node reception mode 'Case7' - Ndelta and Gstep are determined as - Ndelta=-70528 and Gstep=64, if the serving cell providing the Timing Delta MAC CE operates in FR1 - Ndelta=-17664 and Gstep=32, if the serving cell providing the Timing Delta MAC CE operates in FR2

The Gstep is related to carrier frequency, Gstep = 64Tc for FR1 and Gstep = 32Tc for FR2. We think the DL Tx timing between parent node and child node can be adjusted with the accuracy of Tdelta which is much smaller than the CP duration as shown in table 1.

Table 1, Comparison between Tdelta accuracy and CP duartion

| Tdelta accuracy |  | CP duration(normal CP) |  |
| --- | --- | --- | --- |
| FR1 | FR2 | @30kHz FR1 | @120kHz FR2 |
| 32.5ns | 16.3ns | 2343ns | 586ns |

Observation 1: If OTA timing scheme is performed between parent IAB node and child IAB node, the timing difference between parent node and child node can be very smaller than the CP duration of FR1 and FR2 respectively.

If the timing difference between IAB-MT and IAB-DU is within the CP range and OTA timing mechanism is adopted between nodes, the previously reached timing difference between IAB-MT and IAB-DU of the same IAB-Node can be confirmed.

Proposal 1: To confirm the TAE requirement between IAB-DU and IAB-MT as min [3us , 4.69 / (SCS/15 kHz) µs].

- Conclusion

Based on the analysis, the following observation and proposal are given:

Observation 1: If OTA timing scheme is performed between parent IAB node and child IAB node, the timing difference between parent node and child node can be very smaller than the CP duration of FR1 and FR2 respectively.

Proposal 1: To confirm the TAE requirement between IAB-DU and IAB-MT as min [3us , 4.69 / (SCS/15 kHz) µs].

- Reference

RAN4_102-e_BSRF_Demod_Test_ Session_report_EOMv2_AfterPostMeetingApproval.docx

38213-h10.docx