3GPP TSG-RAN Working Group 4 (Radio) Meeting #103-E	R4-2208536

# Electronic Meeting, 9th - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.921 | CR | 0003 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.921: Addition of additional BS antenna parameters in subclause 8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IMT_param |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At previous ITU-R WP 5D meeting parameters to model base station array antennas with vertical sub-arrays was presented. Vertical sub-arrays are used for base stations operating between 1710 to 4990 MHz. The details are captured in TR 38.803, subclause 5.2.3.2.4. Also for operating within the freqeuncy range 4990 to 7125 MHz sub-array antenna structures will be used. In this CR sub-array parameters for some relevant base station deployment scenarious are submitted as additional information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause is added in 8.1.3 with additional array model parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the technical background information does not reflect relevant AAS BS implementations suitable for the freqeuncy range 4990 to 7125 MHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclasue 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.1	BS antenna characteristics

### 8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | j3dB | Degrees |
| Vertical HPBW | q3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | qetilt | Degrees |
| Electrical scan angle | jescan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

### 8.1.2	Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 8.1.2-1 in dBi as:

GE,max=DE,max-LE	(8.1.2-1)

, where the peak directivity DE,max is calculated from given values on j3dB, q3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax-ππ0πAlinθ,φsinθdθdφ	(8.1.2-2)

, where Alin(q,j) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10	(8.1.2-3)

Table 8.1.2-1: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| j3dB (deg.) | 90 | 90 | 90 | 90 |
| q3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) | (4, 4) |
| Number of supported polarizations, P | 2 | 2 | 2 | 2 |
| dh (m) | 0.5l | 0.5l | 0.5l | 0.5l |
| dv (m) | 0.7l | 0.5l | 0.5l | 0.5l |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element, Ptx (dBm) | 22 (Note 4) | 22 (Note 4) | 16 (Note 5) | 9 (Note 6) |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A (Note 7) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |  |  |  |

Based on the array parameters sets fundamental characteristics such as peak EIRP can be derived. For radiated power considerations peak Equivalent Isotropic Radiated Power (EIRP) is calculated in logarithmical scale calculated as:

EIRP=Ptx+GE,max+20log10MN+10log10P 		(Eq. 8.1.2-4)

, where P, M, N, Ptx, GE,max is given by Table 8.1.2-1.

### 8.1.3	Sub-array parameters

In TR 38.803, subclause 5.2.3.2.4 the array antenna model described in Table 8.1.1-2 is extended to support vertical sub-arrays.

In Table 8.1.3-1, representable parameter sets relevant for an AAS base station operating within 4990 to 7125 MHz are provided as additional information.

Table 8.1.3-1: Model parameters relevant for sub-array configurations

| Parameters | Macro suburban | Macro urban |
| --- | --- | --- |
| Element gain | 6.4 dBi | 6.4 dBi |
| Horizontal/vertical 3 dB beam width of single element | 90 º for H 65 º for V | 90 º for H 65 º for V |
| Horizontal/vertical front to back ratio | 30 dB for both H/V | 30 dB for both H/V |
| Antenna polarization | Linear ±45 º | Linear ±45 º |
| Antenna sub-array configuration (Row × Column) | 16 x 16 | 16 x 16 |
| Horizontal/Vertical radiating sub-array spacing | 0.5l for H, 1.4l for V | 0.5l for H, 1.4l for V |
| Number of element rows in sub-array | 2 | 2 |
| Vertical element separation in sub-array (dv,sub) | 0.7l | 0.7l |
| Pre-set sub-array down-tilt | 3 º | 3 º |
| Array Ohmic loss | 2 dB | 2 dB |
| Conducted power (before Ohmic loss) per sub-array | 19 dBm | 19 dBm |
| Base station horizontal coverage range | +/-60 º | +/-60 º |
| Base station vertical coverage range | 90-100 º | 90-100 º |
| Mechanical down-tilt | 3 º | 6 º |