TSG-RAN Working Group 4 (Radio) meeting #103-E	R4-2208538

Electronic Meeting, 9th – 20th May 2022

Source:	Ericsson

Title:	On BS RF receiver requirements for the frequency range 52.6 to 71.0 GHz

Agenda item:	9.15.4.2

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#102-E) specific details related to BS RF core receiver requirements related to clause 10 in TS 38.104 applicable for the frequency range 52.6 to 71.0 GHz was discussed. The meeting outcome and guidance for this meeting was captured in a Way Forward (WF) contribution [1].

In this contribution we present additional information and some proposals necessary to progress the work related to defining RF core requirements for the NR extension up to 71 GHz.

# Discussion

As part of the work to define BS RF core requirements to support up to 71 GHz it is RAN4 responsibility to study and analyse the current technology capabilities before finalizing Spectral Utilization (SU). The SU needs to be decided with both considering BS and UE aspects.

As one example the transmitter in-band requirement such as Adjacent Channel Leakage Ratio (ACLR) and receiver in-band requirement such as Adjacent Channel Selectivity (ACS) will translate to channel filter suppression characteristics. As SU is a common BS and UE system parameter, aspects around other requirements such as occupied bandwidth should be considered. For FR2-1 UE where the decided SU makes the occupied bandwidth requirement to become the strictest requirement in terms of in-band unwanted emissions resulting in reduced available UE power. For the BS is larger than then what is required for occupied bandwidth.

As propagation conditions degrades over frequency, it is essential to have proper SU levels to make sure that the MPR on the UE side is kept to a minimum to ensure reasonable coverage in UL.

Other factors influencing SU for NR in the frequency range 52.6 to 71.0 GHz is as following:

- Large array antenna sizes with reduced physical size due to higher frequency, the filtering resources needed to be optimized considering both size, power consumption and thermal aspects.

- For large carrier bandwidths of up to 2000 MHz the digital filtering could be more challenging depending on ACLR, ACS and occupied bandwidth. Filters with steep slopes requires many taps at high sample rates.

- Higher SCS resulting in higher modulation spectra as 960 kHz SCS will have at least 9 dB higher modulation spectra compared to 120 kHz SCS. This implies that with similar requirement of e.g., OBUE or ACS, the filter attenuation need to be 9 dB higher for 960 kHz SCS compared to 120 kHz SCS.

From the unlicensed perspective requirements [4] upon the occupied bandwidth does not contain any “flat PSD” requirement to what has been discussed on the 5 GHz NR-U range. The occupied channel bandwidth defined in [4] is the bandwidth containing 99% of the power of the signal. The occupied bandwidth shall be less than the nominal channel bandwidth, which is declared and used for the foundation of the spectrum emissions mask requirement. The device must comply with the occupied bandwidth while supporting at least one mode of operation with a necessary bandwidth of 70% of the declared nominal channel bandwidth. This means that the SU for licensed operations in 66 to 71 GHz could be significantly less than 90 to 95% typically assumed for FR2-1 below 52.6 GHz.

Observation: Given the minimum required spectral utilization for RAN1 design needed is 85%, and UE output power should be constrained within 70% of the declared nominal channel bandwidth; together with BS/UE RF design considerations initial spectral utilization should be considered as a range between 85 to 95%.

At previous meeting preliminary values for NRB allocation was captured in draft CRs for BS and UE. It was also captured in way-forward for base station receiver requirements [1] that further work is required with respect to SU levels used for different carrier configurations.

  - Spectrum utilization

In Table 2.1-1 the spectrum utilization for transmission bandwidth and sub-carrier spacing is listed for FR2-1. For FR2-1, the SU is 95 % except for 50 MHz carrier bandwidth and 120 kHz sub-carrier spacing.

Table 2.1-1: FR2-1 spectrum utilization

| SCS (kHz) | Transmission bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| 60 | 95.0% | 95.0% | 95.0% | N/A |
| 120 | 92.2% | 95.0% | 95.0% | 95.0% |

Unlike for FR2-1, it is proposed for FR2-2 to differentiate the spectrum utilization between 85 to 95 %. In Table 2.1-2, the SU allocation for FR2-2 is listed as function of carrier bandwidth and sub-carrier spacing.

Table 2.1-2: FR2-2 spectrum utilization

| SCS (kHz) | Transmission bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 100 | 400 | 800 | 1600 | 2000 |
| 120 | 90.0% | 90.0% | N/A | N/A | N/A |
| 480 | N/A | 88.0% | 88.5% | 86.0% | N/A |
| 960 | N/A | 88.0% | 88.5% | 86.0% | 85.0% |

The relation between SU level in percent and resource block bandwidth, and number of allocated resource block can be expressed as:

SU=100BnrbNRBB

, where Bnrb is the resource block bandwidth for a specific SCS (listed in Table 2.1-3), NRB is the number of allocated resource blocks and B is the transmission bandwidth.

Table 2.1-3: NRB allocation for FR2-1

| SCS (kHz) | NRB bandwidth (MHz) |
| --- | --- |
| 60 | 0.72 |
| 120 | 1.44 |
| 480 | 5.76 |
| 960 | 11.52 |

The spectrum utilization allocation in terms of number of NRB and in percent is listed for NR FR2 in Table 2.1-4. The percentage number is rounded to fit given NRB allocation integer.

Table 2.1-4: NRB and SU allocation FR2

| Frequency range | SCS (kHz) | Transmission bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| FR2-1 | 60 | 66 (95.0%) | 132 (95.0%) | 264 (95.0%) | N/A | N/A | N/A | N/A |
|  | 120 | 32 (92.2%) | 66 (95.0%) | 132 (95.0%) | 264 (95.0%) | N/A | N/A | N/A |
| FR2-2 | 120 | N/A | 62 (89.3%) | N/A | 248 (89.3%) | N/A | N/A | N/A |
|  | 480 | N/A | N/A | N/A | 61 (87.8%) | 120 (86.4%) | 238 (85.7%) | N/A |
|  | 960 | N/A | N/A | N/A | 30 (86.4%) | 60 (86.4%) | 119 (85.7%) | 148 (85.2%) |

Proposal 1: It is proposed for FR2-2 to adopt SU allocation values given in Table 2.1-4.

  - FRC parameters

A Fixed Reference Channel (FRC) is a predefined test signal used to define receiver requirements. For FR2 test signals are defined for receiver sensitivity, ACS, In-band blocking, out-of-band blocking, receiver intermodulation and in-channel selectivity. Detailed information about the FRC definition defined for FR2-1 is captured in Table 2.2-1.

Table 2.2-1: FR2-1 FRC parameters

| Reference channel | G-FR2-A1-1 | G-FR2-A1-2 | G-FR2-A1-3 | G-FR2-A1-4 | G-FR2-A1-5 |
| --- | --- | --- | --- | --- | --- |
| Carrier bandwidth (MHz) | 100 | 50 | 100 | 50 | 25 |
| Subcarrier spacing (kHz) | 60 | 120 | 120 | 60 | 120 |
| Allocated resource blocks | 66 | 32 | 66 | 33 | 16 |
| CP-OFDM Symbols per slot | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5632 | 2792 | 5632 | 2856 | 1416 |
| Transport block CRC (bits) | 24 | 16 | 24 | 16 | 16 |
| Code block CRC size (bits) | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) | 5656 | 2808 | 5656 | 2872 | 1432 |
| Total number of bits per slot | 19008 | 9216 | 19008 | 9504 | 4608 |
| Total symbols per slot | 9504 | 4608 | 9504 | 4752 | 2304 |

Since different SU allocations is proposed for 120 kHz operation within FR2-1 and FR2-2, two new FRC is required for the FR2-2 120 kHz case. The new FRC for 120 kHz is named G-FR2-A1-10 and F-FR2-A1-11.

Table 2.2-2: FR2-2 FRC parameters

| Reference channel | G-FR2-A1-6 | G-FR2-A1-7 | G-FR2-A1-8 | G-FR2-A1-9 | G-FR2-A1-10 | G-FR2-A1-11 |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier bandwidth (MHz) | 400 | 400 | 100 | 100 | 50 | 100 |
| Subcarrier spacing (kHz) | 480 | 960 | 480 | 960 | 120 | 120 |
| Allocated resource blocks | 60 | 30 | 15 | 8 | 31 | 62 |
| CP-OFDM Symbols per slot | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 5120 | 2600 | 1288 | 672 | 2664 | 5376 |
| Transport block CRC (bits) | 24 | 16 | 16 | 16 | 16 | 24 |
| Code block CRC size (bits) | - | - | - | - | - | - |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 |
| Code block size including CRC (bits) | 5144 | 2616 | 1304 | 688 | 2680 | 5400 |
| Total number of bits per slot | 17280 | 8640 | 4320 | 2304 | 8928 | 17856 |
| Total symbols per slot | 8640 | 4320 | 2160 | 1152 | 4464 | 8928 |

Proposal 2: For FR2-2 define FRC as described in Table 2.2-2.

  - Reference sensitivity

Due to the fact that different SU is proposed for FR2-2 compared to FR2-1, a new FRC is required for FR2-2 as visualised in Table 2.3-1.

Table 2.3-1: FR2 OTA reference sensitivity requirement

| Frequency Range | BS channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + ΔFR2_REFSENS |
|  | 50 | 120 | G-FR2-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
|  | 100, 200, 400 | 120 | G-FR2-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| FR2-2 | 100, 400 | 120 | G-FR2-A1-11 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
|  | 400, 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 9 + ΔFR2_REFSENS |
|  | 400, 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 9 + ΔFR2_REFSENS |

Proposal 3: For FR2-2 reference sensitivity update table according to change indicated in Table 2.3-1.

  - In-Channel selectivity

Due to the fact that different SU is proposed for FR2-2 compared to FR2-1, a new FRC is required for FR2-2 as visualised in Table 2.4-1.

Table 2.4-1: OTA in-channel selectivity requirement for BS type 2-O

| Frequency Range | BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) (Note 2) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50 | 60 | G-FR2-A1-4 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 32 RB |
|  | 100, 200 | 60 | G-FR2-A1-1 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 64 RB |
|  | 50 | 120 | G-FR2-A1-5 | EISREFSENS_50M + ΔFR2_REFSENS | EISREFSENS_50M + 10 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 16 RB |
|  | 100, 200, 400 | 120 | G-FR2-A1-2 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
| FR2-2 | 100, 400 | 120 | G-FR2-A1-10 | EISREFSENS_50M + 3 + ΔFR2_REFSENS | EISREFSENS_50M + 13 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 120 kHz SCS, 32 RB |
|  | 400 | 480 | G-FR2-A1-8 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 32 RB |
|  | 800, 1600 | 480 | G-FR2-A1-6 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22 + ΔFR2_REFSENS | DFT-s-OFDM NR signal, 480 kHz SCS, 64 RB |
|  | 400 | 960 | G-FR2-A1-9 | EISREFSENS_50M + 9 + ΔFR2_REFSENS | EISREFSENS_50M + 19+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 16 RB |
|  | 800, 1600, 2000 | 960 | G-FR2-A1-7 | EISREFSENS_50M + 12 + ΔFR2_REFSENS | EISREFSENS_50M + 22+ ΔFR2_REFSENS | DFT-s-OFDM NR signal, 960 kHz SCS, 32 RB |

Proposal 4: For FR2-2 ICS update table according to change indicated in Table 2.4-1.

# Conclusion

In this contribution aspects related to spectrum utilization is discussed. Updates with respect to FR2-2 is proposed to reflect technical concerns.

The following proposals is presented for approval:

Proposal 1: It is proposed for FR2-2 to adopt SU allocation values given in Table 2.1-4.

Proposal 2: For FR2-2 define FRC as described in Table 2.2-2.

Proposal 3: For FR2-2 reference sensitivity update table according to change indicated in Table 2.3-1.

Proposal 4: For FR2-2 ICS update table according to change indicated in Table 2.4-1.

# References

[1]	R4-2207217, “WF on >52.6 GHz BS RF Rx Requirements”, Ericsson

[2] 	R4-2203578, “On BS RF receiver requirements for the frequency range 52 to 71 GHz”, Ericsson

[3]	R4-2205022, “52.6-71 GHz System Parameters”, Ericsson

[4]	EN 302 567, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, ETSI