TSG-RAN Working Group 4 (Radio) meeting #103-E	R4-2208543

Electronic Meeting, 9th – 20th May 2022

Source:	Ericsson

Title:	BS transmitter conformance test specification impact overview

Agenda item:	9.15.5.2

Document for:	Discussion

# Introduction

At previous RAN4 meeting (RAN4#102-E) the work to update the BS conformance test specification (TS 38.141-2) to support frequencies up to 71 GHz was initiated.

In this contribution we summarize the essential updates with respect to the introduction of new carrier bandwidths and sub-carrier spacings to support up to 71 GHz as well as identifying test requirements where new requirement limits are required.

# Discussion

In the following sub-sections different aspects related to OTA test requirements and the extended support up to 71 GHz is presented.

2.1	Test system uncertainty

The maximum OTA test system uncertainty for transmitter tests minimum requirements relevant for FR2 is summarized in Table 2.1-1.

The technical background information used to derive the maximum OTA test system uncertainty is captured in a dedicated technical report, TR 37.941 [1]. This report describes OTA test methodologies, test procedures, calibration procedures and measurement uncertainty evaluation. The measurement uncertainty is evaluated per test method and requirement. Based on the overview of measurement uncertainty for a specific requirement the maximum OTA test system uncertainty can be established.

Table 2.1-2: Maximum OTA Test System uncertainty for FR2 transmitter tests

| Requirement | Subclause | Maximum OTA Test System uncertainty |
| --- | --- | --- |
| Radiated transmit power | 6.2 | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |
|  |  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |
| Base station output power | 6.3 | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |
| RE power control dynamic range | 6.4.2 | N/A |
| Total power dynamic range | 6.4.3 | ±0.4 dB |
| Transmitter OFF power | 6.5.1 | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |
| Transmitter transient period | 6.5.2 | N/A |
| Frequency error | 6.6.2 | ±12 Hz |
| Modulation quality | 6.6.3 | 1% |
| Time alignment error | 6.6.4 | ±25 ns |
| Occupied bandwidth | 6.7.2 | 600 kHz |
| ACLR | 6.7.3 | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |
| Operating band unwanted emissions | 6.7.4 | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |
| Transmitter spurious emissions, mandatory requirements | 6.7.4 | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |
| Transmitter spurious emissions, additional requirements | 6.7.5.4 | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz ±TBD dB (52.6 GHz < f ≤ 71.0 GHz) |

Based on the maximum OTA test uncertainty the test tolerance required to determine the test requirement can be determined according to Table 2.1-2.

Table 2.1-2: Derivation of test requirements

| Test | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- |
| Radiated transmit power | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 43.5GHz < f ≦ 48.2GHz TBD dB, 52.6GHz < f ≦ 71.0GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 43.5GHz < f ≦ 48.2GHz TBD dB, 52.6GHz < f ≦ 71.0GHz | Formula: Upper limit + TT, Lower limit – TT |
| Base station output power | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 43.5GHz < f ≦ 48.2GHz TBD dB, 52.6GHz < f ≦ 71.0GHz | Formula: Upper limit + TT, Lower limit – TT |
| Output power dynamics | 0.4 dB | Formula: Total power dynamic range – TT |
| Transmitter OFF power | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.6 dB, 43.5GHz < f ≦ 48.2GHz TBD dB, 52.6GHz < f ≦ 71.0GHz | Formula: Minimum Requirement + TT |
| Frequency Error | 12 Hz | Formula: Frequency Error limit + TT |
| Modulation quality (EVM) | 1 % | Formula: EVM limit + TT |
| Time alignment error | 25 ns |  |
| Occupied bandwidth | 0 Hz | Formula: Minimum Requirement + TT |
| Adjacent Channel Leakage Power Ratio (ACLR) | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦43.5GHz 2.8 dB, 43.5GHz < f ≦ 48.2GHz TBD dB, 52.6GHz < f ≦ 71.0GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz TBD dB, 52.6GHz < f ≦ 71.0GHz | Formula: Relative limit - TT Absolute limit +TT |
| Operating band unwanted emissions | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz TBD dB, 52.6GHz < f ≦ 71.0GHz 0.1*BWcontiguous f < fmax 0 dB For co-existence with Earth Exploration Satellite Service 0 dB | Formula: Minimum Requirement + TT |
| General transmitter spurious emissions requirements Category A | 0 dB | Formula: Minimum Requirement + TT |
| General transmitter spurious emissions requirements Category B | 0 dB | Formula: Minimum Requirement + TT |
| Transmitter spurious emissions, additional requirements | For co-existence with Earth Exploration Satellite Service 0 dB | Formula: Minimum Requirement + TT |

2.2	Radiated transmit power

For each declared conformance beam direction pair, the EIRP measurement results shall remain within the values provided in Table 2.2-1, relative to the manufacturer's declared rated beam EIRP value. BS type 2-O needs to be extended for the frequency range 52.6 to 71.0 GHz as showed in Table 2.2-1.

Table 2.2-1: Test requirement for radiated transmit power

|  | Normal test environment | Extreme test environment |
| --- | --- | --- |
| BS type 1-H | f ≤ 3 GHz: ± 3.3 dB | N/A |
|  | 3 GHz < f ≤ 6 GHz: ± 3.5 dB For bands n46, n96 and n102: ± 4.0 dB |  |
| BS type 1-O | f ≤ 3 GHz: ± 3.3 dB 3 GHz < f ≤ 6 GHz: ± 3.5 dB | f ≤ 3 GHz: ± 5.2 dB 3 GHz < f ≤ 4.2 GHz: ± 5.3 dB 4.2 GHz < f ≤ 6 GHz: ± 5.3 dB |
| BS type 2-O | 24.15 GHz < f ≤ 29.5 GHz: ± 5.1 dB 37 GHz < f ≤ 43.5 GHz: ± 5.4 dB 43.5 GHz < f ≤ 48.2 GHz: ± 5.6 dB 52.6 GHz < f ≤ 71.0 GHz: ± TBD dB | 24.15 GHz < f ≤ 29.5 GHz: ± 7.6 dB 37 GHz < f ≤ 43.5 GHz: ± 7.8 dB 43.5 GHz < f ≤ 48.2 GHz: ± 8.0 dB 52.6 GHz < f ≤ 71.0 GHz: ± TBD dB |

2.3	BS output power

For BS output power BS type 2-O needs to be extended for the frequency range 52.6 to 71.0 GHz as showed in Table 2.3-1.

Table 2.3-1: Test requirement for radiated transmit power

|  | Normal test environment |
| --- | --- |
| BS type 1-O | f ≤ 3.0 GHz: ± 3.4 dB 3.0 GHz < f ≤ 4.2 GHz: ± 3.5 dB 4.2 GHz < f ≤ 6.0 GHz: ± 3.5 dB |
| BS type 2-O | 24.15 GHz < f ≤ 29.5 GHz: ± 5.1 dB 37 GHz < f ≤ 43.5 GHz: ± 5.4 dB 43.5 GHz < f ≤ 48.2 GHz: ± 5.6 dB 52.6 GHz < f ≤ 71.0 GHz: ± TBD dB |

2.4	Transmit transient period

For transmit transient period the requirement text needs to be extend as shown below:

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -33.1 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 24.15 GHz < f ≤ 29.5 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -32.7 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 37 GHz < f ≤ 43.5 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -32.4 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 43.5 GHz < f ≤ 48.2 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -TBD + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 52.7 GHz < f ≤ 71.0 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

2.4	Occupied bandwidth

For occupied bandwidth the number of measurement points needs to be extended to support the frequency range 52.6 to 71.0 GHz. In Table 2.4-2 the measurement point for FR2-1 is listed.

Table 2.4-2: Span and number of measurement points for OBW measurements for FR2

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 | > 50 |
| Span (MHz) |  |  |  |  |  |
| Minimum number of measurement points |  |  |  |  |  |

RAN4 needs to decide if the concept used for FR2-1 can be scaled up to support larger carrier bandwidths (800, 1600, 2000 MHz) applicable for FR2-2.

2.5	ACLR

Additions with respect to the frequency range 52.6 to 71.0 GHz is visualised in Table 2.5-1.

Table 2.5-1: BS type 2-O ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400, 800, 1600, 2000 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 25.7 (Note 3) 23.4 (Note 4) 23.2 (Note 5) 24 – TBD (Note 6) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 4: Applicable to bands defined within the frequency spectrum range of 37 – 43.5 GHz NOTE 5: Applicable to bands defined within the frequency spectrum range of 43.5 – 48.2 GHz NOTE 6: Applicable to bands defined within the frequency spectrum range of 52.6 – 71 GHz |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in Table 2.5-2

Table 2.5-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit (Note 1) | ACLR absolute limit (Note 2) | ACLR absolute limit (Note 3) |
| --- | --- | --- | --- |
| Wide-area BS | -10.3 dBm/MHz | -10.1 dBm/MHz | -13 + TBD dBm/MHz |
| Medium-range BS | -17.3 dBm/MHz | -17.1 dBm/MHz | -20 + TBD dBm/MHz |
| Local-area BS | -17.3 dBm/MHz | -17.1 dBm/MHz | -20 + TBD dBm/MHz |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 43.5 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 43.5 – 48.2 GHz NOTE 3: Applicable to bands defined within the frequency spectrum range of 52.6 – 71.0 GHz |  |  |  |

2.6	OBUE

For OBUE new tables will be required for Cat A and Cat B emission limits as showed in Table 2.6-1 and Table 2.6-2.

Table 2.6-1: Cat A OBUE limits applicable in the frequency range 52.6 – 71 GHz

| Frequency offset of measurement filter -3B point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | TBD | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.1* BWcontiguous +0.5 MHz f_offset < f_ offsetmax | Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)) | 1 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. |  |  |  |

Table 2.6-2: Cat B OBUE limits applicable in the frequency range 52.6 – 71 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 MHz f_offset < 0.1* BWcontiguous +0.5 MHz | TBD | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.1* BWcontiguous +0.5 MHz f_offset < fB +0.5 MHz | Min(-13 dBm, Max(Prated,t,TRP – 39 dB, -20 dBm)) | 1 MHz |
| fB f < fmax | fB +5 MHz f_offset < f_ offsetmax | Min(-5 dBm, Max(Prated,t,TRP – 29 dB, -10 dBm)) | 10 MHz |
| NOTE 1: For non-contiguous spectrum operation within any operating band the limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. NOTE 2: fB = 2*BWcontiguous when BWcontiguous ≤ 500 MHz, otherwise fB = BWcontiguous + 500 MHz. |  |  |  |

2.7	Spurious emissions

For initial conditions the descriptions need to be updated as shown below:

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier, see clause 4.9.1:

-	For FR1:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	For FR2:

-	B when testing from 30 MHz to FDL_low - ΔfOBUE

-	T when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz for FR2-1)

RF bandwidth positions to be tested in single-band multi-carrier operation, see clause 4.9.1:

-	For FR1:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 12.75 GHz (or 5th harmonic)

-	For FR2:

-	BRFBW when testing from 30 MHz to FDL_low - ΔfOBUE

-	TRFBW when testing from FDL_high + ΔfOBUE to 2nd harmonic (or to 60 GHz for FR2-1)

RF bandwidth positions to be tested in multi-band multi-carrier operation, see clause 4.9.1:

-	For FR1:

-	BRFBW_T'RFBW when testing from 30 MHz to FDL_Blow_low - ΔfOBUE

-	B'RFBW_TRFBW when testing from FDL_Bhigh_high + ΔfOBUE to 12.75 GHz (or to 5th harmonic)

-	BRFBW_T'RFBW and B'RFBW_TRFBW when testing from FDL_Blow_high + ΔfOBUE to FDL_Bhigh_low - ΔfOBUE

Directions to be tested: As the requirement is TRP the beam pattern(s) may be set up to optimise the TRP measurement procedure (see annex I) as long as the required TRP level is achieved.

The test requirement limits will be equal to RF core requirement limits. But the upper test limit needs to be handled separately for FR2-1 and FR2-2 as shown in Table 2.7-1.

Table 2.7-1: General OTA BS transmitter spurious emission limits for BS type 2-O

| Spurious frequency range | Test limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; 60 GHz) for FR2-1 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band in GHz for FR2-1 |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Measurement bandwidth as in ITU-R SM.329 [5], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. |  |  |  |

For Cat B the spurious emission limit tables need to be updated to handle the upper test limit as shown in Table 2.7-2.

Table 2.7-2: BS radiated Tx spurious emission limits in FR2 (Category B)

| Frequency range (Note 4) | Test limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz 1 GHz | -36 dBm | 100 kHz | Note 1 |
| 1 GHz 18 GHz | -30 dBm | 1 MHz | Note 1 |
| 18 GHz Fstep,1 | -20 dBm | 10 MHz | Note 2 |
| Fstep,1 Fstep,2 | -15 dBm | 10 MHz | Note 2 |
| Fstep,2 Fstep,3 | -10 dBm | 10 MHz | Note 2 |
| Fstep,4 Fstep,5 | -10 dBm | 10 MHz | Note 2 |
| Fstep,5 Fstep,6 | -15 dBm | 10 MHz | Note 2 |
| Fstep,6 min(2nd harmonic of the upper frequency edge of the DL operating band in GHz; 60 GHz) for FR2-1 Fstep,6 2nd harmonic of the upper frequency edge of the DL operating band in GHz for FR2-2 | -20 dBm | 10 MHz | Note 2, Note 3 |
| NOTE 1: Bandwidth as in ITU-R SM.329 [5], s4.1 NOTE 2: Limit and bandwidth as in ERC Recommendation 74-01 [26], annex 2. NOTE 3: Upper frequency as in ITU-R SM.329 [5], s2.5 table 1. NOTE 4: The step frequencies Fstep,X are defined in table 6.7.5.2.5.2.3-2. |  |  |  |

, where the parameters Fstep,X is described in Table 2.7-3.

Table 2.7-3-2: Step frequencies for defining the BS radiated Tx spurious emission limits in FR2 (Category B)

| Operating band | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n257 | 18 | 23.5 | 25 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 45 | 47.5 | 59.5 |
| n263 | 18 | 43 | 53.5 | 74.5 | 85 | 127 |
| NOTE 1: Fstep,X are based on ERC Recommendation 74-01 [26], annex 2. NOTE 2: Fstep,3 and Fstep,4 are aligned with the values for ΔfOBUE in table 6.7.1-1. |  |  |  |  |  |  |

RAN4 needs to determine the proper upper measurement limit for FR2-2 as well as finding a good way to differentiate between FR2-1 and FR2-2.

# Conclusion

In this contribution the work effort to update the NR conformance test specification TS 38.141-2 to support up to 71 GHz have been identified.

New test requirement levels based on maximum test uncertainties will be defined for FR2-2. The maximum test uncertainties requires that OTA test methodologies and test procedures is re-evaluated with respect to measurement acceptable uncertainty. The work to update test method descriptions and corresponding measurement uncertainty evaluation will be the grand challenge in the conformance work.

Also, details such as measurement step size, upper test frequency, etc. needs to be further discussed before the test specification can be updated.

# References

[1]	TR 37.941, “Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements”, 3GPP

[2] 	SM.329, “Unwanted emissions in the spurious domain”, ITU-R