3GPP TSG-RAN WG4 Meeting # 103-e 	R4-2208651

Electronic Meeting, May 9-20, 2022

Source:	Ericsson

Title:	Simulation results for Propagation Delay Compensation

Agenda item:	9.22.2

Document for:	Discussion

# 1	Introduction

Simulation results for UE Rx-Tx time difference according to simulation assumptions in section 3.1 of [1]. The assumptions are also repeated in appendix.

# 2	Discussion

The simulation results are collected in the pages below in tables 1 to 4:

The tables contain the simulation output metric, as specified by the simulation assumption in WF section 3.1 [2]:

“At least the following performance characteristics are to be provided for TUE-RX:

- 90%-ile of the TUE-RX errors for each cell

and,

- TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).”

and  the results we are showing are with 100% RE utilization Data and CCH load in non-TRS symbols.

Table 1 TUE-RX accuracy with TRS, 1 sample, AWGN, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| 114 | 24 | 15kHz |
| 59 | 52 |  |
| 29 | 104 |  |
| 58 | 24 | 30kHz |
| 29 | 52 |  |
| 14 | 132 |  |
| 29 | 24 | 60 kHz |
| 14 | 64 |  |
| 7.2 | 132 |  |
| 14 | 32 | 120 kHz |
| 7 | 64 |  |
| 4 | 128 |  |

Table 2 TUE-RX accuracy with TRS, 1 sample, TDL-A, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| 162 | 24 | 15kHz |
| 101 | 52 |  |
| 66 | 104 |  |
| 102 | 24 | 30kHz |
| 69 | 52 |  |
| 44 | 132 |  |
| 69 | 24 | 60 kHz |
| 43 | 64 |  |
| 34 | 132 |  |
| 42 | 32 | 120 kHz |
| 36 | 64 |  |
| 33 | 128 |  |

Table 3 TUE-RX accuracy with TRS, 4 samples, AWGN, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| 114 | 24 | 15kHz |
| 59 | 52 |  |
| 29 | 104 |  |
| 58 | 24 | 30kHz |
| 29 | 52 |  |
| 14 | 132 |  |
| 29 | 24 | 60 kHz |
| 14 | 64 |  |
| 7.2 | 132 |  |
| 14 | 32 | 120 kHz |
| 7 | 64 |  |
| 4 | 128 |  |

Table 4 TUE-RX accuracy with TRS, 4 samples, TDL-A, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| 167 | 24 | 15kHz |
| 87 | 52 |  |
| 55 | 104 |  |
| 86 | 24 | 30kHz |
| 56 | 52 |  |
| 40 | 132 |  |
| 58 | 24 | 60 kHz |
| 41 | 64 |  |
| 32 | 132 |  |
| 41 | 32 | 120 kHz |
| 32 | 64 |  |
| 29 | 128 |  |

# 3	Summary

Please consider Tables 1,2,3 and 4 in the development of PDC requirements based on TRS.

# 4	Annex, Simulation assumptions

Table 2 General parameters:

Table 2 CSI-RS for tracking transmission configuration parameters (TRS) [1]

| Parameter | Value |
| --- | --- |
| Number of transmit TRS antennas | 1 |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |
| Number with CSI-RS Resource sets with trs-Info | 1 |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(10MHz), 52 (20MHz),132 (50MHz) - 60 kHz: 24(20MHz), 64 (50MHz),132 (100MHz) - 120 kHz: 32(50MHz), 64 (100MHz), 128 (200MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz) - 60 kHz: 12864(20MHz), 6432 (4050MHz), 3216 (100MHz) - 12060 kHz: 12832(4050MHz), 6416 (80100MHz), 328 (200MHz) |
| TRS periodicity | 40ms |
| TRS resources number | 2, 4 |
| TRS symbol location | {4,8} |
| TRS frequency density | 3 |
| TRS samples | 1, 4 |
| Evaluation period | 40 ms, 200 ms |

At least the following performance characteristics are to be provided for TUE-RX [2]:

- 90%-ile of the TUE-RX errors for each cell

and,

- TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).

# References

- R4-2207020, WF on NR_IIOT_URLLC_enh_RRM, Nokia, Nokia Shanghai Bell

- R4-2202701, WF on NR_IIOT_URLLC_enh_RRM, Nokia, Nokia Shanghai Bell