# 3GPP TSG-RAN WG4 Meeting # 103-e                                                        R4-2208705

# Electronic Meeting, May. 9-May. 20, 2022

Source:	ZTE Corporation, CHTTL

Title:	TP for TR38.717-03-02_CA_n1A-n8A-n78A

Agenda Item:	8.8.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n1A-n8A-n78A in TR38.717-03-02.

- Reference

- TR38.717-03-02,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 2 band UL,v0.7.0

- Text Proposal

----- Start of TP -----

# 5.1.x	CA_n1-n8-n78

### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n8-n78 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 5.1.x.2		Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n8A-n78A | CA_n1A-n8A CA_n1A-n78A CA_n8A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

### 5.1.x.3 	Co-existence studies

In terms of the co-existence studies for the fallbacks UL CA configurations of CA_n1A-n8A, CA_n1A-n78A, CA_n8A-n78A:

-	3rd order IMD generated by dual uplink of Band n1 + Band n8 may also fall into own Rx of band n78

-	5th order IMD generated by dual uplink of Band n1 + Band n78 may also fall into own Rx of band n8

-    No IMD MSD generated by dual uplink of Band n8 + Band n78 may also fall into own Rx of band n1

### 5.1.x.4	MSD

According to clause 5.1.x.3, there were several IMD problems for this band combination. The MSD value are defined as in the table 5.1.x.4-1:

Table 5.1.x.4-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) | UL CLRB |  | DL Fc (MHz) |  | MSD (dB) |  | Duplex mode |  |  |
| CA_n1-n8-n78 |  | n1 |  | 1945 | 5 |  | 25 |  | 2135 |  | N/A |  | FDD | N/A |
|  |  | n8 |  | 900 | 5 |  | 25 |  | 945 |  | N/A |  | FDD | N/A |
|  |  | n78 |  | 3745 | 10 |  | 50 |  | 3745 |  | 14.9 |  | TDD | IMD3 |
|  |  | n1 |  | 1940 | 5 |  | 25 |  | 2130 |  | N/A |  | FDD | N/A |
|  |  | n8 |  | 895 | 5 |  | 25 |  | 940 |  | 3.3 |  | FDD | IMD5 |
|  |  | n78 |  | 3380 | 10 |  | 50 |  | 3380 |  | N/A |  | TDD | N/A |

----- End of TP -----