# 3GPP TSG-RAN WG4 Meeting # 103-e                                                        R4-2208706

# Electronic Meeting, May. 9-May. 20, 2022

Source:	ZTE Corporation

Title:	TP for TR38.717-03-02_CA_n28A-n39A-n41

Agenda Item:	8.8.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n28A-n39A-n41A and CA_n28A-n39A-n41C in TR38.717-03-02.

- Reference

- TR38.717-03-02,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 2 band UL,v0.7.0

- Text Proposal

----- Start of TP -----

# 5.1.x	CA_n28-n39-n41

### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n39-n41 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 5.1.x.2		Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n39A-n41A | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n39A-n41C | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41C_BCS1 |  |

### 5.1.x.3 	Co-existence studies

In terms of the co-existence studies for the fallbacks UL CA configurations of CA_n28A-n39A, CA_n28A-n41A, CA_n39A-n41A:

- 2nd Intermodulations (i.e.576 - 810MHz) and 5th intermodulations (i.e. 260–768MHz) caused by 2UL CA_n39A-n41A may fall into band n28 own DL Rx range.

- 2nd intermodulations (i.e. 2583–2668MHz) caused by 2UL CA_n28A-n39A may fall into band n41 own DL Rx range.

- 2nd intermodulations (i.e. 1748–1987MHz)  intermodulations caused by 2UL CA_n28A-n41A may fall into band n39 own DL Rx range.

It should be noted for this band combination, CA_n39A-n41A is applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.

### 5.1.x.4	MSD

According to clause 5.1.x.3, there were several IMD problems for this band combination:

1: Due to synchronous operation, i.e. without simultaneous Rx/Tx, between band n39 and band n41, therefore there are no need to define the IMD2 caused by 2UL CA_n28A-n39A may fall into band n41 own DL Rx range and by 2UL CA_n28A-n41A may fall into band n39 own DL Rx range.

2: For IMD5 products produced by Band n39 and n41 that impact the reference sensitivity of NR band n28, the IMD5 frquency range are derived from |2*fn41– 3*fn39|, so considering the minimum channel bandwidth for band n39 and band n41 are 5MHz and 10MHz, the highest DL Fc of band n39 equals to 1917.5MHz, the lowest DL Fc of band n41equals to 2501 MHz, then the highest IMD5 frquency range is 750.5MHz, which is out of the frequency range of band n28 DL, which means there are no suitable test points for the IMD5 MSD, so thre is no need to define the MSD value for IMD5 issue.

3. For IMD2 products produced by Band n39 and n41 that impact the reference sensitivity of NR band n28,  the MSD values caused by the intemodulation are defined as in the table 5.1.x.4-1, which is the same IMD2 value with CA_n1A-n28A-n41A:

Table 5.1.x.4-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) | UL CLRB |  | DL Fc (MHz) |  | MSD (dB) |  | Duplex mode |  |  |
| CA_n28-n39-n41 |  | n28 |  | 707 | 5 |  | 25 |  | 762 |  | 29.3 |  | FDD | IMD2 |
|  |  | n39 |  | 1923 | 5 |  | 25 |  | 1923 |  | N/A |  | TDD | N/A |
|  |  | n41 |  | 2685 | 10 |  | 50 |  | 2685 |  | N/A |  | TDD | N/A |

----- End of TP -----