# 3GPP TSG-RAN WG4 Meeting # 103-e                                                        R4-2208707

# Electronic Meeting, May. 9-May. 20, 2022

Source:	ZTE Corporation

Title:	TP for TR38.717-03-02_CA_n28A-n40A-n41A

Agenda Item:	8.3.1

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n28A-n40A-n41A in TR38.717-03-02.The MSD values are from [2].

- Reference

- TR38.717-03-02,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 2 band UL,v0.6.0

- R4-2208003, DC_28_n40-n41 MSD, Qualcomm Incorporated

- Text Proposal

----- Start of TP -----

# 5.1.132			CA_n28-n40-n41

### 5.1.132.1		Operating bands for CA

Table 5.1.132.1-1: 3DL Inter-band CA operating bands

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28A-n40A-n41A | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 5.1.132.2		Channel bandwidths per operating band for CA

Table 5.1.132.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n40A-n41A | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 5.1.132.3 	Co-existence studies

The co-existence studies for the UL CA configurations of CA_n28A-n40A, CA_n28A-n41A, CA_n40A-n41A have already captured in TR 38.716-02-00, where:

- 4th intermodulations (i.e. 192–780MHz) caused by 2UL CA_n40A-n41A may fall into band n28 own DL Rx range.

- 5th intermodulations (i.e. 2356–2691MHz) caused by 2UL CA_n28A-n40A may fall into band n41 own DL Rx range.

- No intermodulations caused by 2UL CA_n28A-n41A may fall into band n40 own DL Rx range.

It should be noted for this band combination, CA_n40A-n41A is applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.

### 5.1.132.4	MSD

According to clause 5.1.132.3, IMD5 products produced by Band 28 and n40 that impact the reference sensitivity of NR band n41. However, considering the requirements for TDD-TDD NR CA combinations of CA_n40-n41 are defined without simultaneous Rx/Tx capability in TS38.101-1, i.e. synchronous operation. Therefore it is no need to defined MSD requirements due to IMD5 issues.

For IMD4 intermodulations caused by 2UL CA_n40A-n41A may fall into band n28 own DL Rx range, the MSD values caused by the intemodulation are defined as:

Table 5.1.132.4-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n40-n41 | n28 | 710 | 5 | 25 | 765 | 7.6 | FDD | IMD4 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |

----- End of TP -----