3GPP TSG-RAN WG4 Meeting # 103-e	R4-2208725

Electronic Meeting, May 09 – May 20, 2022

Source:	ZTE Corporation

Title:	UE Rx/Tx and gNB Rx/Tx timing delay mitigation

Agenda item:	9.20.1.1

Document for:	Discussion

# 1	Introduction

In RAN4 102-e, there were some discussions on TEG and a few open issues remain. Some of the WF [1] are copied below.

Issue 1-2-1 The temporal validity of UE/TRP Tx TEGs Agreements: - Wait RAN2 progress on how to signal a change in association between SRS resources to Tx TEGs. FFS: The applicability of reported UE Rx TEG agreed in last meeting can be reused for UE/TRP Tx TEG i.e. The applicability of reported UE/TRP Tx TEG is limited to the measurements contained within the measurement report in which the Tx TEG information is provided. And it applies only to the measurements that are tagged with the corresponding Tx TEG ID. Issue 1-2-2 The temporal validity of UE/TRP RxTx TEGs? Candidate options: - Option 1: - The applicability of reported UE Rx TEG agreed in last meeting can be reused for UE/TRP RxTx TEG i.e. The applicability of reported UE/TRP RxTx TEG is limited to the measurements contained within the measurement report in which the RxTx TEG information is provided. And it applies only to the measurements that are tagged with the corresponding RxTx TEG ID. - Option 2: - Up to RAN2.

- This paper discusses the issues listed above and also provide our view on the other related issues.

2	Discussions

One of the main issues discussed was the feasibility of TEG. In the RAN1 LS which triggered the work in RAN4, the definition of TE and TEG is given as follows.

Agreement: The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications): - Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error. - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error. - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin. - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin. - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin. - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin. - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin. - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

According to the definition of TE and TEG, it seems that the relevant impacting factors are at least the RF chains which are actually used for radio transmissions and the antenna panels. Theoretically the TE can be measured and calibrated upon manufacturing and testing of the UE or TRP. However, it’s uncertain whether the TE property will be time-variant and how the drifts would be. It can be also observed that the Rx TEG is described in a similar way with Tx TEG.

The temporal validity of UE/TRP Tx or RxTx TEGs?

In our view the two options are not mutually exclusive, since one option is talking about the signaling perspective of the feature while the one focuses on how to associate the measurement results with a certain TEG. Pending on RAN2 agreement on the signaling aspects, we think that at least the following proposal can be agreed. How to signal such a association can be up to RAN2, while in RAN4 we can carry on our work.

The applicability of reported UE Rx TEG agreed in last meeting can be reused for UE/TRP RxTx TEG i.e. The applicability of reported UE/TRP RxTx TEG is limited to the measurements contained within the measurement report in which the RxTx TEG information is provided. And it applies only to the measurements that are tagged with the corresponding RxTx TEG ID.

3	Conclusion

- Proposal 1: The applicability of reported UE Rx TEG agreed in last meeting can be reused for UE/TRP RxTx TEG i.e. The applicability of reported UE/TRP RxTx TEG is limited to the measurements contained within the measurement report in which the RxTx TEG information is provided. And it applies only to the measurements that are tagged with the corresponding RxTx TEG ID.

References

- R4-2206997