3GPP TSG-RAN WG4 Meeting #103-e	R4-2208774

Electronic Meeting, 9 – 20 May, 2022

Source:	ZTE Corporation

Title:	Views on NCSG

Agenda Item:	9.10.1.3

Document for:	Approval

- Introduction

During 102 meeting, one WF[1] about NCSG was approved. Until the end of 102 meeting, it has been concluded that the core part of NCSG can be mainly completed with a set of CR and WF approved. Only the corresponding UE feature list, some details should be finally identified.

In this contribution, we provide some suggestions on the detailed UE feature list.

- Discussion

The following UE features were proposed during 102 meeting.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (V2X WI only)”. | Consequence if the feature is not configured by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19-X | Network controlled small gap (NCSG) | Support of per-FR NCSG | 19-1 | yes | no | UE cannot be configured with per-FR NCSG | per-UE | No | No |  |  | Optional with capability signalling |
| 19-X+1 | Network controlled small gap (NCSG) | Supported NCSG patterns | 19-1 | yes | no | Network does not know whether some NCSG patterns can be configured to UE | per-UE | No | No |  |  | Optional with capability signalling NCSG patterns #0, #1 are conditional mandatory if UE supports 19-1 NCSG patterns #13, #14 are conditional mandatory if UE supports 19-1 and 19-X |
| 19-X+2 | Network controlled small gap (NCSG) | Supported NR-only NCSG patterns | 19-1 | yes | no | Network does not know whether some NR-only NCSG patterns can be configured to UE | per-UE | No | No |  |  | Optional with capability signalling NCSG patterns #2 and #3 are conditional mandatory if UE supports 19-1 NCSG patterns #17 and #18 are conditional mandatory if UE supports 19-1 and a FR2 band |

According to the agreements achieved in 102 meeting, the following X-Y+1 has been tentatively agreed in the 1st round. X-Y and X-Y+2 are open.

For X-Y, which comes from the newly introduced per-FR/per-UE NCSG capability, it should be optional. so we are fine with such UE feature.

For X-Y+2, actually which is a component of UE supported NCSG pattern report. Similar as UE report or supported legacy MG pattern, the supported NCSG patterns was split into two types: ncsg-MeasGapPatterns and ncsg-MeasGapNR-Patterns, corresponding to X-Y+1 and X-Y+2 respectively. So both X-Y+1 and X-Y+2 are needed.

Proposal 1: All the three UE features(X-Y, X-Y+1, X-Y+2) are necessary.

- Conclusion

In this contribution, we have the following proposals for NCSG:

- Proposal 1: All the three UE features(X-Y, X-Y+1, X-Y+2) are necessary.

- Reference

R4-2206889, “WF on R17 NR MG enhancement - NCSG”, Apple.