3GPP TSG-RAN4 WG4 Meeting #103-e	R4-2208809

# Electronic meeting, May 9 – May 20, 2022

Source: 	vivo

Title: 	Remain issues on RRM impacts for extending NR operation to 71GHz

Agenda Item:	9.15.8.1

Document for:	Discussion

Introduction

In RAN4 #102-bis, there were discussions on RRM impacts due to introducing of extending current NR operation to 71GHz. The agreements/conclusions are captured in [1].

Rx beam sweeping scaling factor - Rx beam sweeping scaling factor is FFS - Option 1: The Rx beam sweeping scaling factor is increased for FR2-2 compared with FR2-1 - FFS: Scaling factor = (12 - 16) - Companies to provide the values for the scaling factor. - Option 2: Reuse the existing FR2-1 scaling factor for Rx beam sweeping for FR2-2. Scheduling restrictions when deriveSSB-IndexFromCell is enabled - Define the scheduling restrictions, when deriveSSB-IndexFromCell is enabled, based on Table below SSB SCS (kHz); Data SCS (kHz); Scheduling restriction including beam switching. 120; 120; Existing requirements. 120; 480; (3+1) 480KHz symbols. 120; 960; (6+1) 960KHz symbols. 480; 120; 1 120KHz symbol. 480; 480; (3+1) 480KHz symbols. 480; 960; (6+1) 960KHz symbols. 960; 120; 1 120KHz symbol. 960; 480; (2+1) 480KHz symbols. 960; 960; (3+1) 960KHz symbols. Scheduling restrictions when deriveSSB-IndexFromCell is disabled - When deriveSSB-IndexFromCell is not enabled for 960kHz SCS, no need to define scheduling restriction on all the symbols within the SMTC window - Define the scheduling restrictions, when deriveSSB-IndexFromCell is disabled, based on Table below SSB SCS (kHz); Data SCS (kHz); Scheduling restriction including beam switching. 960; 120; [1+1] 120KHz symbol. 960; 480; [3+1] 480KHz symbols. 960; 960; [6+1] 960KHz symbols. Cell detection - FFS: The impact of higher sampling rate and number of samples in PSS/SSS detection requirements. PBCH detection for SSB index acquisition - FFS: PBCH detection time for SSB index acquisition for 480/960 kHz SCS - When deriveSSB-IndexFromCell is not enabled, use the following definition of TSSB_time_index_intra for FR2-2: DRX cycle; Without measurement gaps; With measurement gaps. No DRX; Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra; Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra. DRX cycle≤ 320ms; Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra; Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra. DRX cycle>320ms; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra; ceil(MSSB_index_intra Kp) DRX cycle CSSFintra. MSSB_index_intra will depend on the outcome of the PBCH index detection discussion and RF decision on supported power classes for FR2-2.. - FFS: MSSB_index_intra

In this contribution, we further discuss on RRM impacts for extending NR operation to 71GHz.

Discussion

Inter-frequency SSB index acquisition time

In the last meeting, there is no clear conclusion for SSB index acquisition for FR2-2. In this contribution, we provide the simulation results of PBCH detection for SSB index acquisition based on the simulation assumptions provided in Appendix 4.1. Simulation results for SSB index acquisition are provided in Table 1.

Table 1: PBCH detection delay for FR2-2

| Case |  | SCS(kHz) |  |
| --- | --- | --- | --- |
|  |  | 480 | 960 |
| Propagation Condition | SNR (dB) | Number of acquisition attempts | Number of acquisition attempts |
| AWGN | -8 | 1 | 1 |
|  | -7 | 1 | 1 |
|  | -6 | 1 | 1 |
|  | -5 | 1 | 1 |
|  | -4 | 1 | 1 |
|  | -3 | 1 | 1 |
| TDL-A5 | -8 | 6 | 7 |
|  | -7 | 4 | 6 |
|  | -6 | 3 | 3 |
|  | -5 | 2 | 2 |
|  | -4 | 2 | 2 |
|  | -3 | 2 | 2 |
| TDL-A10 | -8 | 6 | 7 |
|  | -7 | 5 | 4 |
|  | -6 | 3 | 4 |
|  | -5 | 2 | 2 |
|  | -4 | 2 | 2 |
|  | -3 | 2 | 2 |
| TDL-A20 | -8 | 6 | 10 |
|  | -7 | 4 | 6 |
|  | -6 | 3 | 4 |
|  | -5 | 2 | 3 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 2 |
| TDL-A30 | -8 | 7 | 16 |
|  | -7 | 5 | 10 |
|  | -6 | 3 | 5 |
|  | -5 | 2 | 4 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 2 |
| TDL-C60 | -8 | 10 | 16 |
|  | -7 | 6 | 12 |
|  | -6 | 3 | 6 |
|  | -5 | 2 | 5 |
|  | -4 | 2 | 3 |
|  | -3 | 2 | 3 |

It can be observed that at least 4 samples are required for SSB index acquisition for 960kHz if the channel model from RAN1 is used. In the existing inter-frequency measurement requirements for FR2-1, 3 samples are assumed for SSB index acquisition. The requirements cannot be met for 960kHz SCS case. Therefore, the inter frequency SSB index acquisition delay should be extended.

Proposal 1: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1, i.e., 6 samples is needed for SSB index acquisition delay for inter-frequency measurement for FR2-2.

So we suggest the requirements of SSB index detection for FR2-2 for inter-frequency can be defined as below:

o	MSSB_index_inter_FR2_2 = 6 * N,

o	N is Rx beam sweeping factor for FR2- 2

Proposal 2:

The requirements of SSB index detection for FR2-2 for inter-frequency measurement can be defined as below:

Table 2: Time period for time index detection for inter-frequency measurements for FR2-2

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_inter Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | MSSB_index_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

MSSB_index_inter = 6*N, N is Rx beam sweeping factor for FR2-2.

Intra-frequency SSB index acquisition time

When deriveSSB-IndexFromCell for 960kHz is not enabled, the requirement of SSB index detection need to be clarified. In the last meeting, some companies suggested that MSSB_index_inter is calculated as 6 x N (N is Rx beam sweeping factor for FR2-2) which is the same as inter-frequency measurement. In our understanding, this would cause the measurement delay to be very long. Due to the frame boundary alignment tolerance (i.e., 6 SSB symbols) has been clarified when deriveSSB-IndexFromCell is not enabled which is helpful to reduce the measurement delay, therefore, we suggest 5 samples may be a reasonable value.

Proposal 3: 5 samples is needed for SSB index acquisition delay for intra-frequency measurement for FR2-2 when deriveSSB-IndexFromCell is not enabled.

So we suggest the requirements of SSB index detection for FR2-2 for intra-frequency can be defined as below:

o	MSSB_index_intra_FR2_2 = 5 * N,

o	N is Rx beam sweeping factor for FR2- 2

Proposal 4:

When deriveSSB-IndexFromCell is not enabled, the requirements of SSB index detection for FR2-2 for intra-frequency measurement can be defined as below:

Table 3: Time period for time index detection for intra-frequency measurements for FR2-2

| DRX cycle | Without measurement gaps | With measurement gaps |
| --- | --- | --- |
| No DRX | Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra | Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra | Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra |
| DRX cycle>320ms | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra |

MSSB_index_inter = 5*N, N is Rx beam sweeping factor for FR2-2.

Conclusion

Proposal 1: SSB index acquisition delay for inter-frequency measurement for FR2-2 should be extended by 1 sample compared with FR2-1 if using the channel model from RAN1, i.e., 6 samples is needed for SSB index acquisition delay for inter-frequency measurement for FR2-2.

Proposal 2:

The requirements of SSB index detection for FR2-2 for inter-frequency measurement can be defined as below:

Table 2: Time period for time index detection for inter-frequency measurements for FR2-2

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_inter Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | MSSB_index_inter DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

MSSB_index_inter = 6*N, N is Rx beam sweeping factor for FR2-2.

Proposal 3: 5 samples is needed for SSB index acquisition delay for intra-frequency measurement for FR2-2 when deriveSSB-IndexFromCell is not enabled.

Proposal 4:

When deriveSSB-IndexFromCell is not enabled, the requirements of SSB index detection for FR2-2 for intra-frequency measurement can be defined as below:

Table 3: Time period for time index detection for intra-frequency measurements for FR2-2

| DRX cycle | Without measurement gaps | With measurement gaps |
| --- | --- | --- |
| No DRX | Max(200ms, ceil(MSSB_index_intra Kp) SMTC period) CSSFintra | Max(200ms, ceil(MSSB_index_intra Kp) Max(MGRP, SMTC period)) CSSFintra |
| DRX cycle≤ 320ms | Max(200ms, ceil(1.5 MSSB_index_intra s Kp) Max(SMTC period, DRX cycle)) CSSFintra | Max(200ms, ceil(1.5 MSSB_index_intra Kp) Max(MGRP, SMTC period, DRX cycle)) CSSFintra |
| DRX cycle>320ms | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra | ceil(MSSB_index_intra Kp) DRX cycle CSSFintra |

MSSB_index_inter = 5*N, N is Rx beam sweeping factor for FR2-2.

Appendix

Simulation Assumptions of PBCH detection

Simulation assumptions for PBCH detection for the new band are based on R4-1713437 with changes highlighted.

Case1: PBCH reading

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

Case2: PBCH-DMRS time index reading (Without decode PBCH data)

DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.

Table 1: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 60GHz |
| Subcarrier spacing | KHz | 480kHz;960kHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 2 |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 100 |
| PBCH symbols within the SS block |  | According to the RAN1 agreements |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Channel coding |  | Polar code with 512 length and 24bit DCRC |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition [38.808] | - | AWGN/TDL-A (5/10/20ns delay spread) |
| UE speed |  | 3km/h |
| Detection Method |  | Baseline: on one shot detection (No combination for different PBCHs,) FFS Soft- combining |

Performance Metric

The proposed simulation assumptions are used to derive the Basic SI reading time.

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

- References

[1] R4-2206919 WF on NR extension to 71 GHz RRM requirements (Part 1)