3GPP TSG-RAN WG4 Meeting #103-e	R4-2208816

Electronic Meeting, May 09- May 20, 2022

Agenda item:	9.21.2

Source: 	vivo

Title: 	Discussion on RRM performance requirements for Multi-Radio Dual-Connectivity enhancements

Document for:	Discussion

Introduction

RRM core requirements for Rel-17 MR-DC enhancements have been concluded in RAN4#102-e meeting. Though there are still some remaining open issues, RAN4 can already start to define RRM test cases for the newly introduced core requirements according to the work plan.

In this paper we will provide our views on the RRM test case for MR-DC enhancements.

Discussion

For MR-DC enhancement core requirements, based on related aspects and the conclusions have been agreed upon, we list the possible scenarios below which need to be considered in the test case coverage design for MR-DC enhancements.

Table 1 scenarios of test case for MR-DC enhancements

| Scenario | Based on RACH or RACH-less | Known or unknown Cell | MR-DC_mode | Target Cell in FR1 or FR2 | Others |
| --- | --- | --- | --- | --- | --- |
| Scenario 1 | RACH-less | known | EN-DC | FR1 | RLM and BFD measurements need to be configured |
| Scenario 2 | RACH-less | known | EN-DC | FR2 |  |
| Scenario 3 | RACH-less | known | NR-DC | FR2 |  |
| Scenario 4 | RACH | known | EN-DC | FR1 |  |
| Scenario 5 | RACH | known | EN-DC | FR2 |  |
| Scenario 6 | RACH | known | NR-DC | FR2 |  |
| Scenario 7 | RACH | unknown | EN-DC | FR1 |  |
| Scenario 8 | RACH | unknown | EN-DC | FR2 |  |
| Scenario 9 | RACH | unknown | NR-DC | FR2 |  |

Proposal 1: Regarding the test case of the SCG activation/deactivation delay, 9 scenarios should be considered in the test requirement coverage:

- Scenario 1: RACH-less SCG activation and deactivation of known PSCell in FR1 in EN-DC

- Scenario 2: RACH-less SCG activation and deactivation of known PSCell in FR2 in EN-DC

- Scenario 3: RACH-less SCG activation and deactivation of known PSCell in FR2 in NR-DC

- Scenario 4: RACH SCG activation and deactivation of known PSCell in FR1 in EN-DC

- Scenario 5: RACH SCG activation and deactivation of known PSCell in FR2 in EN-DC

- Scenario 6: RACH SCG activation and deactivation of known PSCell in FR2 in NR-DC

- Scenario 7: RACH SCG activation and deactivation of unknown PSCell in FR1 in EN-DC

- Scenario 8: RACH SCG activation and deactivation of unknown PSCell in FR2 in EN-DC

- Scenario 9: RACH SCG activation and deactivation of unknown PSCell in FR2 in NR-DC

Regarding the scenarios configured with Radio link Monitoring and Beam Failure Detection, there are several aspects need to be considered including the MR-DC_mode, reference signaling and frequency range. Combined with the below factors, we list the possible cases referring to the test requirement on RLM (totally 12 cases) and BFD (totally 6 cases) for deactivated SCG.

- Test case for Radio link Monitoring on deactivated SCG

  - Case 1: Radio Link Monitoring Out-of-sync Test for FR1 deactivated PSCell configured with SSB-based RLM RS in EN-DC

  - Case 2: Radio Link Monitoring In-sync Test for FR1 deactivated PSCell configured with SSB-based RLM RS in EN-DC

  - Case 3: Radio Link Monitoring Out-of-sync Test for FR1 deactivated PSCell configured with CSI-RS-based RLM RS in EN-DC

  - Case 4: Radio Link Monitoring In-sync Test for FR1 deactivated PSCell configured with CSI-RS-based RLM RS in EN-DC

  - Case 5: Radio Link Monitoring Out-of-sync Test for FR2 deactivated PSCell configured with SSB-based RLM RS in EN-DC

  - Case 6: Radio Link Monitoring In-sync Test for FR2 deactivated PSCell configured with SSB-based RLM RS in EN-DC

  - Case 7: Radio Link Monitoring Out-of-sync Test for FR2 deactivated PSCell configured with CSI-RS-based RLM RS in EN-DC

  - Case 8: Radio Link Monitoring In-sync Test for FR2 deactivated PSCell configured with CSI-RS-based RLM RS in EN-DC

  - Case 9: Radio Link Monitoring Out-of-sync Test for FR2 deactivated PSCell configured with SSB-based RLM RS in NR-DC

  - Case 10: Radio Link Monitoring In-sync Test for FR2 deactivated PSCell configured with SSB-based RLM RS in NR-DC

  - Case 11: Radio Link Monitoring Out-of-sync Test for FR2 deactivated PSCell configured with CSI-RS-based RLM RS in NR-DC

  - Case 12: Radio Link Monitoring In-sync Test for FR2 deactivated PSCell configured with CSI-RS-based RLM RS in NR-DC

- Test case for Beam Failure Detection on deactivated SCG

  - Case 1: Beam Failure Detection for FR1 deactivated PSCell configured with SSB-based BFD in EN-DC

  - Case 2: Beam Failure Detection for FR1 deactivated PSCell configured with CSI-RS-based BFD in EN-DC

  - Case 3: Beam Failure Detection for FR2deactivated PSCell configured with SSB-based BFD in EN-DC

  - Case 4: Beam Failure Detection for FR2 deactivated PSCell configured with CSI-RS-based BFD in EN-DC

  - Case 5: Beam Failure Detection for FR2 deactivated PSCell configured with SSB-based BFD in NR-DC

  - Case 6: Beam Failure Detection for FR2 deactivated PSCell configured with CSI-RS-based BFD in NR-DC

It can be seen that the number of test cases related to RLM/BFD for deactivated SCG will be huge. Based on above analysis, the first thing we need to consider is how to simply the test case design and focus on the typical scenarios.

Observation 1: The number of test cases related to RLM/BFD for deactivated SCG will be huge if considering all of the related aspects.

Further, as for the detail test case design we prefer to consider BFD measurement within the test case for activated/deactivated SCG delay requirement rather than additionally define the separate test requirement on BFD measurement, e.g. considering BFD will be performed during the period T2.

Proposal 2: Define the BFD test requirement within the test case for activated/deactivated SCG delay requirement rather than additionally define the separate test requirement on BFD measurement.

Conclusions

Proposal 1: Regarding the test case of the SCG activation/deactivation delay, 9 scenarios should be considered in the test requirement coverage:

- Scenario 1: RACH-less SCG activation and deactivation of known PSCell in FR1 in EN-DC

- Scenario 2: RACH-less SCG activation and deactivation of known PSCell in FR2 in EN-DC

- Scenario 3: RACH-less SCG activation and deactivation of known PSCell in FR2 in NR-DC

- Scenario 4: RACH SCG activation and deactivation of known PSCell in FR1 in EN-DC

- Scenario 5: RACH SCG activation and deactivation of known PSCell in FR2 in EN-DC

- Scenario 6: RACH SCG activation and deactivation of known PSCell in FR2 in NR-DC

- Scenario 7: RACH SCG activation and deactivation of unknown PSCell in FR1 in EN-DC

- Scenario 8: RACH SCG activation and deactivation of unknown PSCell in FR2 in EN-DC

- Scenario 9: RACH SCG activation and deactivation of unknown PSCell in FR2 in NR-DC

Observation 1: The number of test cases related to RLM/BFD for deactivated SCG will be huge if considering all of the related aspects.

Proposal 2: Define the BFD test requirement within the test case for activated/deactivated SCG delay requirement rather than additionally define the separate test requirement on BFD measurement.

Reference

[1] R4-2207006 WF on R17 further Multi-RAT Dual-Connectivity enhancements, Huawei