3GPP TSG-RAN WG4 Meeting #103-e	R4-2208846

# Electronic Meeting, May 09 – May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS38.133 | CR | 2340 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.133 for the corrections on one shot large UL timing adjustment for FR2 Power Class 6 UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-Core |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Two corrections are needed for one shot large UL timing adjustment requirement: (1) It is agreed that ”UE shall apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than a timing difference threshold”, while it is not implemented in the agreed CR yet. (2) The accuracy needs to be revised for UE UL transmission timing after the TCI state switching procedure with one shot large UL timing adjustment performed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following two corrections are introduced: (1) FR2 HST UE is allowed to perform one shot large UL timing adjustment only if UE identified the DL timing is changed with the magnitude larger than one fourth of OFDM symbol CP length, i.e., 4.5*64*Tc.. (2) Adopt Te as the accuracy of UE UL transmission timing after the TCI state switching procedure with one shot large UL timing adjustment performed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The condition to trigger one shot large UL timing adjustment requirement is not aligned with existing RAN4 agreement, and the accuracy requirement of UL transmission timing is too relaxed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of Change-1 >>>

#### 7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE

When [largeOneStepUL-timingFR2-r17] is enabled for UE supporting FR2 power class 6, and UE is required to perform TCI state switching where the magnitude of the DL timing difference of the target and source TCI states exceeds the threshold [4.5]*64*Tc,

-	The requirement in clause 7.1.2.1 doesn’t apply to the first UL transmission after a TCI state switch

-	The UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew), where:

-	Tnew (in Tc units) is the DL timing defined as the time when UE receives downlink frame with new target TCI state.

-	Told  (in Tc units) is the DL timing defined as the time when UE receives downlink frame with old source TCI state.

-	The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to Te as specified in clause 7.1.2, and the reference point shall be the downlink timing of the new cell minus (NTA+NTA offset)-2 (Told-Tnew).

<<< End of Change-1 >>>