3GPP TSG-RAN4 Meeting #103-e	R4-2208883

# Electronic meeting, May 3 - 20, 2022

Source:		Ericsson

Title:			NTN: Remaining SAN RF Tx radiated issues

Agenda item:		9.12.3.1

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4#102-e meeting, many SAN RF requirements have been agreed and captured in TS 38.108. Nevertheless, some open issues are remaining, those are further discussed in this contribution

# Discussion

## Background

In last RAN4#102-e meeting, the “WF on SAN SEM and spurious emission” ([3]) was agreed. This WF has captured the different options discussed in that meeting, without reaching any agreement on the limits to be added to TS 38.108.

In our other contribution [4], we have given the detailed rationale for the proposed spurious and OBUE requirements. In this contribution, we would then only made our equivalent radiated spurious and OBUE requirements, the reader should refer to [4] for their rationale.

## Spurious emission

Proposal1: The SAN spurious requirement should be specified according to the following Table 1:

| Spurious frequency range | Prated,c,TRP(W) | Basic limit (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 30 MHz – 12.75 GHz | ≤ 50 W | -13 | 4 kHz |
|  | > 50 W | 10 Log(Prated,c,TRP(W)) – 30 |  |

Table 1: SAN spurious requirement

## OBUE

Proposal2: The following OBUE masks should be specified for SAN GEO and LEO classes according to the following Table 3 and Table 4:

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 7dBm-75foffsetMHz-0.002 dBm | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 0 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Aligned with SAN spurious limit | 4kHz |

Table 3: SAN GEO Class OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | -7dBm-75foffsetMHz-0.002dBm+ ΔLEOType | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -14 dBm + ΔLEOType | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Aligned with SAN spurious limit | 4kHz |
| NOTE: ΔLEOType = 0dBm for LEO 600 km satellite and 6dBm for LEO 1200 km satellite |  |  |  |

Table 4: SAN LEO/LEO600 OBUE basic limits

# Conclusion

In this contribution, we discussed the remaining open issues for TS 38.108 specifications. We made the following observations and proposals:

Proposal1: The SAN spurious requirement should be specified according to the following table:

| Spurious frequency range | Prated,c,TRP(W) | Basic limit (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 30 MHz – 12.75 GHz | ≤ 50 W | -13 | 4 kHz |
|  | > 50 W | 10 Log(Prated,c,TRP(W)) – 30 |  |

Proposal2: The following OBUE masks should be specified for SAN GEO and LEO classes according to the following tables:

SAN GEO Class - OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | 7dBm-75foffsetMHz-0.002 dBm | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | 0 dBm | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Aligned with SAN spurious limit | 4kHz |

SAN LEO Class - OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.002 MHz f_offset < 5.002 MHz | -7dBm-75foffsetMHz-0.002dBm+ ΔLEOType | 4kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.002 MHz f_offset < min(10.002 MHz, f_offsetmax) | -14 dBm + ΔLEOType | 4kHz |
| 10 MHz f fmax | 10.002 MHz f_offset < f_offsetmax | Aligned with SAN spurious limit | 4kHz |
| NOTE: ΔLEOType = 0dBm for LEO 600 km satellite and 6dBm for LEO 1200 km satellite |  |  |  |

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2207456, Way Forward on SAN SEM and spurious emission, Thales, ZTE

- R4-2208882, NTN: Remaining SAN RF Tx radiated issues, Ericsson