3GPP TSG-RAN4 Meeting #103-e	R4-2208885

# Electronic meeting, May 3 - 20, 2022

Source:		Ericsson

Title:			NTN: Remaining UE satellite RF Rx issues

Agenda item:		9.12.5.2

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#94-e meeting [2].

In last RAN4#102-e meeting, many satellite UE RF requirements have been agreed and captured in TS 38.101-5. Nevertheless, some open issues are remaining, those are further discussed in this contribution

# Discussion

## Background

In last RAN4#102-e meeting, the “WF on NTN UE RF requirements” ([3]) was agreed. This WF has captured the different options discussed in that meeting, without reaching any agreement on the limits to be added to TS 38.101-5.

## n256 duplexer

In last RAN4#102-e meeting, the type of UE duplexer to be considered for n256 was discussed. Two options have been considered, a preliminary decision on option 2 was made but was finally challenged at the end of the meeting:

- Option 1: Consider only a dedicated 30 MHz duplexer for n256.

- Option 2: Consider both type of duplexer: reusing band n65 duplexer (90MHz wide) or a new 30MHz duplexer.

Both options have their advantages and their drawbacks. If reusing a n65 duplexer might first look attractive, it would also impact n256 UE performances:

- Some NR bands protection might be challenging.

- If n65 duplexer is used, it’s unclear how band n1 should be protected.

- REFSENS requirement would have to be relaxed by 0.5 dB.

- Blocking requirement would have to be relaxed as well: the lower 60MHz adjacent to band n256 could not be filtered properly when reusing a n65 duplexer.

Based on the above, we would make the following proposal:

Proposal: Consider only dedicated 30MHz duplexer for UE operating in band n256. REFSENS and out of band blocking should be specified according to the following tables:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |

Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

|  | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

# Conclusion

In this contribution, we discussed the remaining open issues for UE RF Rx parameters. We made the following proposal:

Proposal: Consider only dedicated 30MHz duplexer for UE operating in band n256. REFSENS and out of band blocking should be specified according to the following tables:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n256 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | FDD |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |

Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

|  | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255, n256 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-213691, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2207446, WF on NTN UE RF requirements, ZTE