3GPP TSG-RAN WG4 Meeting #103-e	R4-2208988

# Online, May 09 – 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on maintaining PL-RS switching delay requirements R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The existing PL-RS switching delay are only applicable when UE does not need to perform beam sweeping on the target PL-RS. However, when a SSB is configured as the PL-RS in FR2, UE needs to perform beam sweeping on the SSB regardless of whether the SSB is known or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To clarify that longer application time is expected if in FR2 the target PL-RS is a SSB on which UE performs L1-RSRP measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PL-RS switching delay requirements are not properly defined for SSB based PL-RS in FR2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 8.14	Pathloss reference signal switching delay

#### 8.14.1	Introduction

The requirements in this clause apply for pathloss reference signal activated or updated on serving cell in MR-DC or standalone NR in clause 7.1.1 in TS 38.213 [3].

UE shall complete the switch of pathloss reference signal within the delay defined in this clause.

#### 8.14.2	Known conditions for pathloss reference signal

The pathloss reference signal is known if the following conditions are met during the period between the last transmission of the RS resource used for L1-RSRP measurement reporting and the completion of pathloss reference signal switch, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal.

-	Pathloss reference signal switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target pathloss reference signal before the pathloss reference signal switch command

-	The target pathloss reference signal remains detectable during the pathloss reference signal switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the pathloss reference signal switching period

-	SNR of the associated SSB ≥-3dB

Otherwise, the pathloss reference signal is unknown.

#### 8.14.3	MAC-CE based pathloss reference signal switch delay

The requirements in this clause apply for a UE to update a pathloss reference signal by MAC-CE for PUCCH, PUSCH, semi-persistent SRS and aperiodic SRS.

If the target pathloss reference signal is known, upon receiving PDSCH carrying MAC-CE activation in slot n, UE shall be able to apply the target pathloss reference signal of the serving cell on which pathloss reference signal switch occurs no later than the slot n + THARQ+3Nslotsubframe,µ + NM* 5*Ttarget_PL-RS + 2msNR slot length.  The UE shall be able to apply old pathloss reference signals until the slot n + THARQ+ 3Nslotsubframe,µ. Where

-	THARQ is the timing between pathloss reference MAC-CE activation command and acknowledgement as specified in TS 38.321 [7].

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS.

Note:	longer application time is expected if measurement sample is not available due to measurement gap, DRX or other UE activities.

Note:	longer application time is expected if the pathloss reference signal is unknown or if in FR2 the target PL-RS is SSB on which UE performs L1-RSRP measurements.

<End of Change 1>