3GPP TSG-RAN WG4 Meeting #103-e	R4-2209006

# Online, May 09 – 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2354 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on maintaining TCI state switching requirements for R17 unified TCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R17, the TCI state switching requirements have been introduced and separately defined for DL and UL. However, for MAC-CE based UL state switching, the delay requirement is still FFS when PL-RS is SSB in FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MAC-CE based UL TCI state switching requirements have been updated. - To define the switching delay requirements when PL-RS is SSB in FR2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL TCI state switching requirements for unified TCI are not completed in R17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of change 1>

## 8.16	Active uplink TCI state switching delay for unified TCI

### 8.16.1	Introduction

The requirements in this clause apply for a UE configured with one or more [TCI state] configurations for UL channels/signals and/or one or more [joint TCI state] configurations for both DL and UL channels/signals and/or one or more [pair of TCI states] configurations with one TCI state for DL channels/signals and one TCI state for UL channels/signals on serving cell or a cell with PCI different from a serving cell in MR-DC or standalone NR. There is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter [TCI state] associated to SRS. UE shall complete the switch of active uplink TCI state within the delay defined in this clause when the UE is requested to switch to a TCI state with the higher layer parameter [TCI state] associated to a DL RS.

The requirements in this clause for a cell with different PCI from serving cell are applicable for such cell is known for a UE. A cell with different PCI from serving cell is known if the the following conditions are met

[Active BWP of cell with different PCI shall be equal to the active BWP of serving cell or initial DL BWP of UE]

Center frequency, SCS and SFN offset of a cell with different PCI from serving cell are as the same as serving cell.

During the last [X]s before L1-RSRP measurement is configured, the UE has sent a valid L3 measurement report for the cell with different PCI;

Timing offset between serving cell and the cell with different PCI from serving cell are within CP.

### Otherwise, the cell with different PCI from serving cell is unknown.

### 8.16.2	Known conditions for uplink TCI state

The uplink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target uplink TCI state to the completion of active uplink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target uplink TCI state or QCLed to the target uplink TCI state

-	Uplink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target uplink TCI state before the uplink TCI state switch command

-	The target uplink TCI state remains detectable during the uplink TCI state switching period

-	The SSB associated with the uplink TCI state remain detectable during the uplink TCI switching period

-	SNR of the uplink TCI state ≥ -3dB

-	The SSB can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

Otherwise, the uplink TCI state is unknown.

### 8.16.3	MAC-CE based uplink TCI state switch delay

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework.

PL-RS may be associated with or included in UL TCI state or joint TCI state. The requirements in this clause shall apply if the following conditions are met:

- PL-RS is identical to source RS in UL TCI state or joint TCI state

- PL-RS and source RS in UL TCI state or joint TCI state are QCL-Type D

In case that source RS in UL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.16.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

    - If target TCI state is known,

- If the target PL-RS is SSB in FR2

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (5*TL1-RSPR_Measurement_Period_SSB + 2ms).

- Otherwise

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms).

    - If target TCI state is unknown,

- If the target PL-RS is SSB in FR2

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (5*TL1-RSPR_Measurement_Period_SSB + 2ms).

- Otherwise

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms.

Where,

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after L1-RSRP measurement when target TCI state is unknown.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after MAC CE command is decoded by the UE for known TCI State.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS when PL-RS is associated with serving cell

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB when PL-RS is associated with PCI different from serving cell

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

### 8.16.4	DCI based uplink TCI state switch delay

When a UE is configured with the higher layer parameter with [tci-StateId_r17] for unified TCI switching and receives DCI format 1_1/1_2 with or without DL assignment providing indicated TCI-State with [tci-StateId_r17] for a CC, the UE transmits a PUCCH with HARQ-ACK information corresponding to the DCI carrying the TCI-State indication.

If the target TCI state is known, the uplink TCI switching to the indicated TCI-State in the DCI format shall be completed starting from the first slot that is at least [BeamAppTime_r17] symbols after the last symbol of the PUCCH carrying HARQ-ACK in response to the DCI triggering TCI state activation. The first slot and the [BeamAppTime_r17 ] symbols are both determined on the carrier with the smallest SCS among the carrier(s) applying the beam indication. The value of [BeamAppTime_r17] is defined in TS 38.331 [2]. The known condition for TCI state defined in clause [8.16.2] is applied. [Editor note : PL-RS is maintained for DCI based UL TCI state switch.]

Editor note : TBD how to capture that pathloss reference singal is switched as a part of uplink TCI state switching.

### 8.16.5	Active Uplink TCI state list update delay

The requirements specified in this clause apply to

-	Separate active TCI list update delay, while the separate TCI list comprises more than one target separate TCIs, and at least one UL TCI is included.

-	Joint active TCI list update delay, while the joint TCI list comprises more than one target joint TCI.

Upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to [FFS: ‘receive PDCCH to schedule PDSCH’ or ‘transmit PUCCH, PUSCH or SRS ’] with the new target TCI states at the first slot that is after

n+ 3Nslotsubframe,µ + (THARQ + NM * (Tfirst_target-PL-RS_List + 4 * Ttarget_PL-RS_List + 2ms))) / NR slot length,

Where

-	If all TCIs are known,

-	if the target PL-RS associated with or included in any UL TCI is not maintained by the UE, NM = 1,

-	If the number of cells associated with the target TCIs that are [not in the active TCI list] is larger than 1, and SSBs associated to the TCIs are overlapped,

-	Tfirst_target-PL-RS_List = Ncell * Tfirst_target-PL-RS, Ttarget-PL-RS_List = Ncell * Ttarget-PL-RS, where Ncell is the number of cells associated with the target TCIs that are not in the active TCI list, whose SSBs are overlapped. Ncell ≤ [Nmax] + 1, where [Nmax] is the number of cells with PCI different from serving cell, and [Nmax] = 1.

-	Otherwise,

-	Tfirst_target-PL-RS_List = Tfirst_target-PL-RS, Ttarget-PL-RS_List = Ttarget-PL-RS.

-	THARQ, Tfirst_target-PL-RS, Ttarget-PL-RS are defined in clause 8.16.3.

When UE receives PDSCH carrying MAC-CE active TCI state list update for

-	joint TCI state list update, or

-	separate TCI list update, while the separate TCI list comprises at least one DL TCI and one UL TCI,

UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch.

### <End of change 1>