3GPP TSG-RAN4 Meeting #103-e	R4-2209020

# Electronic Meeting, May 9-20, 2022

Source:	Huawei, Hisilicon

Title:	TP for TR 37.717-21-11 DC_32A-38A_n28A

Agenda item:			8.12.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_32A-38A_n28A.

The TIB,c and RIB values provided for B32 and n28 are based on the coexistence analysis previously conducted for DC_8-32_n1.

2. Text Proposal

<Start of Text Proposal>

5.x	DC_32A-38A_n28A

5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_32A-38A_n28A | DC_38A_n28A |

5.x.2	Co-existence studies

Based on co-existence studies of Band 38 + Band n28, own Rx impact of the 3rd band is the followings.

-	2nd order harmonics generated by Band n28 uplink may fall into own Rx of band 32.

5.x.3	∆TIB and ∆RIB values

For DC_32-38_n28, the TIB,c and RIB,c values are reused from the LTE combination DC_7-32_n8, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_32-38_n28 | 38 | 0.7 |
|  | n28 | 0.6 |

Table 5.127.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_32-38_n28 | 32 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

5.x.4	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

No additional MSD requirement needs to be defined for this dual connectivity configuration.

<End of Text Proposal>