3GPP TSG-RAN WG4 Meeting #103-e                                                    	R4-2209021

Online, 17th August-28th April, 2020

Source:	Huawei, Hisilicon

Title:	TP for TR 37.717-31-11 DC_3A-32A-38A_n28A and DC_3C-32A-38A_n28A

Agenda item:			8.13.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-31-11 to include DC_3A-32A-38A_n28A and DC_3C-32A-38A_n28A.

# 2. Text Proposal

<Start of Text Proposal>

5.1.x	DC_3-32-38_n28

5.1.x.1	 Configurations for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-32A-38A_n28A DC_3C-32A-38A_n28A | DC_3A_n28A DC_38A_n28A |

5.x.2	Co-existence studies

Based on co-existence studies of Band 38 + Band n28 and Band 3 + Band n28, own Rx impact of the 4th band is the followings.

- The 2nd order harmonics generated by Band n28 may fall into own Rx of Band 32.

- IMD5 may fall into Rx of band 32 when band 3 and n28 transmit.

5.1.x.2	 ∆TIB and ∆RIB values

For DC_3-32-38_n28, the TIB,c and RIB,c values are reused from the band combination DC_ 7-8-32_n1, and are given in the tables below.

Table 5.1.x.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-32-38_n28 | 3 | 0.7 |
|  | 38 | 0.7 |
|  | n28 | 0.6 |

Table 5.1.x.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-32-38_n28 | 3 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

5.1.x.3		Reference sensitivity exceptions

No additional MSD requirement needs to be defined for this dual connectivity configuration.

<End of Text Proposal>