3GPP TSG-RAN WG4 Meeting #103-e	R4-2209058

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.19.5.1

Source:	Ericsson

Title:	Open issues on UE demodulation and CSI requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with the WF on RedCap demodulation and CQI reporting requirements [1]. This contribution discusses the open issues related to both UE demodulation and CSI reporting requirements for RedCap UEs.

# 2	TS38.101-4 structure for RedCap demodulation and CSI reporting requirements

RAN4 main session agreed to define RedCap UE RF requirements with the dedicated sub-clauses with suffix ‘I’ in TS38.101-1/2 according to TS38.101-1 Table 4-3.1 as follows.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation(CA) with UL MIMO |
| I | RedCap |

Since RAN4 agreed to define at least PDSCH/PDCCH/PBCH/SDR/CQI/PMI requirements for RedCap, we propose to define RedCap demodulation and CSI reporting requirements also with the suffix ‘I’ as same as TS38.101-1/2. The detailed specification structure is proposed in [3].

Proposal 1: Define RedCap demodulation and CSI reporting requirements in TS38.101-4 with the suffix ‘I’, as same as TS38.101-1/2.

# 3	Requirement applicability for HD-FDD UE

One remaining open issue for RedCap UE demodulation and CSI reporting requirements is the UL/DL pattern used for 1Rx UE in FDD bands.

- Assume Full-duplex FDD to derive the requirements, e.g., SNR to achieve 70% of maximum throughput. - For the requirement specification, - Option 1: Consider Full-duplex FDD only - Option 2: Consider Full-duplex FDD and half-duplex FDD with applicability rule

For RedCap WI, RAN1 has defined two feature groups: one for FG 28-1 ‘RedCap UE’ and another one for FG 28-3 ‘Half-duplex FDD operation type A for RedCap UE’ [2]. According to RAN1 agreements, FG 28-1 is the fundamental signaling for UE capable of RedCap, such as the supporting of the separated DL BWP for RedCap UE and early indication in Msg1/MsgA. On the other hand, FG 28-3 is the signaling to indicate that UE is capable of the half-duplex FDD operation type A in FR1 FDD bands.

RAN#95-e concluded that FG 28-3 is reported per band, and UE is assumed to support full-duplex FDD in FDD band if half-duplex FDD is not supported. This means that UE is not mandated to support full-duplex FDD if UE supports half-duplex FDD in FDD bands.

conclusion: - "FG 28-1 is reported per UE, and FG 28-3 is reported per band" is agreed - RP-220462 is not pursued, no further RAN4 discussion on these bands, in case of ambiguity problems, they can be addressed under essential cat.F CR maintenance - slide 6 2 main bullets are endorsed - RP-220964 is noted, RP-220965 and RP-220966 are approved - It is not pursued to support RRM relaxation for non-RedCap UE in Rel-17

Observation 1: According to RAN1 feature list and RAN#95-e decision, supporting half-duplex FDD is per-band capability, i.e., UE is not mandated to support full-duplex FDD if UE support half-duplex FDD.

If we consider full-duplex FDD only for FDD test as option 1, UE capable of HD-FDD only (e.g., UE optimized for RedCap supporting a few FDD bands with half-duplex) cannot be tested. To avoid this, we propose to take option 2, i.e., consider both full-duplex FDD and half-duplex FDD with applicability rule for RedCap 1Rx UE in FDD.

Proposal 2: For the requirement specification of FDD 1Rx UE, consider full-duplex FDD and half-duplex FDD with applicability rule.

Proposal 3: Specify the following applicability table for RedCap UE supporting half-duplex FDD.

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Half-duplex FDD operation type A for RedCap UE | FR1 Half-duplex FDD | PDSCH | All tests in Clause [5.2I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | PDCCH | All tests in Clause [5.3I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | PBCH | All tests in Clause [5.4I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | SDR | All tests in Clause [5.5I.1] | The requirements apply for UE supporting 1Rx |

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Half-duplex FDD operation type A for RedCap UE | FR1 Half-duplex FDD | CQI | All tests in Clause [6.2I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | PMI | All tests in Clause [6.3I.1.1] | The requirements apply for UE supporting 1Rx |

As we discussed in RAN4#102-e, half-duplex FDD UE cannot perform UL transmission and DL reception simultaneously, and it requires a few symbols duration for RF center frequency retuning. This is equivalent to the TDD operation from UE demodulation point of view. We therefore propose to define the demodulation and CSI reporting requirements for FDD with the TDD UL/DL pattern FR1.15-1 (i.e., DDDSU) to cover both FD-FDD and HD-FDD. This means the FRC is specified based on HD-FDD pattern and CSI feedback delay may need to be adjusted.

Proposal 4: Define FRC of UE demodulation and CSI reporting requirements for RedCap 1Rx UE in FDD by assuming the half-duplex FDD operation, that is, DDDSU.

# 4	Summary

Proposal 1: Define RedCap demodulation and CSI reporting requirements in TS38.101-4 with the suffix ‘I’, as same as TS38.101-1/2.

Observation 1: According to RAN1 feature list and RAN#95-e decision, supporting half-duplex FDD is per-band capability, i.e., UE is not mandated to support full-duplex FDD if UE support half-duplex FDD.

Proposal 2: For the requirement specification of FDD 1Rx UE, consider full-duplex FDD and half-duplex FDD with applicability rule.

Proposal 3: Specify the following applicability table for RedCap UE supporting half-duplex FDD.

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Half-duplex FDD operation type A for RedCap UE | FR1 Half-duplex FDD | PDSCH | All tests in Clause [5.2I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | PDCCH | All tests in Clause [5.3I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | PBCH | All tests in Clause [5.4I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | SDR | All tests in Clause [5.5I.1] | The requirements apply for UE supporting 1Rx |

| UE feature/capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Half-duplex FDD operation type A for RedCap UE | FR1 Half-duplex FDD | CQI | All tests in Clause [6.2I.1.1] | The requirements apply for UE supporting 1Rx |
|  | FR1 Half-duplex FDD | PMI | All tests in Clause [6.3I.1.1] | The requirements apply for UE supporting 1Rx |

Proposal 4: Define FRC of UE demodulation and CSI reporting requirements for RedCap 1Rx UE in FDD by assuming the half-duplex FDD operation, that is, DDDSU.

# References

- R4-2207206 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

- R4-2202927, “LS on updated Rel-17 RAN1 UE features list for NR”, RAN1

- R4-2209057, “draft big CR: Introduction of UE demodulation and CSI reporting requirements for RedCap”, Ericsson.

# A.1	RAN1 feature list for NR_redcap [2]

| Features | Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap Ues - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included FFS whether to add any other basic features for RedCap UE |  | [Network assumes the UE is not a RedCap UE] | [Per UE] | Optional with capability signaling RedCap UE must indicate this FG is supported |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | [UE is assumed to support FD-FDD in FDD bands] | [Per band] | Optional with capability signaling |

# A.2	Updated simulation assumption

## A.2.1	PDSCH demodulation

### A.2.1.1	FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse |

### A.2.1.2	FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New |

### A.2.1.3	FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse |

## A.2.2	PDCCH demodulation

### A.2.2.1	Candidate options for FR1 FDD 15kHz

| A.2.2.1 Candidate options for FR1 FDD 15kHz1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | New |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | New |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | Reuse |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | Reuse |

### A.2.2.2	Candidate options for FR1 TDD 30kHz

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | [2] |  |  |
| Shift Index |  | 0 |  |  |  |

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | [1] | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 (REV R.PDCCH.2-1.2 TDD) | New |
| 1Rx UE | 20 | 48 | [1] | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 (REV R.PDCCH.2-1.3 TDD) | New |
| 2Rx UE | 20 | 48 | [1] | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 (REV R.PDCCH.2-1.2 TDD) | New |
| 2Rx UE | 20 | 48 | [1] | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 (REV R.PDCCH.2-1.3 TDD) | New |

| Parameters | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | REV R.PDCCH.2-1.2 TDD | REV R.PDCCH.2-1.3 TDD |
| Subcarrier spacing | kHz | 30 | 30 |
| CORESET frequency domain allocation |  | 48 | 48 |
| CORESET time domain allocation |  | 1 | 1 |
| Aggregation level |  | 4 | 8 |
| DCI format |  | 1_1 | 1_1 |
| Payload (without CRC) | bits | 52 | 52 |

### A.2.2.3		Candidate options for FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 1-2 (R.PDCCH. 5-1.2 TDD) | Reuse |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2-1 (R.PDCCH.5-1.3 TDD) | Reuse |

## A.2.3	PBCH demodulation

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCH block index | Reference from TS38.101-4 | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | New |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | New |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Unknown | 5.4.2.1-2 Test 1 | Reuse |
| 2Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Known | 5.4.2.1-3 Test 1 | Reuse |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Unknown | 5.4.2.2-2 Test 1 | Reuse (*1) |
| 2Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Known | 5.4.2.2-3 Test 1 | Reuse (*1) |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 1 | Reuse |
| 2Rx UE | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 1 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Unknown | 7.4.2.2-2 Test 2 | Reuse |
| 2Rx UE | 100MHz / 240kHz | TDLA30-75 | 1x2 low | 1% | Known | 7.4.2.2-3 Test 2 | Reuse |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |

## A.2.4	SDR

- Define SDR tests for RedCap 1Rx/2Rx UE

  - Define only 2Rx requirements for FR2.

## A.2.5	CQI reporting requirements

### A.2.5.1	FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 1Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 ULA high | New |

### A.2.5.2	FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 1Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 2 ULA high | New |

### A.2.5.3	FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | Table 8.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 100 / 120 | Table 1 | 2 | 2 x 2 ULA high | Reuse |
| 2Rx UE | Table 8.2.2.2.2.1-1 Tests 1/2 | Wideband CQI reporting test | 100 / 120 | Table 1 | 1 | 2 x 2 ULA high | Reuse |

## A.2.6	PMI reporting requirements

### A.2.6.1 FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.1.1-1 Test 1 | Single PMI, Type I, rank 1 | 10 / 15 | 4 | 4 x 2 XP high | Reuse |

### A.2.6.2 FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 1 ULA high | New |
| 2Rx | Table 6.3.2.2.1-1 Test 1 | Single PMI, Type I, rank 1 | 20 / 30 | 4 | 4 x 2 XP high | New |

### A.2.6.3 FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | Reference | PMI test | CBW / SCS | Tx ports | Antenna Configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 2Rx | Table 8.3.2.2.1-1 Test 2 (TDD FR2.120-1) | Single PMI, Type I, rank 1 | 100 / 120 | 2 | 2 x 2 ULA low | Reuse |

## A.2.7	Antenna configuration

### A.2.7.1	Static propagation condition for UE receiver with 1Rx

For 2 port transmission the channel matrix is defined in the frequency domain by

H=11