3GPP TSG-RAN WG4 Meeting #103-e	R4-2209060

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.19.5.2.2

Source:	Ericsson

Title:	PDCCH/PBCH demodulation requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with WF on RedCap demodulation and CQI reporting requirements [1]. This contribution provides our initial simulation results and discusses the open issues on PDCCH and PBCH demodulation requirements for RedCap.

# 2	PDCCH demodulation

## 2.1	Discussion

The remaining open issue for PDCCH demodulation requirements is the configuration of aggregation levels.

Aggregation level(s) - Option 1: Specify PDCCH demodulation requirements with AL4 and AL8. - Option 2: Specify PDCCH demodulation requirements with AL4 or AL8.

We still think it is worth defining both AL4 (1Tx) and AL8 (2Tx) for RedCap to ensure the test coverage. However we may understand the proposal there is no baseband processing difference between AL4 and AL8. If the proponents of Option 2 strongly prefer to choose either AL4 or AL8, one possibility is to choose AL4 only for PDCCH. This is because of the reason the existing PDSCH demodulation requirements configures AL8 for PDCCH to transmit DCI 1_1 (See TS 38.101-4 Table 5.2-1 for FR1 and Table 7.2-1 for FR2). If we review the simulation assumption of PDSCH in [1], it assumes 2Tx. Since UE need to decode PDCCH for PDSCH demodulation requirements, and our simulation results show the required SNR for PDSCH QPSK 1/3 is close to PDCCH AL8, we can say PDCCH AL8 DCI 1_1 is implicitly verified with the PDSCH demodulation tests.

Proposal 1: Define both AL4/AL8 for PDCCH demodulation requirements for RedCap.

Observation 1: PDSCH demodulation test uses PDCCH AL8 with 2Tx to transmit DCI 1_1.

Proposal 2: If RAN4 need to choose either AL4 or AL8, choose AL4 for FR1 FDD/TDD and FR2 TDD. PDCCH AL8 performance can be guaranteed during the PDSCH demodulation tests.

For TDD 30kHz, according to the offline discussion, we propose to use the PDCCH parameters given in Table 1, where REG bundle size for AL8 is changed from 6 to 2 because TS38.211 does not allow to configure REG bundle size 6 with interleaver size 3 if RB size is set to 48RB.

Table 1	Common PDCCH parameters for TDD 30kHz.

| Parameters | 1Tx/2Tx antennas |
| --- | --- |
| TDD UL-DL pattern | FR1.30-1 |
| CCE to REG mapping type | Interleaved |
| Interleaver size | 3 |
| REG bundle size | 2 |
| Shift index | 0 |

Proposal 3: Set REG bundle size to 2 for FR1 TDD PDCCH demodulation requirements.

RAN4 set 2 CORESET durations for TDD in [1], but it is also possible to 1 CORESET duration for AL4/8 even with 48RB. Since it is aligned with existing PDCCH demodulation requirements for TDD, we propose to configure 1 CORESET duration for TDD PDCCH demodulation requirements.

Proposal 4: Set CORESET duration to 1 for FR1 TDD PDCCH demodulation requirements.

## 2.1	FDD 15kHz

Table 2 summarizes our PDCCH simulation results for FDD 15kHz according to [1].

Table 2	Summary of PDCCH simulation results for FDD 15kHz.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | SNR to achieve 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x1 | 1% | 5.3.2.1.1-1 Test 3 | 10.5 |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 | 3.5 |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 | Reuse |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 | Reuse |

## 2.2	TDD 30kHz

Table 3 summarizes our PDCCH simulation results for TDD 30kHz according to [1] and Table 1.

Table 3	Summary of PDCCH simulation results for TDD 30kHz.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | SNR to achieve 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | 5.5 |
| 1Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 5.3.2.2.2-1 Test 1 | 1.1 |
| 2Rx UE | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 | 1.2 |
| 2Rx UE | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 | -1.7 |

# 3	PBCH demodulation

Table 4 summarizes our PBCH simulation results for the case SS/BPCH block index is unknown.

Table 4	Summary of PBCH simulation results.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCH block index | Reference from TS38.101-4 | SNR to achieve 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | -1.9 |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | 0.2 |
| Note 1: RAN4 can keep the same SNR requirements but need to change the channel bandwidth configuration. |  |  |  |  |  |  |  |

# 4	Summary

Proposal 1: Define both AL4/AL8 for PDCCH demodulation requirements for RedCap.

Observation 1: PDSCH demodulation test uses PDCCH AL8 with 2Tx to transmit DCI 1_1.

Proposal 2: If RAN4 need to choose either AL4 or AL8, choose AL4 for FR1 FDD/TDD and FR2 TDD. PDCCH AL8 performance can be guaranteed during the PDSCH demodulation tests.

Proposal 3: Set REG bundle size to 2 for FR1 TDD PDCCH demodulation requirements.

Proposal 4: Set CORESET duration to 1 for FR1 TDD PDCCH demodulation requirements.

# References

- R4-2207206 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.