3GPP TSG-RAN WG4 Meeting #103-e	R4-2209061

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.19.5.3.1

Source:	Ericsson

Title:	CQI reporting requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with WF on RedCap demodulation and CQI reporting requirements [1]. This contribution provides our initial simulation results and discusses the open issues on CQI reporting requirements for RedCap.

# 2	CQI reporting in static condition

Figure 1 shows the median of the reported CQI according to the simulation parameters in Table 1. Since RAN4 agreed to use CQI table 1, we used the CQI-MCS mapping table shown in Table 5.

Table 1	Simulation parameters for CQI reporting test in static condition.

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | FDD 15kHz Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | FDD 15kHz Table 6.2.2.1.1.1-1 Test 1/2 | CQI report definition test | 10 / 15 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 1Rx UE | TDD 30kHz Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 1 | 2 x 1 with static channel specified in Annex B.1 in TS 38.101-4 | New |
| 2Rx UE | TDD 30kHz Table 6.2.2.2.1.1-1 Test 1/2 | CQI report definition test | 20 / 30 | Table 1 | 2 | 2 x 2 with static channel specified in Annex B.1 in TS 38.101-4 | New |

| (a) FDD SCS=15kHz | (b) TDD SCS=30kHz |
| --- | --- |

Figure 1	Median CQI for RedCap UE.

The remaining open issue for CQI reporting test in static condition is the number of test points.

Test points for CQI reporting test with CQI table 1 - Set two SNR test points based on simulation results to cover SNR region with CQI report corresponding to: - 64QAM - QPSK or 16QAM

In Rel-15 NR CQI reporting tests in static condition, RAN4 sets the SNR test points to 8/9dB and 14/15dB for 2Rx. For 4Rx UE, it is set to 5/6dB and 11/12dB, i.e., 3dB lower than 2Rx cases. For RedCap UE, since we agreed to use CQI table 1, if we reuse the same test points as the existing test case, SNR=8/9dB and SNR=14/15dB corresponds to CQI index with 16QAM and 64QAM for 2Rx Rank 2. Then we prefer to set SNR test points corresponding to 16QAM and 64QAM. According to our simulation results in Figure 1, SNR range corresponding to QPSK is ≤4dB for 2Rx Rank 2 and ≤1dB for 1Rx Rank 1. We think the SNR test points corresponding to QPSK are low from measurement accuracy point of view and this is also the reason to propose 16QAM and 64QAM.

Proposal 1: For the static condition CQI reporting test with CQI table 1, set two SNR test points corresponding to CQI indexes of 16QAM and 64QAM for RedCap 1Rx and 2Rx UEs.

Regarding the test points, according to our simulation results in Figure 1, we propose to reuse the existing test point for RedCap 2Rx UE, that is, SNR=8/9dB and 14/15dB. For RedCap 1Rx UE, if we follow the existing CQI reporting requirements, we should set SNR=11/12dB and SNR=17/18dB. Unlike Rel-15 tests, however RedCap 1Rx UE uses rank 1 and according to our simulation results, we propose to set SNR=5/6dB and 11/12dB.

Proposal 2: For the static condition CQI reporting test with CQI table 1 for 2Rx UEs (rank 2), set SNR=8/9dB and SNR=14/15dB.

Proposal 3: For the static condition CQI reporting test with CQI table 1 for 1Rx UEs (rank 1), set SNR=5/6dB and SNR=11/12dB.

# 3	CQI reporting in fading condition

Table 3 and Table 4 shows our simulation results of the CQI reporting in fading condition according to the simulation parameters in Table 2, where the CQI-MCS mapping table is shown in Table 5. Note the rank highlighted in yellow in Table 2 is different from [1], and this is the same configuration as TS38.101-4 Table 6.2.2.1.2.1-1 Test 1/2.

Table 2	Simulation parameters for CQI reporting test in fading condition.

| 1Rx UE or 2Rx UE | Reference | CQI test | CBW / SCS | CQI table | Rank | Antenna configuration | New / Reuse |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | FDD 15kHz Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | FDD 15kHz Table 6.2.2.1.2.1-1 Test 1/2 | Wideband CQI reporting test | 10 / 15 | Table 1 | 1 | 2 x 2 ULA high | New |
| 1Rx UE | TDD 30kHz Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 1 ULA high | New |
| 2Rx UE | TDD 30kHz Table 6.2.2.2.2.1-1 Test 1/2 | Wideband CQI reporting test | 20 / 30 | Table 1 | 1 | 2 x 2 ULA high | New |

Table 3	Simulation results of CQI reporting test in fading condition for FDD SCS=15kHz.

|  | 1Rx UE |  |  | 2Rx UE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | BLER with following CQI | TP ratio with following CQI and Median CQI | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | BLER with following CQI | TP ratio with following CQI and Median CQI |
| 0 | 0.27 | 0.13 | 1.49 | 0.49 | 0.11 | 1.45 |
| 2 | 0.40 | 0.08 | 1.45 | 0.51 | 0.11 | 1.45 |
| 4 | 0.50 | 0.08 | 1.48 | 0.51 | 0.13 | 1.38 |
| 6 | 0.54 | 0.08 | 1.48 | 0.53 | 0.15 | 1.33 |
| 8 | 0.55 | 0.09 | 1.43 | 0.52 | 0.16 | 1.38 |
| 10 | 0.58 | 0.12 | 1.37 | 0.52 | 0.18 | 1.30 |
| 12 | 0.57 | 0.13 | 1.44 | 0.49 | 0.17 | 1.31 |
| 14 | 0.55 | 0.15 | 1.34 | 0.38 | 0.15 | 1.35 |
| 16 | 0.52 | 0.16 | 1.33 | 0.24 | 0.14 | 1.40 |
| 18 | 0.41 | 0.15 | 1.33 | 0.11 | 0.11 | 1.23 |
| 20 | 0.27 | 0.14 | 1.38 | 0.04 | 0.09 | 1.11 |

Table 4	Simulation results of CQI reporting test in fading condition for TDD SCS=30kHz.

|  | 1Rx UE |  |  | 2Rx UE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | BLER with following CQI | TP ratio with following CQI and Median CQI | Percentage of reported CQI Index not in {medCQI-1, medCQI+1} | BLER with following CQI | TP ratio with following CQI and Median CQI |
| 0 | 0.40 | 0.08 | 1.40 | 0.45 | 0.09 | 1.56 |
| 2 | 0.42 | 0.06 | 1.49 | 0.50 | 0.09 | 1.55 |
| 4 | 0.52 | 0.04 | 1.58 | 0.47 | 0.10 | 1.48 |
| 6 | 0.58 | 0.06 | 1.53 | 0.46 | 0.12 | 1.40 |
| 8 | 0.52 | 0.06 | 1.50 | 0.49 | 0.14 | 1.45 |
| 10 | 0.56 | 0.07 | 1.48 | 0.47 | 0.16 | 1.35 |
| 12 | 0.55 | 0.09 | 1.49 | 0.45 | 0.16 | 1.34 |
| 14 | 0.56 | 0.11 | 1.41 | 0.34 | 0.16 | 1.33 |
| 16 | 0.51 | 0.11 | 1.41 | 0.19 | 0.16 | 1.35 |
| 18 | 0.40 | 0.13 | 1.41 | 0.08 | 0.12 | 1.20 |
| 20 | 0.24 | 0.12 | 1.63 | 0.01 | 0.09 | 1.09 |

In Rel-15, RAN4 set the SNR test points to 6/7dB and 12/13dB for 2Rx. For RedCap UE with CQI table 1, even if we reuse the same test points as the existing test case, it is observed the existing requirements can be satisfied, i.e., α=5%, γ=1.05, PDSCH BLER=0.02. We therefore propose to set the same test points and requirements for RedCap 2Rx UE.

Regarding the RedCap 1Rx UE, we can set SNR=9/10dB and 15/16dB, which is 3dB higher than the test points of 2Rx UE requirements. According to our simulation results it is observed we can reuse α=5%, γ=1.05, PDSCH BLER=0.02.

Proposal 4: Reuse the existing test metrics of CQI reporting test in fading condition for RedCap 1Rx/2Rx UEs, i.e.,

- A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ.

- When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Proposal 5: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=6/7dB and 12/13dB for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 6: Set α=5%, γ=1.05 for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 7: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=9/10dB and 15/16dB for RedCap 1Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 8: Set α=5%, γ=1.05 for RedCap 1Rx UE (both FDD 15kHz and TDD 30kHz).

# 4	Summary

Proposal 1: For the static condition CQI reporting test with CQI table 1, set two SNR test points corresponding to CQI indexes of 16QAM and 64QAM for RedCap 1Rx and 2Rx UEs.

Proposal 2: For the static condition CQI reporting test with CQI table 1 for 2Rx UEs (rank 2), set SNR=8/9dB and SNR=14/15dB.

Proposal 3: For the static condition CQI reporting test with CQI table 1 for 1Rx UEs (rank 1), set SNR=5/6dB and SNR=11/12dB.

Proposal 4: Reuse the existing test metrics of CQI reporting test in fading condition for RedCap 1Rx/2Rx UEs, i.e.,

- A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ.

- When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Proposal 5: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=6/7dB and 12/13dB for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 6: Set α=5%, γ=1.05 for RedCap 2Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 7: For the wideband CQI reporting test with CQI table 1 in fading condition, set SNR=9/10dB and 15/16dB for RedCap 1Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 8: Set α=5%, γ=1.05 for RedCap 1Rx UE (both FDD 15kHz and TDD 30kHz).

# References

- R4-2207206 “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.

# Appendix

Table 5	Mapping of CQI Index to Information Bit payload (CQI table 1)

| CBW/SCS |  |  |  | 10MHz / 15kHz | 10MHz / 15kHz | 20MHz / 30kHz | 20MHz / 30kHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 51 | 51 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 |
| Number of DMRS REs |  |  |  | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |
| Available REs |  |  |  | 6240 | 6240 | 6120 | 6120 |
| CQI index | Spectrum efficiency | MCS index | Modulation |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |
| 1 | 0.2344 | 0 | 2 | 1480 | 2976 | 1480 | 2856 |
| 2 | 0.2344 | 0 | 2 | 1480 | 2976 | 1480 | 2856 |
| 3 | 0.3770 | 2 | 2 | 2408 | 4744 | 2408 | 4616 |
| 4 | 0.6016 | 4 | 2 | 3752 | 7424 | 3752 | 7296 |
| 5 | 0.8770 | 6 | 2 | 5504 | 11016 | 5376 | 10760 |
| 6 | 1.1758 | 8 | 2 | 7296 | 14600 | 7168 | 14344 |
| 7 | 1.4766 | 11 | 4 | 9224 | 18432 | 8968 | 17928 |
| 8 | 1.9141 | 13 | 4 | 12040 | 24072 | 11784 | 23568 |
| 9 | 2.4062 | 15 | 4 | 15112 | 30216 | 14600 | 29192 |
| 10 | 2.7305 | 18 | 6 | 16896 | 33816 | 16896 | 33816 |
| 11 | 3.3223 | 20 | 6 | 20496 | 40976 | 20496 | 40976 |
| 12 | 3.9023 | 22 | 6 | 24576 | 49176 | 24072 | 48168 |
| 13 | 4.5234 | 24 | 6 | 28168 | 56368 | 27656 | 55304 |
| 14 | 5.1152 | 26 | 6 | 31752 | 63528 | 31240 | 62504 |
| 15 | 5.5547 | 28 | 6 | 31752 | 69672 | 33816 | 67584 |