3GPP TSG-RAN WG4 Meeting #103-e	R4-2209066

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.6.4.3

Source:	Ericsson

Title:	CQI reporting requirements for DL 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with the way forward on CQI reporting test for DL 1024QAM [1]. We discuss the SNR test points for the CQI reporting tests according to the simulation results.

Discuss the final CQI definition requirements with CQI table 4 considering the impairment margin and SNR levels provided by TE with the assumption of Tx EVM 2.5%. - 2Rx UE: - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 28-30dB for 2Rx UE without impairment margin. - Option 1: SNR = 28/29dB - Option 2: SNR = 29/30dB - 4Rx UE: - This is the common understanding SNR corresponding to CQI index 14 for 1024QAM table is around 26-28dB for 4Rx UE without impairment margin. - Option 1: SNR = 25/26dB - Option 2: SNR = 26/27dB

# 2	Discussion

Figure 1 shows the median CQI from our simulation results to find the SNR test points corresponding to rank 1 CQI index 14 for 2Rx UE with Tx EVM 2.5%. The detailed simulation parameters are given in Appendix and the CQI to MCS (TBS) mapping table is shown in Table 1 in Appendix.

Figure 1	Median CQI to satisfy RAN4 CQI definition requirements (2Rx UE).

According to the simulation results, the required SNR test points to report CQI index 14 with rank 1 are from 29dB to 32dB without impairment margin. If we consider the impairment margin, the required SNR is expected to be higher. However as we discussed in RAN4#102-e, it is reasonable to assume TE can provides up to SNR=30dB including the test system uncertainty for 1024QAM (See A.3) due to Tx EVM of 2.5%. We therefore propose to set the maximum possible SNR test points for 2Rx UE, that is, SNR=29/30dB. For 4Rx UE, we propose SNR=26/27dB.

Observation: Test system can provide up to SNR=30dB including the test system uncertainty for 1024QAM.

Proposal 1: Set SNR=29/30dB for 2Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 2: Set SNR=26/27dB for 4Rx UE (both FDD 15kHz and TDD 30kHz).

# 3	Summary

Observation: Test system can provide up to SNR=30dB including the test system uncertainty for 1024QAM.

Proposal 1: Set SNR=29/30dB for 2Rx UE (both FDD 15kHz and TDD 30kHz).

Proposal 2: Set SNR=26/27dB for 4Rx UE (both FDD 15kHz and TDD 30kHz).

# References

- R4-2207254, “Way forward for NR DL1024QAM demodulation and CQI reporting requirements”, Ericsson.

# Appendix

Table 1	CQI-to-TBS mapping table for CQI definition test (Table 4, rank 1)

|  |  |  |  | FDD 15kHz Rank 1 | TDD 30kHz Rank 1 |
| --- | --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 1 |
| Number of DMRS REs |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available REs for PDSCH |  |  |  | 6240 | 12720 |
| CQI index | Spectrum efficiency | MCS index | Modulation |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 |
| 2 | 0.377 | 1 | QPSK | 2408 | 4744 |
| 3 | 0.877 | 2 | QPSK | 5504 | 11016 |
| 4 | 1.4766 | 3 | 16QAM | 9224 | 18960 |
| 5 | 2.4063 | 5 | 16QAM | 15112 | 30728 |
| 6 | 3.3223 | 8 | 64QAM | 20496 | 42016 |
| 7 | 3.9023 | 10 | 64QAM | 24576 | 49176 |
| 8 | 4.5234 | 12 | 64QAM | 28168 | 57376 |
| 9 | 5.1152 | 14 | 64QAM | 30216 | 61480 |
| 10 | 5.5547 | 16 | 256QAM | 34816 | 69672 |
| 11 | 6.2266 | 18 | 256QAM | 38936 | 79896 |
| 12 | 6.9141 | 20 | 256QAM | 43032 | 88064 |
| 13 | 7.4063 | 22 | 256QAM | 46104 | 94248 |
| 14 | 8.3301 | 24 | 1024QAM | 52224 | 106576 |
| 15 | 9.2578 | 26 | 1024QAM | 57376 | 116792 |

## A.1	CQI table with 1024QAM entries

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

## A.2	Simulation parameters

| Parameter |  |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 |
| Duplex Mode |  |  |  | FDD | TDD |
| Subcarrier spacing |  |  | kHz | 15 | 30 |
| SNR |  |  | dB | TBD | TBD |
| Propagation channel |  |  |  | AWGN | AWGN |
| Antenna configuration |  |  |  | Test 1-1: 2×2 with static channel specified in Annex B.1 Test 1-2: 2×4 with static channel specified in Annex B.1 | Test 2-1: 2×2 with static channel specified in Annex B.1 Test 2-2: 2×4 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6,-) | Row 3,(6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (4, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| ReportConfigType |  |  |  | Periodic | Periodic |
| CQI-table |  |  |  | Table 4 | Table 4 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 16 |
| Csi-ReportingBand |  |  |  | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 010000 | 010000 |
|  |  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 |
| Measurement channel |  |  |  | See Table 1 | See Table 1 |

# A.3	Achievable SNR test points in FR1

## A.3.1	TS38.521-4 Table F.1.1.2-2

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 3% | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |

## A.3.2	Test system downlink SNR

According to TS38.521-4 4.4.4.2.1, SNR of the applied signal due to EVM impairment on the wanted Es is given by 20*log10(1/EVM). For example, given allowed EVM of 3%, achievable SNR is derived from 20*log10(1/0.03) = 30.5dB. If we assume Tx EVM of 2.5% the achievable SNR is 32dB (=20*log10(1/0.025)).